3GPP TSG RAN WG1 #109-e                                                                                                   R1-2203441

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	On unified TCI framework extension for multi-TRP operation

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

Introduction

In RAN #94 e-meeting, the enhancement for extending Rel-17 Unified TCI framework for multi-TRP use cases was agreed and the following objectives were approved in the WID for Rel-18 NR MIMO [1]:

2. Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. 6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. 7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, our views on extending Rel-17 Unified TCI to multi-TRP use cases in Rel-16/Rel-17 as well as multi-panel UL transmission are analyzed, including power control related issues.

Extension for indication of multiple DL/UL TCI states

Applicable scenarios

In NR Rel-16, mTRP for PDSCH has been specified as a first step, where S-DCI based SDM-1a/FDM-2a/FDM-2b/TDM3/TDM4 PDSCH transmission and M-DCI based PDSCH transmission are supported. Then, in NR Rel-17, mTRP transmission is further extended to the other channels, i.e. PDCCH (including HST-SFN), PUCCH and PUSCH. In addition, inter-cell mTRP is also supported. In Rel-18, simultaneous uplink transmission will be investigated under Rel-17 unified TCI framework. Rel-17 unified TCI aims to simplify the beam indication mechanism and reduce signaling overhead. Considering such benefits and backward compatibility, Rel-17 unified TCI framework should be applied to all these mTRP cases.

Proposal-1: Rel-17 unified TCI framework should be applied to all Rel-16/17/18 mTRP cases.

Under the unified TCI framework, the number of DL TCI states is defined as M, and the number of UL TCI states is defined as N. In Rel-17, only M=1 and N=1 are supported. For Rel-16/17 mTRP cases, for each channel, at most two TCI states are indicated to support two TRPs. Since the scheme for simultaneous uplink transmission has not been discussed yet, the number of TCI states is not clear. Therefore, as an extension in Rel-18, M<=2 and N<=2 should be supported at least for Rel-16/17 mTRP cases.

Proposal-2: Support M<=2 (the number of DL TCI states) and N<=2 (the number of UL TCI states) at least for Rel-16/17 mTRP cases.

In Rel-17, either joint TCI mode or separate DL/UL TCI mode is configured to UE by RRC. Considering mTRP as shown in Figure 1, there are two TRP-to-UE links. As channel characteristics are independent for each link, it is possible that one link (e.g. link-0) may suffer from MPE and the other one does not. Then, hybrid configuration of joint TCI and separated DL/UL TCI seems necessary to be supported in Rel-18. As TCI state mode could only be configured statically by RRC, TCI state mode is preferred to be configured per link instead of per UE. For example, link-0 would be configured with separate DL/UL TCI and link-1 would be configured with joint TCI by RRC.

Figure 1: TCI state mode configuration for multi-TRP

Proposal-3: In Rel-18, TCI state mode (i.e. joint TCI or separate DL/UL TCI) could be configured per link for multi-TRP.

TCI state association

For Rel-16 S-DCI based multi-TRP, PDSCH could be transmitted from two TRPs simultaneously (SDM-1a/FDM-2a/FDM-2b) or be transmitted from two TRPs according to a time domain repetition pattern (TDM3/TDM4). For Rel-17 multi-TRP PDCCH, PDCCH could be transmitted by two associated search spaces with each search space or its associated CORESET transmitting from one individual TRP. For Rel-17 HST-SFN, each CORESET is associated with two TCI states for SFN transmission. For Rel-17 PUCCH/PUCSH multi-TRP, time domain repetition transmission is used where each resource would be transmitted from two TRPs. To support all these cases within the unified TCI framework, when multiple TCI states are indicated, UE has to correctly associate each TCI state to corresponding channel resource(s) or transmission repetition(s). Therefore, Rel-18 has to specify the TCI state association with resource and/or time domain repetition. To define the association, TCI state indicators (the order index of the indicated TCI states, e.g. TCI_ind_0 denotes the first TCI state, TCI_ind_1 denotes the second TCI state,…), which refer to the indicated TCI states, could be employed in Rel-18.

Proposal-4: Rel-18 has to specify the TCI state association with resource and/or time domain repetition.

Proposal-5: To define the association, TCI state indicators referring to the indicated TCI states could be employed in Rel-18.

The above association could be defined for each channel independently. For PDCCH, in the case of S-DCI, each CORESET could be associated with one or two joint/DL TCI states. For example, if two joint TCI states (denoted by TCI_ID0, TCI_ID1) are indicated, according to the association, each CORESET would either use TCI_ID0 or TCI_ID1 for transmission. If two TCI states are both associated with the CORESET, SFN transmission would be used.

Proposal-6: To support multi-TRP PDCCH transmission, each CORESET has to be associated with one or two indicated TCI state.

For PDSCH, in the case of SDM-1a/FDM-2a/FDM-2b, there is no need to define the association since both TCI states have to be used. For TDM-3/TDM-4, the association between TCI state and each time domain repetition has been specified in Rel-16, which could be reused in Rel-18. For PUCCH, similar as PDSCH, SpatialRelationInfo and PUCCH resource association has also been defined in Rel-17. Therefore, Rel-17 unified TCI could be directly extended to support multi-TRP PDSCH/PUCCH by replacing Rel-15/16 TCI states/SpatialRelationInfo with Rel-17 TCI states and defining the association with TCI indicators.

Proposal-7: Rel-17 unified TCI could be directly extended to support multi-TRP PDSCH/PUCCH by replacing Rel-15/16 TCI states/SpatialRelationInfo with Rel-17 TCI states and defining the association with TCI indicators.

For PUSCH multi-TRP transmission in Rel-17, the TX beam of each time domain repetition is determined by the SpatialRelationInfo of a SRS indicated by an SRI. Following the same mechanism, one extension scheme is to associate a joint/UL TCI state with a SRS resource set. But according to Rel-17 unified TCI, SRS resources within an SRS resource set could share the indicated TCI state. If SRS does not share the indicated TCI state, the association scheme would be problematic. To avoid such issue, another candidate scheme is to associate a joint/UL TCI state directly with each repetition.

Proposal-8: To support Rel-17 multi-TRP PUSCH transmission, each time domain repetition is associated with an joint/UL TCI state.

The associations mentioned above could be configured by RRC, MAC-CE or DCI. For example, each CORESET could be configured with its associated TCI indicator(s) by RRC. Another alternative is to use the activated MAC-CE to configure the association. Namely, for each activated TCI state in the MAC-CE, its associated CORESET ID(s) are also indicated in the same MAC-CE. For PUSCH, the association between joint/UL TCI state and repetition could be indicated by the scheduling DCI. The SRS resource set(s) field in DCI format 0_1/0_2 would be re-defined as the TCI state association. For example, “codepoint = 01” means single TRP mode and PUSCH is associated with TCI_ind_1. “codepoint = 10” means multi-TRP mode and PUSCH repetition with TCI_ind_0 and TCI_ind_1 order.

Proposal-9: The TCI state association for PDCCH and PUSCH could be configured by RRC, MAC-CE or DCI.

For M-DCI based multi-TRP, CORESETPoolIndex is configured for each CORESET. For each TRP (represented by a CORESETPoolIndex), an independent DCI (corresponding to the CORESETPoolIndex) would be used for beam indication. In this way, the TCI state and CORESET association has already be supported through CORESETPoolIndex in the current spec. In addition, DL and UL transmission is also implemented independently per TRP. Consequently, with unified TCI in Rel-18, each indicated TCI state (or TCI state pair) would be used for UE-dedicated reception on all CORESETs with a same CORESETPoolIndex, and their scheduled UE-dedicated PDSCH, PUCCH and PUSCH.

Proposal-10: For M-DCI based multi-TRP, each indicated TCI state (or TCI state pair) is used for UE-dedicated reception on all CORESETs with a same CORESETPoolIndex, and their scheduled UE-dedicated PDSCH, PUCCH and PUSCH.

Single TRP and multi-TRP switching

In Rel-16, single TRP and multi-TRP PDSCH transmission are dynamically switched according to the number of TCI states indicated in the scheduling DCI format 1_1/1_2. Similarly, for Rel-17 PUSCH, dynamic switching is also supported according to the “SRS resource set” field in DCI format 0_1/0_2. Although common TCI states are indicated for all the channels in Rel-18, the same transmission flexibility as Rel-16/17 should be maintained for different channels. Namely, each channel (PDCCH/PDSCH/PUCCH/PUSCH) could determine its single TRP or multi-TRP transmission separately. For PDSCH/PUSCH, similar as current spec, dynamic switching is achieved by its scheduling DCI. For PDCCH, switching between single TRP and multi-TRP depends on selecting different resources or update the TCI state association configured for a resource. For PUCCH, the switching could either be done using DCI or similar as that of PDCCH.

Proposal-11: For flexibility, each channel could determine its single TRP or multi-TRP transmission separately.

Beam indication signaling

In Rel-17, unified TCI has been agreed to be indicated with MAC-CE or MAC-CE+DCI. If only one or one pair of TCI state(s) is activated, MAC-CE would be directly used for beam indication. Otherwise, depending on UE capability, MAC-CE is used for multiple TCI states activation, and DCI format 1_1/1_2 with/without DL assignment would be used for beam indication. These signalings should at least be reused in Rel-18.

Proposal-12: MAC-CE/MAC-CE+DCI format 1_1/1_2 with/without DL assignment should at least be reused for beam indication in Rel-18.

In Rel-15/16 S-DCI based multi-TRP, switching between single TRP and multi-TRP PDSCH transmission is determined by the number of TCI states in the scheduling DCI. Namely, when the TCI codepoint indicated in DCI format 1_1/1_2 with DL assignment is associated with a single TCI state, the scheduled PDSCH is assumed to be transmitted by single TRP. When two TCI states are indicated, the scheduled PDSCHs are to be received based on both TCI states. These indicated TCI states would be applied to the scheduled PDSCH transmission. In Rel-17, beam application time (BAT) has been defined for unified TCI. In case of DCI indication, to guarantee the robustness of beam indication, BAT is defined as the first slot that is at least  Y symbols after the last symbol of the acknowledgement of the joint or separate DL/UL beam indication. Such BAT definition would impact PDSCH dynamic switching in Rel-18. Taking Figure 2 as an example, the TCI codepoint indicated in the former DCI is associated with two TCI states. These two TCI states are applied after Y symbols after the ACK/NACK. For the latter DCI, the TCI codepoint is associated with only one TCI state. It intends for single TRP transmission. The number of activated TCI states during PDSCH transmission is not be aligned with the intended single TRP/multi-TRP transmission scheduled by DCI format 1_1/1_2. In this way, switching between single TRP and multi-TRP could not be done immediately after DCI indication.

Observation-1: Switching between single TRP and multi-TRP could not be done immediately after DCI indication due to BAT.

Figure 2: Issue on single TRP and multi-TRP dynamic switching for PDSCH

To deal with the problem, we should try to decouple beam indication and DL assignment. In our opinion, a new DCI field could be introduced in DCI format 1_1/1_2 with DL assignment, which is used to indicated single TRP or multi-TRP transmission for the scheduled PDSCH. In this way, the indicated TCI state(s) only aims for beam indication, and the specified BAT mechanism in Rel-17 is reused. On the other hand, the indicated single TRP or multi-TRP transmission in this new DCI field would be applied to the schedule PDSCH immediately, which is independent of the BAT. It is possible that the applied number of TCI states and the indicated single TRP/multi-TRP are not aligned. For example, in the dot circle of Figure 2, two TCI states are applied, but DCI indicates single TRP transmission. Therefore, only one of the two TCI states is used for PDSCH transmission. The selection of TCI state could also been indicated by the new DCI field, where 2bits are used for both single TRP/multi-TRP indication and TCI state selection. Another example is only one TCI state is applied, but DCI indicates multi-TRP transmission. This could be regarded as an error case, which is not allowed to be configured by gNB. Therefore, with this scheme, gNB would flexibly switch between single TRP and multi-TRP for the scheduled PDSCH.

Proposal-13: In Rel-18, introduce new DCI field(s) in DCI format 1_1/1_2 with DL assignment to indicate switching between single TRP and multi-TRP.

Simultaneous multi-panel UL transmission

Association of panel and TCI state

In Rel-17, UE-initiated UL panel activation and selection is supported for fast uplink panel selection, via UE reporting a list of UE capability values along with the pairs of SSBRI/CRI and L1 RSRP in a beam reporting instance. Each UE capability value comprises the maximum supported number of SRS antenna ports, and the maximum supported numbers of SRS antenna ports of any two UE capability values are different.

For simultaneous multi-panel UL transmission in Rel-18, up to 2 TRPs and up to 2 panels is considered. When two TCI states are selected, the two TCI states may correspond to the same panel or correspond to two panels. In terms of RF functionality, a UE panel is not able to generate two analog beams simultaneously. Therefore indicating two beams corresponding to the same panel for simultaneous UL transmission should be avoided. That means that for the configuration of unified TCI for simultaneous UL transmission, gNB has to know the correspondence between panels and TCI states.

Proposal-14: For simultaneous multi-panel UL transmission, how to facilitate gNB to know the correspondence between panels and TCI states should be studied.

There are various approaches to identify the correspondence between panels and TCI states. Panel ID may be not needed for indicating the association of RSs for TCI state and panels. For example, similar as that for Rel-17 fast panel selection, UE capability values can be used to indicate the association between SSBs/CSI-RSs and panels.

Proposal-15: For simultaneous multi-panel UL transmission, solutions for panel identification without panel ID are considered,

- UE capability value can be considered as a start.

Indicating of unified TCI

For a 2-panel UE capable of simultaneous transmission with multi panels in the scenario with 2 TRPs, 4 TCI states are needed to reflect the transmission relationship of UE panels and TRPs. Since one panel is not capable of generating two beams simultaneously, including two UL TCI states in a codepoint for unified TCI indication is sufficient. Each of the two UL TCI states corresponds to a panel, and the two UL TCI states correspond to one or two TRPs. Note that the two UL TCI states can be TCI states for joint TCI or UL TCI states for separate UL TCI.

Proposal-16: For simultaneous multi-panel UL transmission, up to two UL TCI states are included in a TCI codepoint for unified TCI indication (i.e. N<= 2).

Association between TCI states and DMRS ports for PUSCH

Simultaneous multi-panel PUSCH transmission is beneficial for throughput improvement and reliability/coverage improvement. It should be supported for both codebook based PUSCH and non-codebook based PUSCH, for PUSCH scheduled by DCI or with configured grant.

Proposal-17: Simultaneous multi-panel PUSCH transmission should be supported for codebook based PUSCH and non-codebook based PUSCH, for PUSCH scheduled by DCI or with configured grant.

For simultaneous multi-panel PUSCH transmission, SDM based, FDM based or SFN based transmission scheme can be considered, with the details of each transmission schemes are provided in our companion contribution [2].

For the association of TCI states and PUSCH transmissions for two panels, similar mechanisms as that for DL multi-TRP can be considered. For UL SDM scheme 1a, two sets of PUSCH layers are associated with two TCI states respectively. For FDM scheme 2a, the TB of PUSCH consists two parts, and the two parts are mapped to two frequency domain resources, respectively. The two frequency domain resources are associated with two TCI states respectively. For FDM scheme 2b, the two TBs of PUSCH are mapped to two frequency domain resources, respectively, and the two frequency domain resources are associated with two TCI states respectively. For SFN based PUSCH transmission, each DMRS ports of the PUSCH is associated with two TCI states.

In Rel-16 and Rel-17, dynamic switching of single TRP and multi-TRP is supported for all DL multi-TRP transmission schemes and UL multi-TRP transmission schemes scheduled by S-DCI. It is natural to support dynamic switching of single panel and multi-panel transmission for Rel-18 simultaneous multi-panel UL transmission scheduled by S-DCI.

Proposal-18: For simultaneous multi-panel PUSCH transmission scheduled by S-DCI,  dynamic switching of single panel and multi-panel transmission is supported.

Both Rel-17 multi-TRP PUSCH transmission schemes and Rel-18 simultaneous multi-panel PUSCH transmission schemes map the PUSCH transmission to two beams. The only difference of the two approaches is that different PUSCH repetitons are associated to different beams in Rel-17 and different PUSCH transmission layer sets (for SDM based solutions) or frequancy domain resources (for FDM based solutions) are assocaited to different beams in Rel-18. Therefore the indication scheme for dynamic switching of single TRP and multi-TRP for Rel-17 multi-TRP PUSCH transmission under unified TCI framwork can be used for Rel-18 multi-panel PUSCH transmission.

Proposal-19: On dynamic switching of single panel and multi-panel transmission for Rel-18 simultaneous multi-panel PUSCH transmission under unified TCI framework, similar scheme as that for dynamic switching of single TRP and multi-TRP for Rel-17 multi-TRP PUSCH transmission shall be used.

Simultaneous multi-panel PUSCH transmission scheduled by M-DCI also can be considered in Rel-18, with two PUSCHs scheduled by two PDCCHs associated with different CORESETPoolIndex values. The two PUSCHs can be fully/partially/non-overlapped in time and frequency domain. For such transmission scheme, one important issue is that which PUSCHs can be transmitted simultaneously. One possible solution is that PUSCHs scheduled by PDCCH resources associated with different CORESETPoolIndex can be transmitted simultaneously.

Proposal-20: For simultaneous multi-panel PUSCH transmission scheduled by M-DCI, which PUSCHs can be transmitted simultaneously should be studied.

For PUSCH transmission with Type 2 configured grant, same TCI indicating schemes as that for Rel-17 multi-TRP PUSCH transmission with Type 2 configured grant can be considered..

For PUSCH transmission with Type 1configured grant, same TCI states determination schemes as that for Rel-17 multi-TRP PUSCH transmission with Type 1 configured grant can be considered.

Proposal-21: On TCI state determination for simultaneous multi-panel PUSCH transmission with Type 1/Type 2 configured grant under unified TCI framework, same scheme as that for Rel-17 multi-TRP UL repetition with Type 1/Type 2 configured grant shall be used.

Unified TCI for PUCCH

Different to PUSCH, only one layer is supported for a PUCCH resource. Therefore the enhancement on simultaneous multi-panel PUCCH transmission is mainly for reliability/coverage improvement. SFN based or FDM based schemes can be considered for PUCCH. For SFN based PUCCH transmission, the DMRS port of the PUCCH resource is associated to two TCI states. For FDM based PUCCH transmission, the PUCCH resource is mapped to two frequency domain resources, with the two frequency domain resources associated to two TCI states respectively.

Proposal-22: For simultaneous multi-panel PUCCH transmission, SFN based and FDM based transmission schemes can be considered.

In Rel-17, a PUCCH resource can be associated with one or two spatialRelationInfos, and the spatialRelationInfo(s) can be activated by MAC-CE. When one spatialRelationInfo is activated, the PUCCH resource corresponds to single TRP transmission, and when two spatialRelationInfos are activated, the PUCCH resource corresponds to multiple TRP transmission. Under unified TCI framework, similar scheme can be used by replacing spatialRelationInfo by TCI state. For Rel-18 simultaneous multi-panel UL transmission, if one PUCCH resource associated to two TCI states is supported, same beam indication scheme for PUCCH as that for Rel-17 multi-TRP PUCCH transmission can be used.

Proposal-23: If one PUCCH resource associated to two TCI states is supported for simultaneous multi-panel transmission, MAC CE is used to indicate the associated TCI state(s) of the PUCCH resource.

For a UE capable of transmitting with multi-panel simultaneously, simultaneous PUCCH transmission with two PUCCH resources can be considered. The two PUCCHs can be fully/partially/non-overlapped in time and frequency domain. And the two PUCCH resources may be scheduled by S-DCI or M-DCI. If the two PUCCH resources are scheduled by M-DCI, the two PDCCHs carrying the scheduling DCIs can be associated with different CORESETPoolIndex values. Similar as PUSCH transmission scheduled by M-DCI, which PUCCH resources can be transmitted simultaneously shall be studied.

Proposal-24: For simultaneous multi-panel PUCCH transmission, whether a single DCI can trigger more than one PUCCH resources shall be studied .

Proposal-25: PUCCH transmission scheduled by M-DCI can be considered for simultaneous multi-panel UL transmission.

Proposal-26: For simultaneous multi-panel PUCCH transmission scheduled by M-DCI, which PUCCH resources can be transmitted simultaneously should be studied.

Power control enhancement for Multi-TRP transmission

As approved in RAN #94-e meeting, power control enhancements include the following objectives [1]:

- Study, and if justified, specify the following

  - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

- For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this section, possible power control mechanisms for multi-TRP uplink transmission under unified TCI framework are discussed.

Unified TCI framework was introduced in NR Rel-17 for multi-beam operation, it was agreed that power control parameters for PUSCH/PUCCH/SRS are associated with the joint TCI state/UL TCI state indicated by gNB. TRP specific power control has been supported for Rel-17 PUSCH/PUCCH TDM repetition transmission, where two SRIs/spatialRelationInfos are used to indicate the power control settings associated with each TRP individually. The scheme of power control setting determination for Rel-17 PUSCH/PUCCH TDM repetition doesn’t work in the unified TCI framework where the uplink transmission beam is indicated by joint TCI states/UL TCI states instead of SRI/spatialRelationinfo.

Proposal-27: Enhancement of power control for UL single DCI for multi-TRP operation is supported, at least for Rel-17 PUSCH/PUCCH TDM repetition.

For Rel-17 PUSCH/PUCCH TDM repetition transmission scheme, transmission occasions are cyclically/sequentially mapped to the two beams (TRPs). When a DLorjoint TCI state/UL TCI state is indicated in transmission occasion i, the power control parameter setting that is included in the indicated DLorjoint TCI state/UL TCI state is used to determine the corresponding transmission power by the current formula provided in [3] as shown below.

(1)

Proposal-28: For Rel-17 PUSCH/PUCCH TDM repetition, replace the power control parameter setting(PL-RS, P0, alpha) associated with SRI/spatial relation info by the one included in the indicated DLorjoint TCI state/UL TCI state to perform open loop power control.

In NR Rel-18, simultaneous UL transmission from multiple panels is an important use case to be considered, since the transmission power of each panel might be different and the maximum transmission power for each panel is limited, panel-specific power control should be considered. Suppose UE has two panels, for once transmission, when two TCI states are indicated to UE to indicate two PUSCH transmission to the same/different TRPs, UE can determine two individual Tx PC parameter settings accordingly and use it to perform open-loop power control of each transmission.

Proposal-29: Support enhancement of power control for UL single DCI in NR Rel-18 uplink simultaneous transmission.

Proposal-30: For Rel-18 uplink simultaneously transmission, two sets of individual power control parameters are determined according to the DLorjoint TCI state/UL TCI state indicated by gNB for multi-TRP transmission, use the determined power control parameter setting to perform open loop power control.

Proposal-31: Reuse two TPC fields in Rel-17 S-DCI based MTRP PUCCH/PUSCH repetition to achieve TRP/panel-specific closed-loop power control.

For NR Rel-18 uplink simultaneous transmission, considering each panel/TRP has its own maximum transmission power, and the total transmission power of a UE is also limited. Power control mechanisms need to be considered for these limitations. Suppose UE has two panels, the maximum transmission power for each panel is Pcmax1, Pcmax2. The maximum total transmission power of the UE is Pcmax which is configured per CC[3]. Basically, power control mechanism of the following cases should be studied:

Case1: P1 + P2 <= Pcmax

Case2: P1 + P2  > Pcmax

where P1 and P2 represents the transmission power of panel 1 and panel 2 determined from equation (1) respectively, with Pcmax replaced by Pcmax1 and Pcmax2 in the calculation. For Case 2, how to determine actual transmission power of panel 1 and panel 2 on the basis of P1 and P2 to meet the requirement of total transmission power limitation need to be further studied.

Figure 3. Power restriction cases in NR Rel-18 uplink simultaneous transmission

For Case1, only power control of per-panel power limitation need to be considered, whether the current framework(i.e. equation (1) to determine transmission power of PUSCH transmission) can be extended to the case of per-panel power limitation need to be further studied. In detail, the definition of Pcmax in the equation may need to be modified.

For Case 2, power control schemes considering both per-panel power limitation and total transmission power limitation need to be considered. Suppose UE has two panels, the maximum transmission power for panel 1 and panel is Pcmax1 = 23dBm, Pcmax2 = 23dBm. The maximum total transmission power of the UE Pcmax = 20dBm. For once transmission, UE determines its real PUSCH transmission power P1 = 21dBm and P2 = 22dBm individually according to equation (1) with Pcmax replaced by Pcmax1 and Pcmax2 respectively. In addition, considering Pcmax is limited by 20dBm and P1 + P2  > Pcmax, power control mechanism, e.g. power back-off mechanism can be used to reduce the transmission power of both TRPs or one of the TRPs, e.g. UE can reduce the power of one TRP with a smaller pathloss by a scaling factor provided or determined by UE.

Proposal-32: In order to support TRP-specific UL power control in sDCI-mTRP operation. The case of Per-panel power limitation and/or total power limitation should be considered. For the case of total power limitation, some power back-off mechanisms can be further studied. For the case of per-panel power limitation, the formula in TS38.213 to determine PUSCH transmission power needs to be further refined.

Conclusions

In this contribution, we discuss extending Rel-17 Unified TCI to multi-TRP use cases in Rel-16/Rel-17 as well as multi-panel UL transmission. The power control related issues are also analyzed. We have the following observation and proposals:

Observation-1: Switching between single TRP and multi-TRP could not be done immediately after DCI indication due to BAT.

Proposal-1: Rel-17 unified TCI framework should be applied to all Rel-16/17/18 mTRP cases.

Proposal-2: Support M<=2 (the number of DL TCI states) and N<=2 (the number of UL TCI states) at least for Rel-16/17 mTRP cases.

Proposal-3: In Rel-18, TCI state mode (i.e. joint TCI or separate DL/UL TCI) could be configured per link for multi-TRP.

Proposal-4: Rel-18 has to specify the TCI state association with resource and/or time domain repetition.

Proposal-5: To define the association, TCI state indicators referring to the indicated TCI states could be employed in Rel-18.

Proposal-6: To support multi-TRP PDCCH transmission, each CORESET has to be associated with one or two indicated TCI state.

Proposal-7: Rel-17 unified TCI could be directly extended to support multi-TRP PDSCH/PUCCH by replacing Rel-15/16 TCI states/SpatialRelationInfo with Rel-17 TCI states and defining the association with TCI indicators.

Proposal-8: To support Rel-17 multi-TRP PUSCH transmission, each time domain repetition is associated with an joint/UL TCI state.

Proposal-9: The TCI state association for PDCCH and PUSCH could be configured by RRC, MAC-CE or DCI.

Proposal-10: For M-DCI based multi-TRP, each indicated TCI state (or TCI state pair) is used for UE-dedicated reception on all CORESETs with a same CORESETPoolIndex, and their scheduled UE-dedicated PDSCH, PUCCH and PUSCH.

Proposal-11: For flexibility, each channel could determine its single TRP or multi-TRP transmission separately.

Proposal-12: MAC-CE/MAC-CE+DCI format 1_1/1_2 with/without DL assignment should at least be reused for beam indication in Rel-18.

Proposal-13: In Rel-18, introduce new DCI field(s) in DCI format 1_1/1_2 with DL assignment to indicate switching between single TRP and multi-TRP.

Proposal-14: For simultaneous multi-panel UL transmission, how to facilitate gNB to know the correspondence between panels and TCI states should be studied.

Proposal-15: For simultaneous multi-panel UL transmission, solutions for panel identification without panel ID are considered,

- UE capability value can be considered as a start.

Proposal-16: For simultaneous multi-panel UL transmission, up to two UL TCI states are included in a TCI codepoint for unified TCI indication (i.e. N<= 2).

Proposal-17: Simultaneous multi-panel PUSCH transmission should be supported for codebook based PUSCH and non-codebook based PUSCH, for PUSCH scheduled by DCI or with configured grant.

Proposal-18: For simultaneous multi-panel PUSCH transmission scheduled by S-DCI,  dynamic switching of single panel and multi-panel transmission is supported.

Proposal-19: On dynamic switching of single panel and multi-panel transmission for Rel-18 simultaneous multi-panel PUSCH transmission under unified TCI framework, similar scheme as that for dynamic switching of single TRP and multi-TRP for Rel-17 multi-TRP PUSCH transmission shall be used.

Proposal-20: For simultaneous multi-panel PUSCH transmission scheduled by M-DCI, which PUSCHs can be transmitted simultaneously should be studied.

Proposal-21: On TCI state determination for simultaneous multi-panel PUSCH transmission with Type 1/Type 2 configured grant under unified TCI framework, same scheme as that for Rel-17 multi-TRP UL repetition with Type 1/Type 2 configured grant shall be used.

Proposal-22: For simultaneous multi-panel PUCCH transmission, SFN based and FDM based transmission schemes can be considered.

Proposal-23: If one PUCCH resource associated to two TCI states is supported for simultaneous multi-panel transmission, MAC CE is used to indicate the associated TCI state(s) of the PUCCH resource.

Proposal-24: For simultaneous multi-panel PUCCH transmission, whether a single DCI can trigger more than one PUCCH resources shall be studied .

Proposal-25: PUCCH transmission scheduled by M-DCI can be considered for simultaneous multi-panel UL transmission.

Proposal-26: For simultaneous multi-panel PUCCH transmission scheduled by M-DCI, which PUCCH resources can be transmitted simultaneously should be studied.

Proposal-27: Enhancement of power control for UL single DCI for multi-TRP operation is supported, at least for Rel-17 PUSCH/PUCCH TDM repetition.

Proposal-28: For Rel-17 PUSCH/PUCCH TDM repetition, replace the power control parameter setting(PL-RS, P0, alpha) associated with SRI/spatial relation info by the one included in the indicated DLorjoint TCI state/UL TCI state to perform open loop power control.

Proposal-29: Support enhancement of power control for UL single DCI in NR Rel-18 uplink simultaneous transmission.

Proposal-30: For Rel-18 uplink simultaneously transmission, two sets of individual power control parameters are determined according to the DLorjoint TCI state/UL TCI state indicated by gNB for multi-TRP transmission, use the determined power control parameter setting to perform open loop power control.

Proposal-31: Reuse two TPC fields in Rel-17 S-DCI based MTRP PUCCH/PUSCH repetition to achieve TRP/panel-specific closed-loop power control.

Proposal-32: In order to support TRP-specific UL power control in sDCI-mTRP operation. The case of Per-panel power limitation and/or total power limitation should be considered. For the case of total power limitation, some power back-off mechanisms can be further studied. For the case of per-panel power limitation, the formula in TS38.213 to determine PUSCH transmission power needs to be further refined.

References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN #94-e.

[2] R1-2203446, On UL precoding indication for multi-panel transmission, CATT, RAN1 #109-e.

[3] 3GPP RAN1, TS 38.213 V17.1.0