3GPP TSG RAN WG1 meeting #109-e                                                 R1-2203597

e-Meeting, May 9th – 20th, 2022

Source: 	ZTE, Sanechips

Title:	Discussion on power saving for RRC connected UEs

Agenda item:	8.7.3

Document for:	Discussion and decision

Introduction

In previous RAN1 meeting, a framework which supports both PDCCH skipping and SSSG switching behaviors was agreed [1]. In this contribution, of the details of application delay for PDCCH monitoring adaptation are provided.

Application delay for PDCCH skipping

In RAN1 #108-emeeting, the following agreement was provided.

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

Whether or not an additional UE behavior is needed to handle the case of PDCCH skipping interaction with HARQ retransmission hasn’t been agreed. The reason to handle this case is to reduce the latency of retransmission. However, the complex of UE implementation also increases while solving the problem. The figure below shows an example when considering the case of DRX Retransmission timer for PDCCH skipping.

Figure 1.  PDCCH skipping with interaction with re-transmission

In Figure 1, when DRX Retransmission timer is running, it is considered that the UE will monitor PDCCH. And the UE receives a retransmission DCI which indicates SSSG switching when drx-retransmissionTimerDL is running. In this case, the UE performs 5 changes of PDCCH monitoring behavior in several slots. The 1st behavior transition is performed due to a PDCCH skipping indication, the 2nd and 4th behavior transition are performed in order to monitor retransmission DCI, the 3rd behavior transition is performed after receiving a retransmission DCI which indicates SSSG switching for UE power saving, the 5th behavior transition is performed after an application delay. The switching between different PDCCH monitoring states is too frequent and complicated for UE implementation.

The switching between different PDCCH monitoring states is too frequent and complicated for UE implementation if the UE will monitor PDCCH when DRX Retransmission timer is running.

According to simulation results, the power saving gain is related to the length of PDCCH skipping duration. A longer PDCCH skipping duration is beneficial to achieve greater power saving gain. However, if retransmission is considered, the relationship between the length of PDCCH skipping duration and power saving gain is unpredictable, which makes it more difficult for NW to select an appropriate PDCCH skipping duration value.

If retransmission is considered, the relationship between the length of PDCCH skipping duration and power saving gain is unpredictable, which may makes it more difficult for NW to select an appropriate PDCCH skipping duration value.

Conclusion

In this contribution, we discuss the PDCCH monitoring adaptation for RRC connected UE. We have the following observations.

Observation 1: The switching between different PDCCH monitoring states is too frequent and complicated for UE implementation if the UE will monitor PDCCH when DRX Retransmission timer is running.

Observation 2: If retransmission is considered, the relationship between the length of PDCCH skipping duration and power saving gain is unpredictable, which may makes it more difficult for NW to select an appropriate PDCCH skipping duration value.

References

Draft_Minutes_report_RAN1#108-e v0.1.0.