3GPP TSG-RAN WG1 #109	R1-2203754

e-Meeting, 9th May – 20th May, 2022

Agenda item:		9.5.3

Source:	MediaTek Inc.

Title:	The potential solutions for RedCap UEs for positioning

Document for:		Discussion

# Introduction

A RedCap UE has the following noticeable properties,

- Maximum bandwidth of 20MHz for FR1 and 100MHz for FR2

- Type-A HD-FDD

- 1 or 2 RX branches

The most influential property to the positioning performance is the limited maximum UE bandwidth. To provide the “equivalent” BW expansion, there could be two methods,

- BW hopping, further containing

  - UE BW hopping for reception

  - gNB DL-PRS BW hopping for transmission

  - UE SRS BW hopping for transmission outside UL BWP

- BW expansion through space domain

  - Whether phase difference based AOD is a promising solution?

In this contribution, we provide the feasibility analysis for the above two methods.

# The solutions related to BW hopping

If a DL-PRS resource in large BW is also configured with sufficient repetition number, a RedCap UE is able to perform BW hopping for reception so that UE, after assembling several small pieces of DL-PRS BWs, could observe a larger one, as shown in Fig. 2-1. There is no specification impact to the case of BW hopping for reception.

If a network doesn't transmit a DL-PRS resource in large BW with sufficient repetitions, due to the RS overhead/deployment consideration, the network may, upon request, transmit DL-PRS in smaller BW with the hopping manner, so that UE is also able to adapt the reception BW, as shown in Fig. 2.2.

The RF retuning for UE to adapt the reception BW doesn't guarantee the phase consistency. The change of phase may need to be measured and compensated when UE takes several observed DL-PRS BWs to assemble for a larger one. To facilitate the UE for the measurement of the phase change, the partial BW overlapping in frequency domain between the hops in transmission could be considered, also depicted in Fig. 2.2.

Similarly, for the positioning techniques requiring both DL and UL measurements, the hopping of the SRS transmission BW may also be considered. To do so, the transmission outside the UL BWP may be needed.

Fig. 2-1

Fig. 2-2,

Proposal 2-1: Support DL-PRS transmission hopping

Proposal 2-2: For DL-PRS transmission hopping, the partial overlapping in frequency domain between the BW before and after the hopping is preferred

Proposal 2-3: The SRS transmission outside UL BWP may also be considered, especially for the positioning techniques of requiring DL and UL measurements

# The solutions related to BW expansion through space domain

The Bluetooth Core Specification v5.1 has a feature of direction finding. By utilizing an array of multiple antennas, the signals are transmitted across antennas. Among the received signals which are transmitted by each TX antenna to a same UE, the received phase may be slightly different due to the slightly different propagation distance between any antenna in an array to the same UE, as shown in Fig. 3-1.

The similar concept as the phase difference based AOD has been proposed but not agreed in Rel-17. Note that in Rel-16, the RSRP based AOD has been supported in NR. During the Rel-18 study phase, the phase difference based AOD, in our view, needs to be justified to be better than the RSRP based AOD. Otherwise there is no need to specify another solution for angle based measurement.

Fig. 3-2 shows the receiver view for the observed phases of the channels between each TX antenna in an array to a same antenna of UE. For each subcarrier, the observed channels across TX antennas may have the dependency in terms of the phase rotation, and the phase rotation may also be related to the antenna spacing at TX side. Let’s name it as “signature vector” for representing the phase rotations across TX antennas to be observed by UE. For each direction, there is the corresponding signature vector. Generally, the signature vector under a horizontal linear array could be expressed as

where the parameter θ may be related to the direction between TX and RX, and the array placement such as the spacing at TX.

The phase difference based AOD may turn into the search of the signature vector with an unknown value representing the direction, for example the θ1. Then UE may form a steering vector with a same phase rotation structure (the same signature) by changing θ to search. The search could be similar to perform the inner product. Then the θ with maximum output power within the search range may determine the direction (θ1) of incoming signal.

The algorithms for array signal processing may also be applied to provide the finer resolution, such as MUSIC and CAPON, both requiring matrix inversion. For each subcarrier carrying DL-PRS, the vector formed in space domain could be used to construct the correlation matrix of channel and the further inversion of the matrix.

Fig. 3-3 shows the operation of the RSRP based AOD. A TRP may transmit multiple beams, and UE may measure the RSRP of the beams. The vector of collecting the measured RSRPs among TX beams, or the corresponding differential RSRPs, has the information of direction between a TRP and a UE. Let’s name it as RSRP vector. For each direction, there is the corresponding RSRP vector. Then the RSRP vector under the RSRP based AOD is similar to the signature vector under the phase difference AOD, both representing the direction between a TRP and a UE.

For RSRP based AOD, the beam shape and the orientation of each beam could be pre-determined. For each direction, the vector of collecting the perfect EIRPs or differential EIRPs among TX beams, say an EIRP vector, may represent the transmission direction. At LMF, a steering vector of using this format could search the direction by matching it with the reported RSRP vector.

Under the RSRP based AOD, the beam shape and the orientation of each beam are also related to the antenna placement. Fig. 3-4 gives an example that, if a beam is designed with the orientation at -120o, the intentional delay or advance for transmitting the signal from each antenna may take into account the additional propagation delay from each antenna toward that direction (-120o in the example), so that the signals from the antennas could be summed coherently at the desired direction, achieving the maximum power.

In above, the additional propagation delay from each antenna of an array toward a same direction are equivalent to form the signature vector.

Then, both the RSRP based AOD and the phase difference AOD utilize the property of antenna placement such as the spacing. In other words, under the RSRP based AOD for a beam, the EIRP in a direction is equivalent to the inner product between the signature vector and the designated direction vector with the amounts of the phase rotations for adjusting the delay for each antenna. The matching means the coherent sum within the inner product of the two vectors.

This also means, the signature vector under the phase difference based AOD, has been transformed to an EIRP vector under the RSRP based AOD.

|  | Phase difference based AOD | RSRP based AOD |
| --- | --- | --- |
| What to measure | Direct measurement on the signature vector | Indirect measurement on the signature vector, since the signature vector has been transformed to an EIRP vector. And the UE is to measure the EIRP vector by RSRP measurement |
| How to measure | 1, Apply a changing steering vector to search the phase vector through the inner product 2, Fix the steering vectors at several orientations to measure the RSRPs on these orientations through inner product. The RSRPs have the information of direction, and this method is very similar to that for the RSRP based AOD | 1, Measure the RSRP of the beams with several orientations |
| How to transmit | A different antenna port for each antenna, with different time/freq resource allocation | Same antenna port for all antennas |
| RS consumption | Antenna port number x RS number for a port | Beam number x RS number for a beam |

TABLE 3-1

Proposal 3-1: The phase difference based AOD needs to be justified to be better than the RSRP based AOD. Otherwise there is no need to define another solution for the angle based measurement

Proposal 3-2: Move the discussion of the phase difference based AOD to AI 9.5.2.2 for carrier phase measurement

Fig. 3-1,

Fig. 3-2, receiver view for the observed channel between each TX antenna and a same UE

Fig. 3-3,                                           Fig. 3-4,

# Conclusion

Proposal 2-1: Support DL-PRS transmission hopping

Proposal 2-2: For DL-PRS transmission hopping, the partial overlapping in frequency domain between the BW before and after the hopping is preferred

Proposal 2-3: The SRS transmission outside UL BWP may also be considered, especially for the positioning techniques of requiring DL and UL measurements

Proposal 3-1: The phase difference based AOD needs to be justified to be better than the RSRP based AOD. Otherwise there is no need to define another solution for the angle based measurement, since there are quite a lot similarity between the RSRP based AOD and the phase difference based AOD

Proposal 3-2: Move the discussion of the phase difference based AOD to AI 9.5.2.2 for carrier phase measurement