3GPP TSG RAN WG1 #109-e	R1-2204435

e-Meeting, May 9 – 20, 2022

Agenda item:	8.6.1

Source:	NEC

Title: 	Remaining details on BWP operation for RedCap

Document for:	Discussion and Decision

Introduction

Following proposals were not concluded in RAN1#108-e [1].

High Priority Proposal 2-1-2d: For the case that the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth,

- A separate initial DL BWP is always configured for RedCap if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth.

- From RAN1 perspective, if generic parameters (location, bandwidth, SCS, and cyclic prefix) of this separate initial DL BWP are not configured absent and the separate initial downlink BWP contains the entire CORESET#0, RedCap UE can continue to use the location, bandwidth, SCS, and cyclic prefix of the MIB-configured CORESET#0. Necessity and feasibility of signaling optimizations are up to RAN2.

Note: For TDD, the center frequencies of the separate initial DL BWP and the initial UL BWP are aligned (in accordance with earlier agreement).

High Priority Proposal 3-1h: Down select between the following options during RAN1#108-e:

- Option 1:

  - For FR1, for BWP#0 configuration option 1,

    - A RedCap UE in connected mode does not expect to operate in a separate initial DL BWP that does not include CD-SSB and the entire CORESET#0.

  - For FR2, for BWP#0 configuration option 1,

    - A RedCap UE in connected mode does not expect to operate in a separate initial DL BWP that does not include CD-SSB and the entire CORESET#0.

- Option 2:

  - For FR1, for BWP#0 configuration option 1,

    - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

      - During a random access procedure in connected mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB.

  - For FR2, for BWP#0 configuration option 1,

    - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

      - During a random access procedure in connected mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB.

  - For BWP#0 configuration option 1, upon successful completion of the random access procedure, a RedCap UE supporting FG 6-1 only (but not FG 6-1a) in connected mode is not required to receive any DL signals except for RACH-related messages and RRC-based BWP switch signal operate on a separate initial DL BWP that does not contain SSB.

Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP.

FL1/FL2 High Priority Question 3-2: For BWP#0 configuration option 1, should it be supported to use the separate initial DL BWP in connected mode for other purposes than random access? If the answer is yes, please comment in the Comments field on whether the UE should expect it to always contain SSB.

High Priority Proposal 4-1-2a:

- A RedCap UE is not required to perform measurements on more than one SSB at a time in a the same BWP.

A RedCap UE mandatorily supports configurable time offsets (including zero) between CD-SSB and NCD-SSB.

In this contribution, we discuss the first two proposals, High Priority Proposal 2-1-2d and High Priority Proposal 3-1h.

Discussion

High priority proposal 2-1-2d

High Priority Proposal 2-1-2d: For the case that the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth,

- A separate initial DL BWP is always configured for RedCap if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth.

- From RAN1 perspective, if generic parameters (location, bandwidth, SCS, and cyclic prefix) of this separate initial DL BWP are not configured absent and the separate initial downlink BWP contains the entire CORESET#0, RedCap UE can continue to use the location, bandwidth, SCS, and cyclic prefix of the MIB-configured CORESET#0. Necessity and feasibility of signaling optimizations are up to RAN2.

Note: For TDD, the center frequencies of the separate initial DL BWP and the initial UL BWP are aligned (in accordance with earlier agreement).

We can support High Priority Proposal 2-1-2d as it is.

The first bullet of the proposal would be fine, which is in line with legacy behavior.

The second bullet of the proposal is an optimization for a special case where a separate initial BWP contains all RB of MIB-configured CORESET#0, which would not always be possible in TDD. Though this would not be essential, we are OK with it as far as signaling design is up to RAN2.

Regarding the note, this should always be valid including the case a separate initial DL BWP shares the location and bandwidth of MIB-configured CORESET#0.

Proposal:

- Agree High Priority Proposal 2-1-2d in RAN1#108-e as it is, otherwise, no agreement is needed

- Note will be valid including the case a separate initial DL BWP shares the location and bandwidth of MIB-configured CORESET#0 in TDD.

High priority proposal 3-1h

High Priority Proposal 3-1h: Down select between the following options during RAN1#108-e:

- Option 1:

  - For FR1, for BWP#0 configuration option 1,

    - A RedCap UE in connected mode does not expect to operate in a separate initial DL BWP that does not include CD-SSB and the entire CORESET#0.

  - For FR2, for BWP#0 configuration option 1,

    - A RedCap UE in connected mode does not expect to operate in a separate initial DL BWP that does not include CD-SSB and the entire CORESET#0.

- Option 2:

  - For FR1, for BWP#0 configuration option 1,

    - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

      - During a random access procedure in connected mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB.

  - For FR2, for BWP#0 configuration option 1,

    - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

      - During a random access procedure in connected mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB.

  - For BWP#0 configuration option 1, upon successful completion of the random access procedure, a RedCap UE supporting FG 6-1 only (but not FG 6-1a) in connected mode is not required to receive any DL signals except for RACH-related messages and RRC-based BWP switch signal operate on a separate initial DL BWP that does not contain SSB.

Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP.

RAN2 made the following agreement in RAN2#117-e.

In connected mode if RA occasions are not configured on the active BWP, RedCap UEs should use the RedCap-specific initial UL BWP, if configured, or else legacy BWP#0

Case of the RAN2 agreement would correspond to BWP#0 configuration option 1, where RAN2 agreed a RedCap UE should initiate RA in a separate initial UL BWP if configured, otherwise legacy BWP#0. This behavior would require option 2 of the proposal.

Proposal:

- Further discuss option 2 for High Priority Proposal 3-1h considering RAN2 agreement in RAN2#117-e

Summary

In this contribution, we discuss remaining moderator’s proposals, Proposal 2-1-2d and Proposal 3-1h in RAN1#108-e and made following proposals.

Proposal:

- Agree High Priority Proposal 2-1-2d in RAN1#108-e as it is, otherwise, no agreement is needed

- Note will be valid including the case a separate initial DL BWP shares the location and bandwidth of MIB-configured CORESET#0 in TDD.

Proposal:

- Further discuss option 2 for High Priority Proposal 3-1h considering RAN2 agreement in RAN2#117-e

Reference

[1] R1-2202532	FL summary #5 on reduced maximum UE bandwidth for RedCap	Moderator (Ericsson)