3GPP TSG RAN WG1#109-e	           R1-2204562

e-Meeting, 09th – 20th May 2022

Agenda item:	9.5.2.3

Source: 	Lenovo

Title: 	LPHAP considerations

Document for:	Discussion

# Introduction

The Rel-18 NR positioning evolution SID was agreed upon during the RAN#94-e [1] meeting, where one of the objectives included improved energy efficiency using LPHAP. The following highlighted SID objectives were outlined to support the discussion:

Improved Energy efficiency: - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

Although efforts were made in Rel-17 to support RRC_INACTIVE positioning, there is sufficient room for further enhancements to enable fully energy efficient positioning, while maintaining high accuracy performance. This contribution provides a discussion into the use cases in addition to a discussion aspects related power consumption evaluation model for LPHAP.

# Use cases

Low power high accurate positioning is needed for the IIoT asset/tool tracking and process automation applications. As agreed in RAN#94-e only a limited use case is evaluated for power consumption using the existing NR positioning technique developed in NR Rel-16/Rel-17.

Table 1: Low power high accuracy positioning use cases [2]

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

Use cases that require moderate and longer battery time should be taken for study which corresponds to Use case #6 – flexible modular assembly area related to tracking of workpiece, Use case#8 - flexible modular assembly area related to positioning of autonomous vehicles and Use case#9 related to asset tracking for logistics. These can be considered representative of the LPHAP systems, where longer battery lifetimes are desirable.

Proposal 1: RAN 1 to study representative LPHAP Use cases #6, #8 and #9 for evaluation, which requires moderate to longer battery times.

UE Power Model Evaluation

The evaluation in the study is limited to UEs in RRC_INACTIVE/RRC_IDLE state and the baseline for the power consumption evaluation can be taken using the UE power saving model reported in TR 38.840 [3]. More specifically, Sec 8.1.4 based on RRM measurement can be taken as a reference for the LPHAP evaluation of RRC_INACTIVE/RRC_IDLE UEs. The factors affecting the positioning evaluation can be considered on top of the RRM measurement procedure since DL-based positioning is also dependent on the measurement gap (MG) configuration, where measurements need to be considered from serving and non-serving cells. Since most of the frequencies deployed in the factories are in the C-band i.e., 3.5 GHz carrier frequency, FR1 should be prioritized compared to higher mmWave frequencies in FR2.

In addition, the cost of such IIoT devices should be cheaper and hence the RF chain/number of antennas should be limited. This aspect is similar to RedCap baseline evaluation, where the number of antennas is already limited to 1 or 2. However, the RedCap device bandwidth is limited and hence the positioning accuracy is also inherently limited. In the case of LPHAP, no such limitation on bandwidth is assumed. The NR bandwidths (e.g., 100 MHz) for the evaluation of LPHAP devices should consider FR1, since LPHAP demands a very high accuracy.

The coverage requirement for LPHAP is not provided in the use case description and the loss of coverage and accuracy due to the limited number of receive antenna should also be considered for the selected use cases assumed for the evaluation.

Observation 1: Coverage and accuracy of the LPHAP is affected due to limited Rx antenna.

Proposal 2: RAN1 to use the power saving TR 38.840, Sec 8.1.4 - UE power consumption model for RRM measurements, as baseline for LPHAP power model evaluation.

Proposal 3: RAN 1 to prioritize the FR1 frequency range for LPHAP evaluations.

Proposal 4: RAN 1 to evaluate LPHAP use cases considering low complexity devices with a limited number of antennas/RF chains while leveraging higher bandwidths (e.g., 100MHz) for FR1.

Other factors that may affect the positioning power model evaluation include:

- Number of positioning frequency layers

- Number of TRPs transmitting DL PRS

- Positioning method processing time depending on the positioning method

- Number of PRS occasions

- Positioning reporting, which impacts the positioning update interval

Considering that the complexity of the IIoT devices is generally low, the UE capability is already limited with respect to the supported antenna/beam, type of positioning method, etc., which should be factored during the evaluation phase. The DL-PRS received during the RRC_INACTIVE state within a certain measurement window can be used for the positioning measurement calculation.  The power consumption calculation is a function of the total energy consumption of the UE, which should include DL-PRS measurement and processing, periodical/triggered/immediate reporting within the specified duration.

Proposal 5: RAN1 to consider following for the LPHAP power model evaluation

- Number of positioning frequency layers

- Number of TRPs transmitting DL PRS

- Positioning method processing

- Positioning measurement window

- Positioning reporting

The type of positioning model is also important in the context of the LPHAP evaluation since NR already supports UE-based positioning and UE-assisted positioning. Further consideration should also be given to the UE-based positioning model. The lack of mandatory measurement reporting to the network may also offer power savings gains for LPHAP devices.

Proposal 6: Consider UE-based positioning model in the LPHAP evaluation.

# Conclusion

The following observations are summarized as follows:

Observation 1: Coverage and accuracy of the LPHAP is affected due to limited Rx antenna.

The proposals are summarized as follows:

Proposal 1: RAN 1 to study representative LPHAP Use cases #6, #8 and #9 for evaluation, which requires moderate to longer battery times.

Proposal 2: RAN1 to use the power saving TR 38.840, Sec 8.1.4 - UE power consumption model for RRM measurements, as baseline for LPHAP power model evaluation.

Proposal 3: RAN 1 to prioritize the FR1 frequency range for LPHAP evaluations.

Proposal 4: RAN 1 to evaluate LPHAP use cases considering low complexity devices with a limited number of antennas/RF chains while leveraging higher bandwidths (e.g., 100MHz) for FR1.

Proposal 5: RAN1 to consider following for the LPHAP power model evaluation

- Number of positioning frequency layers

- Number of TRPs transmitting DL PRS

- Positioning method processing

- Positioning measurement window

- Positioning reporting

Proposal 6: Consider UE-based positioning model in the LPHAP evaluation.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RAN#94-e, Dec. 2021.

- 3GPP TS 22.104, “Service requirements for cyber-physical control applications in vertical domains”, V17.2.0, Dec. 2019.

- 3GPP TR 38.840, “Study on User Equipment (UE) power saving in NR”, V16.0.0, June 2019.