3GPP TSG RAN WG1 Meeting #109-e	R1-2204767

e-Meeting, May 09 – 20, 2022

Source:	Intel Corporation

Title:	Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz

Agenda item:	8.2.2

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our views on remaining open issues for PDCCH monitoring capability and search space set configuration for the PDCCH monitoring.

# Search space set configuration

The multi-slot PDCCH monitoring is based on a fixed pattern of slot groups, where each group has X consecutive slots and the slot groups are consecutive and non-overlap. The group (1) SS and group (2) SS are configured in a slot group independently. Group (1) SS is only monitored in the Y consecutive slots in a slot group. The same position of the Y slots in a slot group maintains in all slot groups. The default value of Y is 1. Note that the slot/symbol position of a SS set with searchSpaceId = 0 is determined by the associated SS/PBCH block, while the other SS sets in group (2) can be anywhere in a slot group. The budget of maximum BD/CCE are shared by all configured PDCCHs in a slot group.

In RAN1#108-e meeting, the details on SS set configuration was agreed together with a working assumption on Group (2) SS handling. However, it is questionable if a common understanding was achieved among interested companies.

- Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Two sizes L are supported for this parameter: L=4 bits and L=8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in each group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - For Group (1) SSs - The slots indicated in the bitmap should be consecutive per group of L slots - The number of slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE - Working assumption: For Group (2) SSs - For Type0/0A/2 CSS - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L - For Type1 CSS without dedicated RRC - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than M, where M is FFS

From SS set configuration point of view, the working assumption allows to configure Type0/0A/2 CSS sets in up to Xs slots in the slot group. This allows the full flexibility on SS set configuration. On the other hand, it limits to up to M slots for Type1 CSS set without dedicated RRC configuration. It would be helpful to clarify what is the implied UE capability on the blind detection in the slot group by the WA.

As commented by several companies in last meeting, the configuration of Type0/0A/2 CSS sets in multiple or all slots in a slot group doesn’t necessarily mean that UE must do PDCCH detection in the multiple or all slots.

For example, the MOs of Type0A/2 CSS sets with searchSpaceID non-zero are sequentially numbered and mapped to the actual transmitted SSBs according to ssb-PositionsInBurst in SIB1, where the actual transmitted SSBs are sequentially numbered in ascending order of their SSB indexes. Accordingly, a UE only needs to detect PDCCH in the MO associated with the proper SSB of the UE. As a results, UE normally only monitor one MO or even not monitor any MO at all in a slot group. Since it is not to enforce UE to be capable of monitoring Type0/0A/2 CSS sets in all slots in the slot group, it is important to clarify how to count the number of BD/CCE for Type0/0A/2 CSS sets in the slot group. A single MO in a slot may have up to 28 CCEs (32 CCEs for the CORESET #0 and 1/2/4 candidates for AL 16/8/4 respectively), which almost consumes all the BD/CCE budget (20/32) for the slot group of Xs slots. Therefore, it is preferred to clarify that the BD/CCE of a MO for Type0A/2 CSS is only counted towards the BD/CCE budget when the UE actually detects Type0A/2 PDCCH in the MO.

Similarly, the number of MOs for Type1 CSS without RRC configuration in a slot group should be limited, otherwise it causes problem from BD/CCE budget point of view. We prefer to limit M=1.

Observation:

- On the WA for SS set configuration, it is necessary to clarify what is the implied UE capability on the blind detection in the slot group by the WA.

Proposal 1:

- The BD/CCE of a MO for Type0A/2 CSS sets are only counted towards the BD/CCE budget when the UE actually detects Type0A/2 PDCCH in the MO.For SS set configuration

- Agree on M=1 for Type1 CSS without RRC configuration

# SSSG switching

The search space set group (SSSG) switching was firstly introduced in NR-U in Rel-16, which provides a means to balance the fast channel access right after LBT is successful and effective scheduling and power saving in the COT. Further, SSSG switching is extended in power saving WI in Rel-17. The feature is useful for FR2-2 too.

Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320.

When two SSSGs are configured for a UE, it is preferred that the BDs/CCEs of the inactive SSSG are not counted toward the BD/CCE budget (20/32 per slot group of Xs slots). Otherwise, the USS set in the inactive SSSG, if configured with a small SS set index, may block a USS set in the active SSSG. As a results, there may be no applicable PDCCH candidate to schedule DL/UL transmission of the UE. Following the logic, the active combination (Xs, Ys) should be separately determined for each SSSG.

Figure 1 provides two examples for the SSSG switching with corresponding switching between combination (4, 1) and (4, 2). In Figure 1A, at the time for SSSG switching, UE needs to decode more PDCCHs if no additional restriction is introduced. A simple solution is that UE can cancel the PDCCH detection in X slots after the time of SSSG switching. On the other hand, Figure 1B shows another example that there exists a distance between the slots for PDCCH monitoring at SSSG switching, which means UE can monitor all PDCCHs. Based on Figure 1A/1B, a unified solution could be defined. That is, within a period of X slots after the time of SSSG switching, the UE can monitor the PDCCHs in the slots that are the intersection of the Y1 & Y2 slots of combinations (X, Y) used before and after SSSG switching.

Figure 1: SSSG switching resulting in burst PDCCH detections

The following TP is proposed to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y).

Reasons for change: UE may be required to perform PDCCH monitoring beyond its capability when search space set group switching from one set of search space to another occurs.

Summary of change: within a period of maxXs0,Xs1 slots after SSSG switching, the UE can monitor the PDCCHs in the slots that are the intersection of the YS1 & YS2 slots of combinations (XS1, YS1) and (XS2, YS2) used before and after SSSG switching

------------------------------ TP#4c: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** ## 10.4 Search space set group switching *** Unchanged text is omitted *** If a UE is provided by SearchSpaceSwitchTrigger a location of a search space set group switching flag field in a DCI format 2_0, as described in clause 11.1.1, for a serving cell where the UE has active DL BWP with SCS configuration μ - if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell - at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈0, 1, 2, 3 - at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈5, 6 - if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 1, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell - at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, when μ∈0, 1, 2, 3 - at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈5, 6 and the UE sets the timer value to the value provided by searchSpaceSwitchTimer - if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell - at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈0, 1, 2, 3 - at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈5, 6 If a UE indicates a capability to monitor PDCCHs according to multiple combinations Xs,Ys, the search space sets with group index 0 and 1 are respectively associated with different combinations Xs0,Ys0 and Xs1,Ys1, the UE monitors PDCCHs of search space sets with group index g in the slot(s) that belongs to both the Ys0 slots of combinations Xs0,Ys0 and the Ys1 slots of combinations Xs1,Ys1 within the first maxXs0,Xs1 slots after switching to search space sets with group index g. *** Unchanged text is omitted ***

Proposal 2:

- The BDs/CCEs of the inactive SSSG are not counted toward the BD/CCE budget in a slot group of Xs slots.

- The search space set configurations of the two SSSG can correspond to two different PDCCH monitoring capabilities combinations (X, Y)

- UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of the two combinations (X, Y) before and after SSSG switching

- Agree on TP 1 to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y).

# Conclusions

In this contribution, we discussed the potential enhancements on PDCCH monitoring in 52.6-71GHz frequency. The following is a summary of proposals:

Observation:

- On the WA for SS set configuration, it is necessary to clarify what is the implied UE capability on the blind detection in the slot group by the WA.

Proposal 1:

- The BD/CCE of a MO for Type0A/2 CSS sets are only counted towards the BD/CCE budget when the UE actually detects Type0A/2 PDCCH in the MO.For SS set configuration

- Agree on M=1 for Type1 CSS without RRC configuration

Proposal 2:

- The BDs/CCEs of the inactive SSSG are not counted toward the BD/CCE budget in a slot group of Xs slots.

- The search space set configurations of the two SSSG can correspond to two different PDCCH monitoring capabilities combinations (X, Y)

- UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of the two combinations (X, Y) before and after SSSG switching

- Agree on TP 1 to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y).