3GPP TSG RAN WG1 #109-e	R1-22xxxxx

e-Meeting, May 9th - 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for HARQ-ACK multiplexing on PUSCH in the absence of PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator(Apple), …. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specification the UE behavior is unknown when a PUSCH is scheduled by a DAI field in a PUCCH slot but the UE does not determine any PUCCH resource with HARQ-ACK information for transmission in the PUCCH slot. There are two cases for the undefined behavior with respect to scheduled PUSCH(s) in the PUCCH slot: - A single PUSCH within the PUCCH slot - Multiple PUSCHs within the PUSCH slot The following agreement in RAN1#107-e was made to address the first case, that is single PUSCH in the slot. Agreement For Rel-16 with one PUSCH and no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot (both single carrier and UL CA), if the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE multiplexes HARQ-ACK following the UL-TDAI into the PUSCH. For case of multiple PUSCHs, it was agreed in RAN1#109-e that the UE assumes a “slot-length virtual PUCCH” in the PUCCH slot and assumes all the PUSCHs in the PUCCH slot are candidate PUSCHs. Then, the UE applies the existing procedures to determine the PUSCH for UCI multiplexing. This behavior is subject to UE capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description of UE behavior according to the agreement in RAN1#107-e. - Add description of UE behavior to describe two scenarios for multiple PUSCHs in the slot. The first one according to the excising specification when PUCCH is present in the slot, and the second one for the case when PUCCH with HARQ-ACK is absent in the slot but at least one of the multiple PUSCHs is scheduled with a DCI with UL-TDAI field. For the first case, the candidate PUSCHs for UCI multiplexing are those PUSCHs that overlap with the PUCCH as the existing procedures. For the second case, the candidate PUSCHs for HARQ-ACK multiplexing are all PUSCHs in the slot except the ones with UL-TDAI=4 or 0 in case of dynamic or semi-static HARQ-ACK codebook, respectively. After determining the candidate PUSCHs, the UE follows the existing procedures to determine a PUSCH for UCI multiplexing where the second case is subject to UE capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This CR completes an incompletely specified Rel-15 functionality for multiplexing HARQ-ACK in a PUSCH in a PUCCH slot when the UE misses any HARQ-ACK to transmit in any PUCCH but receives UL grant(s) with UL-TDAI field to transmit a single PUSCH or multiple PUSCHs in the PUCCH slot. A UE implemented based on an earlier version of the specification, and for case of multiple PUSCHs not able to indicate the new UE capability, may still be compliant with this CR, while a UE not compliant with the CR has unknown behavior. See isolated impact analysis. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... 38.306 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: If a gNB is implemented according to the CR, but the UE is not, the UE behavior is unclear when HARQ-ACK is to be multiplexed in a PUSCH on a PUCCH slot where the UE transmits one or more PUSCHs scheduled by UL grant(s) with UL-TDAI field in the corresponding DCI in the PUCCH slot on one or more than 1 carrier, potentially leading to loss of UCI as well as loss of PUSCH. If the UE is implemented according to the CR, but the gNB is not, the gNB assumption on where it expects to find the UCI is unclear when HARQ-ACK is to be multiplexed in a PUSCH on a PUCCH slot where the UE transmits one or more PUSCHs scheduled by UL grant(s) with UL-TDAI field in the corresponding DCI in the PUCCH slot on one or more than 1 carrier, potentially leading to loss of UCI as well as loss of PUSCH. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 9	UE procedure for reporting control information

<unchanged text omitted>

A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, a DCI format 1_1 indicating SCell dormancy, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

If a UE would transmit a single  PUSCH scheduled by a DCI format that includes a DAI field on a serving cell in a slot with reference to slots for PUCCH transmissions without any other PUSCH that would be transmitted on any serving cell in the slot, and if the value of the DAI field is not equal to 4 in case the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic or with pdsch-HARQ-ACK-Codebook-r16, or is equal to 1 in case the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static, and the UE does not determine any PUCCH carrying HARQ-ACK information in the slot, the UE multiplexes HARQ-ACK information in the PUSCH transmission [subject to the conditions in clause 9.2.5 for UCI multiplexing where the group defined in clause 9.2.5 refers to the single PUSCH only].

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

When a UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the multiple PUSCHs overlap with a PUCCH carrying UCI in the slot, the UE selects all the PUSCHs overlapping with the PUCCH as the candidate PUSCHs for UCI multiplexing within the slot.

If the UE indicates the corresponding capability [the name of the capability], when a UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the UE does not determine any PUCCH carrying HARQ-ACK information in the slot and at least one of the multiple PUSCHs is scheduled by a DCI format that includes a DAI field, the UE selects all the multiple PUSCHs in the slot as the candidate PUSCHs for HARQ-ACK multiplexing within the slot except for any PUSCH among the multiple PUSCHs that is scheduled by a DCI format that includes a DAI field that is equal to 4 in case the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic or with pdsch-HARQ-ACK-Codebook-r16, or is equal to 0 in case the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

The UE determines the PUSCH for UCI multiplexing by applying the following procedure on the candidate PUSCHs as described in this clause:

- If the candidate PUSCHs include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the candidate PUSCHs, and the candidate PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs.

- If the UE would multiplex UCI in one of the candidate PUSCHs and the UE does not multiplex aperiodic CSI in any of the candidate PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot.

If a UE transmits a PUSCH over multiple slots and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent.

If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects all actual repetitions of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition of the PUSCH transmission that would overlap with the PUCCH transmission and includes more than one symbol. The UE does not expect that all actual repetitions that would overlap with the PUCCH transmission do not include more than one symbol.

If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information.

When a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-UCI-Multiplexing; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.