3GPP TSG RAN Meeting #96	RP-20xxxx

Budapest, Hungary, June 6-9, 2022	(revision of RP-202846)

- Source:	LG Electronics

- Title:	Revised WID on NR Sidelink enhancement

- Document for:	Approval

Agenda Item:	9.4.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	NR Sidelink enhancement

## Acronym: NR_SL_enh

## Unique identifier: 860042

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720003 | Study on Enhancement of 3GPP support for V2X services (FS_eV2X) | Rel-15 Stage 1 study on eV2X service requirements |
| 750002 | Enhancement of 3GPP support for V2X scenarios (eV2X) | Rel-15 Stage 1 work on eV2X service requirements |
| 790002 | Study on Improvement of V2X Service Handling (FS_V2XIMP) | Rel-16 SA1 study item on eV2X service requirements |
| 820024 | Improvement of V2X service Handling (V2XIMP) | Rel-16 SA1 work item on eV2X service requirements |
| 760043 | Study on architecture enhancements for 3GPP support of advanced V2X services (FS_eV2XARC) | Rel-15/16 SA2 study item on eV2X architecture |
| 820018 | Architecture enhancements for 3GPP support of advanced V2X services | Rel-16 SA2 work item on eV2X architecture |
| 800096 | Study on NR Vehicle-to-Everything (FS_NR_V2X) | Rel-16 RAN study item on NR V2X feasibility |
| 830078 | 5G V2X with NR sidelink (5G_V2X_NRSL) | Rel-16 RAN work item on 5G V2X |
| 840042 | New WID on Complete Gap Analysis for Railways (MONASTERYEND) | Rel-17 Stage-1 PC5 communication requirement from Railway perspective |
| 840044 | Network Controlled Interactive Service (NCIS) | Rel-17 PC5 interface stage-1 requirement for interactive services |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage (REFEC) | Rel-17 Stage-1 requirement for multi hop relay |
| 840045 | Audio-Visual Service Production (AVPROD) | Rel-17 Stage-1 requirement including AV production for PC5 |
| 850013 | Study on architecture enhancements for 3GPP support of advanced V2X services – Phase 2 (FS_eV2XARC_Ph2) | Rel-17 SA2 study item on eV2X architecture – Phase 2 |
| 830033 | Study on System enhancement for Proximity based Services in 5GS (FS_5G_ProSe) | Rel-17 SA2 study item on ProSe system |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

## 3	Justification

3GPP has been developing standards for sidelink as a tool for UE to UE direct communication required in various use cases since LTE. The first standard for NR sidelink is to be completed in Rel-16 by the work item “5G V2X with NR sidelink” where solutions including NR sidelink are being specified mainly for vehicle-to-everything (V2X) while they can also be used for public safety when the service requirement can be met.

Meanwhile, the necessity of NR sidelink enhancement has been identified. For V2X and public safety, the service requirements and operation scenarios are not fully supported in Rel-16 due to the time limitation, and SA works are ongoing on some enhancement in Rel-17 such as architecture enhancements for 3GPP support of advanced V2X services – Phase 2 (FS_eV2XARC_Ph2) and System enhancement for Proximity based Services in 5GS (FS_5G_ProSe). In addition, other commercial use cases related to NR sidelink are being considered in SA WGs via several work/study items such as Network Controlled Interactive Service (NCIS), Gap Analysis for Railways (MONASTERYEND), Enhanced Relays for Energy eFficiency and Extensive Coverage (REFEC), Audio-Visual Service Production (AVPROD). In order to provide a wider coverage of NR sidelink for these use cases and be able to provide the radio solutions in accordance with the progress in SA WGs, it is necessary to specify enhancements to NR sidelink in TSG RAN.

TSG RAN started discussions in RAN#84 to identify the detailed motivations and work areas for NR sidelink enhancements in Rel-17. Based on the latest summary in RP-192745, significant interest has been observed for the several motivations including the following:

- Power saving enables UEs with battery constraint to perform sidelink operations in a power efficient manner. Rel-16 NR sidelink is designed based on the assumption of “always-on” when UE operates sidelink, e.g., only focusing on UEs installed in vehicles with sufficient battery capacity. Solutions for power saving in Rel-17 are required for vulnerable road users (VRUs) in V2X use cases and for UEs in public safety and commercial use cases where power consumption in the UEs needs to be minimized.

- Enhanced reliability and reduced latency allow the support of URLLC-type sidelink use cases in wider operation scenarios. The system level reliability and latency performance of sidelink is affected by the communication conditions such as the wireless channel status and the offered load, and Rel-16 NR sidelink is expected to have limitation in achieving high reliability and low latency in some conditions, e.g., when the channel is relatively busy. Solutions that can enhance reliability and reduce latency are required in order to keep providing the use cases requiring low latency and high reliability under such communication conditions.

While several work areas have been identified in the discussion, some important principles were also discussed regarding the 3GPP evolution for NR sidelink. In dealing with different use cases in the evolution of NR sidelink, WGs should strive to achieve maximum commonality between commercial, V2X, and Critical Communication usage of sidelink in order to avoid duplicated solutions and maximize the economy of scale. In addition, enhancements introduced in Rel-17 should be based on the functionalities specified in Rel-16, instead of designing the fundamental NR sidelink functionality again in Rel-17.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to specify radio solutions that can enhance NR sidelink for the V2X, public safety and commercial use cases.

1. Sidelink evaluation methodology update: Define evaluation assumption and performance metric for power saving by reusing TR 36.843 and/or TR 38.840 (to be completed by RAN#89) [RAN1]

- Note: TR 37.885 is reused for the other evaluation assumption and performance metric. Vehicle dropping model B and antenna option 2 shall be a more realistic baseline for highway and urban grid scenarios.

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

- Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

  - Note: RAN2 work will start after RAN#89.

3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2]

- Define on- and off-durations in sidelink and specify the corresponding UE procedure

- Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other

- Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

4. Support of new sidelink frequency bands for single-carrier operations [RAN4]

- Support of new sidelink frequency bands should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum.

- The exact frequency bands are to be determined based on company input during the WI, considering both licensed and ITS-dedicated spectrum in both FR1 and FR2.

5. Define mechanism to ensure sidelink operation can be confined to a predetermined geographic area(s) for a given frequency range within non-ITS bands [RAN2].

- This applies areas where there is no network coverage.

6. UE Tx and Rx RF requirement for the new features introduced in this WI [RAN4]

7. UE RRM core requirement for the new features introduced in this WI [RAN4]

Enhancements introduced in Rel-17 should be based on the functionalities specified in Rel-16, and Rel-17 sidelink should be able to coexist with Rel-16 sidelink in the same resource pool. This does not preclude the possibility of operating Rel-17 sidelink in a dedicated resource pool.

The solutions should cover both the operating scenario where the carrier(s) is/are dedicated to ITS and the operating scenario where the carrier(s) is/are licensed spectrum and also used for NR Uu/LTE Uu operation.

The solutions should support the network control of NR sidelink as in Rel-16, i.e., NR Uu controls NR sidelink using Layer 1 and Layer 2 signalling and LTE Uu controls NR sidelink using Layer 2 signalling.

In ITS carriers, it is assumed that any co-channel coexistence requirements and mechanisms of NR sidelink with non-3GPP technologies will not be defined by 3GPP.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify the following requirements [RAN4]

- UE demodulation performance requirements

- UE RRM performance requirements

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| Type | TS/TR number | Title | For approval at TSG# | Remarks |
| --- | --- | --- | --- | --- |
| Internal | TR38.785 | User Equipment (UE) radio transmission and reception for enhanced NR sidelink | RAN#95 | Core Part Led by RAN4 WG Editor: Sang-Wook Lee, LG Electronics sangwook1.lee@lge.com This TR was actually just provided for approval to RAN#96 |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.201 | Introduction of NR sidelink enhancement | RAN#94 | Core Part |
| 38.211 | Introduction of NR sidelink enhancement | RAN#94 | Core Part |
| 38.212 | Introduction of NR sidelink enhancement | RAN#94 | Core Part |
| 38.213 | Introduction of NR sidelink enhancement | RAN#94 | Core Part |
| 38.214 | Introduction of NR sidelink enhancement | RAN#94 | Core Part |
| 38.215 | Introduction of NR sidelink enhancement | RAN#94 | Core Part |
| 38.331 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 38.321 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 38.304 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 38.306 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 38.300 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 37.322 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 38.323 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 38.324 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 38.101-1 | Introduction of NR sidelink enhancement | RAN#95 | Core part |
| 38.101-2 | Introduction of NR sidelink enhancement | RAN#95 | Core part |
| 38.101-3 | Introduction of NR sidelink enhancement | RAN#95 | Core part |
| 38.101-4 | Introduction of NR sidelink enhancement | RAN#97 | Performance part |
| 38.133 | Introduction of NR sidelink enhancement | RAN#95 | Core part |
| 38.133 | Introduction of NR sidelink enhancement | RAN#97 | Performance part |
| 36.331 | Introduction of cross-RAT control of NR sidelink enhancement | RAN#95 | Core Part |
| 37.985 | Introduction of NR sidelink enhancement | RAN#95 | Core Part |
| 37.885 | Introduction of additional evaluation methodology | RAN#94 | Core Part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Seungmin Lee

Company:	LG Electronics

Email: edison.lee@lge.com

## 7	Work item leadership

Primary: RAN1

Secondary: RAN2, RAN4

## 8	Aspects that involve other WGs

This work can be related to SA2 for the 3GPP architecture enhancement for PC5.

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

A.S.T.R.I.D S.A.

AT&T

BlackBerry

BMWi

Continental Automotive GmbH

Deutsche Telekom

ETRI

FirstNet

Fraunhofer HHI

Fraunhofer IIS

Futurewei

HiSilicon

Huawei

Intel

ITL

InterDigital

KT

Kyocera Corporation

Lenovo

LG Electronics

LG Uplus

Mediatek

Mitsubishi Electric

Motorola Mobility

NEC

Nokia

Nokia Shanghai Bell

OPPO

Qualcomm

Samsung

Sanechips

Sharp

Sierra Wireless

Sony

Spreadtrum

Sync Techno

TCL Communication

Xiaomi

UK Homeoffice

Vivo

ZTE