3GPP TSG-RAN Meeting #96	RP-221xxx

Budapest, Hungary, 6 – 10 June 2022

Title:	Summary for NB-IoT/eMTC support for Non-Terrestrial Networks (NTN)

Type:	Work Item Summary

Agenda Item:	X.X

WI Code: 	LTE_NBIOT_eMTC_NTN

Source:	MediaTek Inc., Gilles Charbit

Contact:	gilles.charbit@mediatek.com

This document is a contribution to TR 21.917 ("Release 17 Description; Summary of Rel-17 Work Items"). It is meant to be endorsed by the plenary. The "NB-IoT/eMTC support for Non-Terrestrial Networks (NTN)" work item adds or enhances a number of features in NB-IoT/eMTC in order to support Non-Terrestrial Networks (NTN).

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) | LTE_NBIOT_eMTC_NTN | R1 (Lead)/R2/R3 | RP-211601 | Gilles Charbit, MediaTek (RAN1) / Rene Faurie, Eutelsat (RAN2), Jiren Han, ZTE (RAN3) |

Introduction

The "NB-IoT/eMTC support for Non-Terrestrial Networks (NTN)" work item specifies enhanced features necessary for the support of Bandwidth reduced Low complexity (BL) UEs, UEs in enhanced coverage and NB-IoT UEs by Non-Terrestrial Networks (NTN).

Description

Overall architecture and general aspects:

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Non-terrestrial networks encompasses platforms that provide radio access through satellites in Geosynchronous orbits (GSO) as well as Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO).

As illustrated in Figure 1, non-terrestrial access is provided by means of an NTN payload, i.e. a network node on-board a satellite, and an NTN Gateway interconnected by a feeder link, the UE accessing NTN network services through the NTN payload via a service link.

Figure 1: Overall illustration of an NTN

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over NTN is only applicable to E-UTRA connected to EPC.

Only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported.

Timing and Synchronization:

The network broadcast ephemeris information and common Timing Advance (common TA) parameters in each NTN cell. A UE shall acquire its GNSS position as well as the satellite ephemeris and common TA before connecting to an NTN cell. To achieve uplink synchronisation, before performing random access, the UE shall autonomously pre-compensate the Timing Advance, as well as the frequency doppler shift by considering the common TA, the UE position and the satellite position through the satellite ephemeris. In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation, but the UE is not expected to perform GNSS acquisition. The UE does not perform any transmissions due to outdated satellite ephemeris, common TA or GNSS position based on timers. In connected mode, upon outdated satellite ephemeris and common Timing Advance, the UE re-acquires the broadcasted parameters and upon outdated GNSS position the UE moves to idle mode. The UEs may be configured to report Timing Advance at initial access or in connected mode. In connected mode triggered reporting of the Timing Advance is supported.

For downlink synchronization in case of NB-IoT, the two LSB of the ARFCN is signalled in MIB for bands for which a 200 kHz channel raster is not supported, and the legacy 100 kHz raster is used. Otherwise, for bands for which a 200 kHz channel raster is supported, there is no signalling of ARFCN information in MIB.

Downlink and uplink timings are frame aligned at the uplink time synchronization reference point (RP). To accommodate the long propagation delays in NTN, the timing relationships are enhanced by the support of two scheduling offsets:  and  as illustrated in Figure 2:

Figure 2 Timing relationship parameters

Uplink segmented transmission is supported for uplink transmission with repetitions.  The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for BL UEs and UEs in enhanced coverage and NPUSCH/NPRACH for NB-IoT from one segment to the next segment. The configuration of uplink transmission segment is indicated on SIB for initial access and can be re-configured by RRC signalling.

Discontinuous coverage and assistance information:

As a satellite moves on a specified orbit, for example in case of a NGSO satellite, the satellite beam(s) coverage area may move and cover different portions of a geographical area due to the orbital movement of the satellite. As a consequence, a UE located in the concerned geographical area may experience a situation of discontinuous coverage, due to e.g. a sparse satellite constellation deployment.

The network may broadcast assistance information relating to the serving satellite and other satellites of the constellation to enable UEs to predict upcoming satellites fly-over periods and save power during periods of no coverage. The broadcast assistance information includes SGP4 ephemeris elements based on the TLE (Two-Line Elements) sets industry standard. Additional assistance information, such as coverage footprint parameters, may also be optionally broadcast by the network.

Predicting out of coverage and in coverage periods is up to UE implementation. When out of coverage, the UE is not required to perform Access Stratum (AS) functions.

Mobility Management:

The network may broadcast more than one Tracking Area Code (TAC) per PLMN in a cell in order to reduce the signalling load at cell edge in NTN, in particular for Earth-moving cell coverage. The AS layer indicates all received TACs for the selected PLMN to the NAS layer. The network may update the UEs upon TAC removal. UEs may by UE implementation also check whether a TAC has been removed from the TACs broadcast by the network.

For quasi-Earth-fixed cells, timing information on when the cell is going to stop serving the area may be broadcast by the network. This may be used by the UE to start measurements on neighbour cells before the broadcast stop time of the serving cell, while the exact start of the measurements is up to UE implementation.

Radio link failure and RRC connection re-establishment are supported in NTN. To enable mobility in NTN, the network provides target cell satellite parameters needed to access the NTN cell in the handover command. Conditional handover is supported for BL UEs and UEs in enhanced coverage.

Feeder-link switch-over:

The NTN Control function determines the point in time when a feeder link switch over between two eNBs is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UE(s)' context between the two eNBs at feeder link switch over is performed by means of either S1 based or X2 based handover, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

Network-interfaces signalling aspects:

The Cell Identity in NTN corresponds to a fixed geographical area identified by a Mapped Cell ID, irrespective of the orbit of the NTN payload or of the type of the service link. For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and the Core Network (e.g. pre-configured depending on operator's policy, or based on implementation). For a BL UE or a UE in enhanced coverage or a NB-IoT UE that supports S1-U data transfer or User Plane CIoT EPS optimisation, the eNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Pre-configuration of special mapped cell identifiers may be used to indicate areas outside the serving PLMN's country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE's location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.

MME(Re-)Selection by eNB:

For a RRC_CONNECTED UE, when the eNB is configured to ensure that the BL UE or the UE in enhanced coverage is using an MME that serves the country in which the UE is located. If the eNB detects that a BL UE or a UE in enhanced coverage is in a different country from that served by the serving MME, it should perform an S1 handover to change to an appropriate MME or initiate an UE Context Release Request procedure towards the serving MME (in which case the MME may decide to detach the UE).

For a RRC_CONNECTED NB-IoT UE, when the eNB is configured to ensure that the NB-IoT UE is using an MME that serves the country in which the UE is located. If the eNB detects that the UE is in a different country to that served by the serving MME, it should initiate a UE Context Release Request procedure towards the serving MME (in which case the MME may decide to detach the UE).

O&M Requirements:

The NTN related parameters shall be provided by O&M to the eNB providing non-terrestrial access, as specified in TS 38.300 for NR NTN.

Support for E-UTRAN:

For S1 and X2 interfaces,  codepoints in the RAT Restriction Information IE (in the Handover Restriction List) allow the selection of different constellation types, i.e. “LEO”, “MEO”, “GEO”, “OTHERSAT” for satellite access.

For S1 interface, additional codepoints are added to the RAT Type IE associated with a TAC, i.e. “NBIoT-LEO”, “NBIoT-MEO”, “NBIoT-GEO”, “NBIoT-OTHERSAT”, “EUTRAN-LEO”, “EUTRAN-MEO”, “EUTRAN-GEO”, “EUTRAN-OTHERSAT”.

For S1 interface, the UE Context Reference at Source IE (eNB UE S1AP ID) is introduced in the Source eNB to Target eNB Transparent Container.

For S1 interface, a new cause value is added signaling that a UE is not within the serving area of its current PLMN.

For TAC reporting over S1, the LTE NTN TAI Information IE and the semantics description for the TAI IE are added in the User Location Information IE;  the LTE NTN TAI Information IE and the semantics description for the TAI IE are added to the eNB CP RELOCATION INDICATION, HANDOVER NOTIFY, PATH SWITCH REQUEST, INITIAL UE MESSAGE, UPLINK NAS TRANSPORT, and LOCATION REPORT messages.

References

[1] TS 36.300

[2] TS 38.300

[3] TS 36.306

[4] TS 36.413

[5] TS 36.423