3GPP TSG RAN WG1 #109-e			R1-2203020

e-Meeting, May 9th – 20th, 2022

3GPP TSG-RAN WG4 Meeting # 102-e	                               			R4-2206905

Electronic Meeting, 21st Feb.-3rd Mar., 2022

Title:	Reply LS on combination of open and closed loop TA control in NTN

Response to:	R4-2119413 (R1-2110604) LS on Combination of open and closed loop TA control in NTN

Release:	Rel-17

Work Item:	NR_NTN_solutions-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	CH Park

E-mail Address:	chparkqc@qti.qualcomm.com

Attachments:

# 1. Overall Description:

RAN4 would like to provide further updates on agreements about double-correction issue. RAN4 agreed to use a framework of gradual timing adjustment requirement to alleviate the impact of double-correction in NTN, i.e. Option 2 in R4-2120417. The agreement below is a gradual timing adjustment requirement for NTN UEs.

Gradual timing adjustment requirement

Agreement:

    - When the transmission timing error between the UE and the reference timing exceeds ±Te_NTN then the UE is required to adjust its timing to within ±Te_NTN.

    - The reference timing shall be (NTA+NTA,UE-specific+NTA,common+NTA,offset)×Tc before the downlink timing of the reference cell.

    - All adjustments made to the UE uplink timing shall follow these rules:

      - The maximum amount of the magnitude of the timing change, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be Tq.

      - The minimum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common during the last one second, shall be Tp per second.

      - The maximum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common during the last 200ms, shall be Tq per 200 ms.

      - where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table.

    - The maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table below.

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | [5.5]*64*Tc | [5.5]*64*Tc |
|  | 30 | [5.5]*64*Tc | [5.5]*64*Tc |
|  | 60 | NA | NA |
| NOTE: Tc is the basic timing unit defined in TS 38.211 |  |  |  |

  - (Note) Te_NTN is a timing error limit of NTN UE initial transmission timing error.

# 2. Actions:

To: RAN2

ACTION: RAN4 kindly ask RAN1 to take the above information into account in the relevant work.

# 3. Date of Next RAN4 Meetings:

TSG-RAN WG4 Meeting #103-e     16 – 27 May 2022		Electronic Meeting

TSG-RAN WG4 Meeting #104-e     22 – 26 Aug. 2022		Toulouse, FR