3GPP TSG RAN WG1 #109-e			R1-2205406

e-Meeting, May 9th – 20th, 2022

Title:	LS on updates of RRC parameters for Rel-17 positioning enhancements

Response to:

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN1

To:	RAN2, RAN3

Cc:	RAN4

Contact Person:

#### Name:	Ren Da

E-mail Address:	renda@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: R1-2205539

1. Overall Description:

RAN1 would like to inform RAN2 and RAN3 that RAN1 has updated the list of higher-layer parameters for Rel-17 positioning enhancements based on the following agreements made in RAN1#109e. The updates of the list for Rel-17 positioning enhancements as compared to the previous list in RAN1 LS R1-2202542 are highlighted in red (see attached).

Agreement The agreement from RAN1#108e on LMF initiated request of on-demand PRS is amended as follow: - Note: no RAN1 specification impact is expected - send the agreement as part of the reply LS to RAN3 Agreement (RAN#109e) - From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per UETRP - Start/end time of DL PRS transmission - either per resource, or per resource set, or per UETRP - ON/OFF indicator (for LMF initiated request only). Agreement - Support up to 32 measurement instances in a single measurement report. - Inform RAN2/RAN3 on RAN1’s decision Agreement Support the LMF to request the Rx beam sweeping factor. Agreement The request from LMF on the Rx beam sweeping factor is a single bit per positioning method, which can take two values. - Value 1: Equal to the UE’s reported Rx beam sweeping factor in the corresponding capability for the band UE supports the feature, and equal to 8 for the FR2 bands that UE does not support the feature. - Value 2: Equal to 8 (default assumption) for FR2 bands. - The bit value should be set to the same across DL-TDOA, DL-AoD, and Multi-RTT for hybrid positioning. Agreement Adopt the following changes to the previous agreement made in RAN1#105e - For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) theN PRS RSRPP of the first path, where N can be {1, 2, 3, …, 24}.

In addition, RAN1 also reached the following conclusion on the maximum number of PPWs that can be activated/deactivated by a single DL MAC-CE.

Conclusion It is RAN1 understanding that the maximum number of PPWs that can be activated/deactivated by a single DL MAC-CE is up to RAN2. · The previous agreement of maximum number of PPWs that can be activated/deactivated by a single DL MAC-CE is reverted. Note: It means from RAN1 perspective, RAN1 intends to agree that up to 4 PPWs can be activated/deactivated by a single DL MAC CE as specified in TS 38.321 v17.0.0.

2. Actions:

To RAN2 and RAN3

ACTION: 	RAN1 would like to kindly ask RAN2 and RAN3 to take the information into account in their design of the corresponding higher-layer parameters in Rel-17.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #110	22 Aug – 26 Aug 2022		Toulouse, France

TSG-RAN WG1 Meeting #110-e-Bis	10 Oct – 19 Oct 2022		E-Meeting