3GPP TSG RAN WG1 #109-e			R1-2205507

e-Meeting, May 9th – 20th, 2022

Title:	LS on Rel-17 URLLC/IIoT RRC parameter updates

Response to: 	-

Release:	Rel-17

Work Item:	NR_IIOT_URLLC_enh

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Klaus Hugl

E-mail Address:	klaus.hugl@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R1-2205505 (Updated RRC parameter sheet for Rel-17 URLLC / IIoT)

# 1. Overall Description:

RAN1 reached the agreements during RAN1#109-e with RRC impact, which are reflected in attached RRC parameter sheet that shows in RED color the new updates in R1-2202759 corresponding to the following agreements for Rel-17 URLLC/IIoT sheet:

- In the enhanced Type 3 HARQ-ACK codebook DCI field configuration for the primary PUCCH group, a mistake in the description has been recognized which is corrected in row 17 of the updated RRC parameter sheet based on the following agreement:

Agreement - The RRC parameter description for the RRC parameter pdsch-HARQ-ACK-enhType3DCIfield in the RAN1 RRC parameter sheet for Rel-17 URLLC/IIoT is updated as follows: pdsch-HARQ-ACK-enhType3DCIfield; Enables the enhanced Type 3 CB through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in the primarysecondary cell group if the more than one enhanced Type HARQ-ACK codebook is configured for the primary PUCCH cell group..

- The proper maximum values for the RRC parameter offsetUE in SemiStaticChannelAccessConfigUE have been changed which is reflected in row 44 of the updated RRC parameter sheet based on the following agreement:

Agreement Include in the IIoT/URLLC RRC parameters LS to RAN2, the following clarification: - The proper maximum values for the parameter offsetUE in SemiStaticChannelAccessConfigUE are 139/279/559 for 15/30/60kHz SCS rather than 279/559/1119, and the proper value range is thus 559 rather than 1119.

2. Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully asks RAN2 to take the answers to the questions into account in your further work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #110 	22 – 26 August 2022		Toulouse, France

TSG-RAN WG1 Meeting #110-e Bis	10 – 19 October 2022		Electronic Meeting