3GPP TSG-RAN WG1 Meeting #110		R1-2205804

Toulouse, France, August 22 – 26, 2022

Title:	Draft reply LS on RB set configuration for IAB

Response to:	R3-224031/R1-2205705

Release:	Rel-17

Work Item:	NR_IAB-Core

Source:	Huawei [RAN WG1]

To:	RAN WG3

CC :

Contact Person:

#### Name:	Xinghua Song

E-mail Address:	songxinghua@huawei.com

1. Overall Description:

RAN1 would like to thank RAN3 for the LS (R1-2205705) on RB set configuration in F1AP and XnAP specification. Regarding to the two questions, RAN1 would like to provide following answer:

Question 1: Whether the RB set needs to be configurable to the IAB-donor-DU?

Answer: No. The original intention of supporting frequency domain H/S/NA configuration of DU is to address the resource multiplexing between MT and DU. Since the IAB-donor-DU do not have a MT, hence the RB set is not applicable to IAB-donor-DU.

Question 2: Whether the current F1AP signalling about RB set size is clear enough. If not, which kind of clarification should be added?

Answer: There is no need to add any more clarifications for RB set configuration. The agreements "N is at least the # PRBs corresponding to the MT’s configured #PRB of an RBG" only clarifies the candidate values for RB set size not a configuration restriction.

2. Actions:

To RAN3

ACTION: RAN1 respectfully asks RAN3 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1#110bis-e 		10 October – 19 October 2022       				e-Meeting

TSG-RAN WG1#111 			14 November – 18 November 2022       			Canada