3GPP TSG-RAN WG1 Meeting #110 	R1-2205871

Toulouse, France, August 22 – 26, 2022

Agenda Item:	9.5.2.3

Source:	Huawei, HiSilicon

Title:	Evaluation and solutions for LPHAP

Document for:	Discussion and decision

# Introduction

In RAN1#109-e, preliminary evaluation assumptions were agreed to identify whether the existing RAN functionality can support LPHAP service and whether enhancements are needed.

In this paper, we present our evaluations for the battery life of LPHAP and propose the enhancements to overcome the limitation of the existing RAN functionality.

# Remaining issues of the evaluation assumptions

## Battery life evaluation methodology

In RAN1#109-e, the following agreement was made on how to identify the performance gap.

Agreement - Study further at least the following models and parameter values of conversion between the relative power unit and the battery life to identify the performance gap: - Alt. 1: battery life is used as the metric to identify the gap - Example: T2= P1*T1X* C2C1* 1P2 GapBatLife= T2req-T2 - Alt. 2: relative power unit is adopted as the metric to identify the gap - Example: P2req= P1*T1X* C2C1* 1T2req GapPowUnit= P2req-P2 in which - C1 is the battery capacity of the reference device; - T1 is the battery life of the reference device; - P1 is the relative power unit obtained based on the reference traffic type; - X is the percentage of the power consumed by the reference traffic type; - C2 is the battery capacity of the LPHAP device; - P2 is the evaluated relative power unit of the LPHAP device; - P2_req is the target relative power unit of the LPHAP device; - T2_req is the target battery life of the LPHAP device - Examples of these parameters are provided as follows: C1; T1; X; reference traffic type; C2; T2req. [4500] mAh; [10] hours; [20] %; [FTP (model 3)]; [800] mAh; [12] months.

Two alternatives are under consideration, which provide two perspectives from describing the gap.

Alt.1 seems straightforward and flexible when other non-positioning related operations are added, which discounts the overall battery life.

Alt.2, on the other hand, may also be considered to evaluate the maximum allowed power unit for any “ultra-deep sleep” if T2req needs to be met.

On the table of the parameters, we think the current table provides a good starting point. For T1 value, we think this is somehow related to configuration of C-DRX, and would suggest the following modification.

| C1 | T1 | X | reference traffic type | C2 | T2req |
| --- | --- | --- | --- | --- | --- |
| 4500 mAh | [8~12] hours under the reference C-DRX configuration of 160ms DRX cycle 10ms on duration 100ms inactivity timer | 20 % | FTP (model 3) | 800 mAh | 12 months |

In summary, we have the following proposal.

- Proposal 1: Adopt Alt.1 to use the battery life as the metric to identify the gap.

Alt.2 can be considered to evaluate the maximum allowed power unit for any “ultra-deep sleep” to meet the battery life target.

Adopt the following table as the starting point for evaluating the battery life of LPHAP.

| C1 | T1 | X | reference traffic type | C2 | T2req |
| --- | --- | --- | --- | --- | --- |
| 4500 mAh | [8~12] hours under the reference C-DRX configuration of 160ms DRX cycle 10ms on duration 100ms inactivity timer | 20 % | FTP (model 3) | 800 mAh | 12 months |

## Ultra-deep sleep

The following agreement was reached on the sleep type in the evaluation.

Agreement - Adopt the power consumption model, additional transition energy and total transition time of the three sleep types (deep sleep, light sleep, and micro sleep) in TR38.840 as the evaluation baseline: - FFS: whether/how an additional new ultra-deep sleep mode can be considered in the evaluation of potential solutions to maximize the battery life, including the determination of the relative power, additional transition energy and total transition time, if necessary.

Based on our evaluation to be shown in section 3 of the paper, we think ultra-deep sleep is essential for the device to meet the LPHAP target, because the existing deep sleep consumes more than 90% of the energy.

Ultra-deep sleep is not new to either 3GPP or positioning industry.

In 3GPP, during the CIoT study TR 45.820, the evaluation assumes PSS in the power evaluation, and it has been stated in the TR that

PSS denotes a Power Saving State such as that achieved with the Rel-12 Power Save Mode feature. In Idle, the device may be consuming more power than in the PSS state because, for example, it is maintaining a more accurate time/frequency synchronization with the network.

Figure 1 Example of events affecting energy consumption for IP packet exchange (Figure 5.4-1 [3])

The power of PSS state is assumed as 15uW according to table 5.4-1 [3], which translates to 5uA if the voltage is 3.3V.

Table 1 Key input parameters for energy consumption analysis (Table 5.4-1 [3])

| (1) Battery capacity (Wh) | (2) Battery power during Tx (mW) | (3) Battery power for Rx (mW) | (4) Battery power when Idle but not in PSS (mW) | (5) Battery power in Power Save State (PSS) (mW) | (6) Time between end of IP packet carrying "report" and start of IP packet carrying "ack" on radio (ms) | (7) Number of reports per day |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  |  |  | [0,015] | 1000 |  |

In positioning industry, as shown in Figure 2 from the Decawave 1000 data sheet for the UWB product, the sleep current and deep sleep current can be as low as 1uA and 50nA, respectively, which helps UWB to claim the battery life of more than a year. Note that the current of the sleep mode for the UWB device is less than a thousandth of that in the IDLE mode.

Figure 2 Operating States and their effect on power consumption (Table 24 [4])

- Observation 1: Both CIoT and UWB considered a very deep sleep mode with current in the order of uA, which is much lower than the assumption of the current in the deep sleep in TR 38.840.

For the new ultra-deep sleep, we think the following relative power, transition energy, and total transition time can be considered.

- Proposal 2: Define a new ultra-deep sleep type for the evaluation of LPHAP.

The power unit of ultra-deep sleep can take 0.01 power unit as the starting point.

The transition energy is 450 power unit.

The transition time is 25ms.

## Paging

RAN1#109-e agreed to the following modeling of paging. Some parameters were worth revisiting.

- SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%;

It was agreed that paging is with 2ms duration, but in general the paging search space set configuration could follow the corresponding search space 0 configuration that two consecutive slots for each associated with SSB according to the description of TS 38.213 and TS 38.304.

For simplicity, we suggest to consider the following paging configuration at least for FR1, where the PO/paging search space sets are half frame offset from the associated with SSB, i.e. M=0.5 sec.

| Paging | Radio Frame 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Slot | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| DSU | D | D | D | D | D | D | D | S | U | U | D | D | D | D | D | D | D | S | U | U |
| SSB | 0/1 | 2/3 | 4/5 | 6/7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SS#0 | SS#0 configuration 5 for SS/RMSI multiplexing pattern 1 with O=5 (offset = 5 ms) M=0.5 Number of SS sets per slot=2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SS#0 monitoring for SSB0 | SSB0 |  |  |  |  |  |  |  | UL | UL | Paging | Paging |  |  |  |  |  |  | UL | UL |
| SS#0 monitoring for SSB1 | SSB1 |  |  |  |  |  |  |  |  |  | Paging | Paging |  |  |  |  |  |  |  |  |
| SS#0 monitoring for SSB2 |  | SSB2 |  |  |  |  |  |  |  |  |  | Paging | Paging |  |  |  |  |  |  |  |
| SS#0 monitoring for SSB3 |  | SSB3 |  |  |  |  |  |  |  |  |  | Paging | Paging |  |  |  |  |  |  |  |
| SS#0 monitoring for SSB4 |  |  | SSB4 |  |  |  |  |  |  |  |  |  | Paging | Paging |  |  |  |  |  |  |
| SS#0 monitoring for SSB5 |  |  | SSB5 |  |  |  |  |  |  |  |  |  | Paging | Paging |  |  |  |  |  |  |
| SS#0 monitoring for SSB6 |  |  |  | SSB6 |  |  |  |  |  |  |  |  |  | Paging | Paging |  |  |  |  |  |
| SS#0 monitoring for SSB7 |  |  |  | SSB7 |  |  |  |  |  |  |  |  |  | Paging | Paging |  |  |  |  |  |

When UE wakes up from sleep, UE first attempts to receive the SSB transmitted within a 2-msec duration (4 slots for 8 SSBs), finds the best SSB, and receive paging only in the slots that corresponds to the best SSB.

For the two paging slots, UE attempts to decode the paging DCI, with the 10% probability (group paging rate) that UE decodes a valid DCI transmission that schedules a PDSCH that does not contain the target UE ID assuming that the UE within the LPHAP device will never be paged.

In this regard, we think that at least for 30kHz SCS, the paging duration can be reduced to 1msec (2 slots) to follow the current common search space configuration. During the 2-slot duration, at least one slot is PDCCH only, while the other slot is 90% probability of PDCCH only and 10% probability of PDCCH+PDSCH.

In the worse case scenario, UE needs to receive paging in slots 13 and 14 that corresponds to SSB#6 and SSB#7, and between SSB and paging reception, UE may either stay (micro-)sleep or perform positioning related procedures.

Proposal 3: For paging reception, update the previous assumption that

UE attempts to receive paging DCI in two consecutive slots associated with the SSB.

One slot is treated as PDCCH-only.

The other slot is treated as 90% PDCCH-only and 10% PDCCH+PDSCH.

Note: This reverts the previous agreement of paging receive duration of 2ms.

The power unit for paging reception can be calculated as:

50+50×90%+120×10%2=53.5

Proposal 4: For the two slots associated with paging reception, the mean power unit per slot is 53.5.

# Power consumption and battery life evaluation

## Reference device

The following agreements were made with regards to the reference device in RAN1#109-e [1].

Agreement A reference device (e.g., a mobile phone) with reference traffic type, reference battery capability, and reference battery life is defined for the purpose of identification of the performance gap that achieved by the Rel-17 RRC_INACTIVE state positioning baseline and the target battery life of LPHAP use case 6.

The assumption of the reference traffic for the reference device is listed in Table 2.

Table 2 Reference traffic for the reference device

| Parameters | Reference traffic |
| --- | --- |
| Traffic model | FTP (model 3) |
| Packet Size | 0.5 Mbytes (TR 38.840) |
| Mean interval | 200 ms |
| DRX setting | Period: 160ms On duration: 8ms Inactivity timer: 100ms |
| Positioning period | N/A |
| RRM | Not accounting for RRM |
| Deep sleep power unit | 1 |

The remaining power evaluation assumptions follow the agreements and TR 38.840.

Due to the randomness of the traffic type, Monte-Carlo method is used in the evaluation. The power consumption can be evaluated as below.

State Time TimeRatio Power PwrRatio LightSleep 2903 0.14515% 69760 0.09395% MicroSleep 214 0.01070% 9630 0.01297% PDCCH_Only 689137 34.45685% 68913700 92.80733% PDCCH_PDSCH 4896 0.24480% 1468800 1.97806% DeepSleep 1302850 65.14250% 3792700 5.10770% Total Number of State: 5 Total Number of Slots: 2000000 Total Power: 74254590

P1, the power consumption rate, for the reference device and other assumptions of battery volume, power percentage used for the reference traffic, and battery life are listed in Table 3.

Table 3 Power consumption rate for the reference device

| Power consumption rate | P1 = 74254590 per 1000s = 74000 [PU/s] |
| --- | --- |
| Battery volume for the traffic | C1=4500 mAh X=20% |
| Battery life | T1=12 hours |

## Rel-17 INACTIVE state positioning baseline

### UL positioning

The assumption of the traffic for the Rel-17 UL positioning in RRC_INACTIVE state is listed in Table 4.

Table 4 Traffic for UL positioning in RRC_INACTIVE state for Rel-17

| Parameters | Reference traffic |
| --- | --- |
| Traffic model | N/A |
| Packet Size | N/A |
| Mean interval | N/A |
| DRX setting | I-DRX with 1.28s I-DRX with 10.24s |
| Positioning period | 1.28s for I-DRX with 1.28s 10.24s for I-DRX with 1.28s and 10.24s |
| RRM | Not accounting for RRM |
| Deep sleep power unit | 1 |

The timeline for UL positioning with paging is modelled as below in Figure 3.

(Ultra-)Deep sleep

Ramp down

SRS [23dBm]

SSB

Micro sleep

Ramp up

(Ultra-)Deep sleep

2ms

1.5ms

2.5ms

Paging

Micro sleep

1ms

0.5ms

(Ultra-)Deep sleep

Ramp down

SSB

Ramp up

(Ultra-)Deep sleep

2ms

4.5ms

Paging

Micro sleep

1ms

Figure 3 UL positioning timeline with paging (SRS+Paging and paging only)

The power consumption for the SRS+paging and paging only are shown in Table 5.

Table 5 Power consumption for SRS+paging and paging only

| SRS Tx + Paging | Duration (ms) | Power |
| --- | --- | --- |
| Micro sleep | 4 | 360 |
| SSB Sync | 2 | 200 |
| SRS Tx (23dBm) | 0.5 | 210 |
| Paging | 1 | 107 |
| Total | 7.5 | 877 |
| Paging | Duration (ms) | Power |
| Micro sleep | 4.5 | 405 |
| SSB Sync | 2 | 200 |
| Paging | 1 | 107 |
| Total | 7.5 | 712 |

The power consumption for the following three cases are shown.

SRS periodicity 1.28 sec, and I-DRX periodicity 1.28 sec

SRS periodicity 10.24 sec, and I-DRX periodicity 1.28 sec

SRS periodicity 10.24 sec, and I-DRX periodicity 10.24 sec

Table 6 Power consumption for Rel-17 UL positioning in RRC_INACTIVE state

| INACTIVE state UL every 1.28s (SRS 1.28s, paging 1.28s) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Power state | Power unit | Duration | Instances | Sum power | Sum duration | Power ratio |
| Ramp up + down | 450 | 20 | 8 | 3600 | 160 | 11.7432% |
| SRS Tx + Paging | 877 | 7.5 | 8 | 7016 | 60 | 22.8862% |
| Deep sleep | 2 | 1 | 10020 | 20040 | 10020 | 65.3706% |
| Total |  |  |  | 30656 | 10240 | 100.0000% |
| INACTIVE state UL every 10.24s (SRS 10.24s, paging 1.28s) |  |  |  |  |  |  |
| Power state | Power unit | Duration | Instances | Sum power | Sum duration | Power ratio |
| Ramp up + down | 450 | 20 | 8 | 3600 | 160 | 12.2030% |
| SRS Tx + paging | 877 | 7.5 | 1 | 877 | 7.5 | 2.9728% |
| Paging | 712 | 7.5 | 7 | 4984 | 52.5 | 16.8943% |
| Deep sleep | 2 | 1 | 10020 | 20040 | 10020 | 67.9299% |
| Total |  |  |  | 29501 | 10240 | 100.0000% |
| INACTIVE state UL every 10.24s (SRS 10.24s, paging 10.24s) |  |  |  |  |  |  |
| Power state | Power unit | Duration | Instances | Power | Sum duration | Power ratio |
| Ramp up + down | 450 | 20 | 1 | 450 | 20 | 2.0688% |
| SRS Tx + Paging | 877 | 7.5 | 1 | 877 | 7.5 | 4.0318% |
| Deep sleep | 2 | 1 | 10212.5 | 20425 | 10212.5 | 93.8994% |
| Total |  |  |  | 21752 | 10240 | 100.0000% |

### DL positioning

The assumption of the traffic for the Rel-17 UL positioning in RRC_INACTIVE state is listed in Table 4.

Table 7 Traffic for DL positioning in RRC_INACTIVE state for Rel-17

| Parameters | Reference traffic |
| --- | --- |
| Traffic model | N/A |
| Packet Size | N/A |
| Mean interval | N/A |
| DRX setting | I-DRX with 1.28s I-DRX with 10.24s |
| Positioning period | 1.28s for I-DRX with 1.28s 10.24s for I-DRX with 1.28s and 10.24s |
| RRM | Not accounting for RRM |
| Deep sleep power unit | 1 |

The timeline for DL positioning with paging is modelled as below in Figure 4.

(Ultra-)Deep sleep

Ramp down

SSB

Ramp up

(Ultra-)Deep sleep

2ms

4.5ms

Paging

Micro sleep

1ms

(Ultra-)Deep sleep

Ramp down

SSB

Ramp up

(Ultra-)Deep sleep

2ms

4.5ms

Paging

Micro sleep

1ms

PRS

Light sleep

19.5ms

Figure 4 DL positioning timeline with paging (PRS+Paging and paging only)

The power consumption for the PRS+paging and paging only are shown in Table 8.

Table 8 Power consumption for PRS+paging and paging only

| PRS Rx + Paging | Duration (ms) | Power |
| --- | --- | --- |
| PRS Rx | 0.5 | 120 |
| Light sleep | 13.5 | 540 |
| Ramp up/down for light sleep | 6 | 100 |
| SSB Sync | 2 | 200 |
| Micro sleep | 4.5 | 405 |
| Paging | 1 | 107 |
| Total | 27.5 | 1472 |
| Paging | Duration (ms) | Power |
| Micro sleep | 4.5 | 405 |
| SSB Sync | 2 | 200 |
| Paging | 1 | 107 |
| Total | 7.5 | 712 |

The power consumption for the following three cases are shown in Table 9.

PRS periodicity 1.28 sec, and I-DRX periodicity 1.28 sec for UE-based DL positioning

PRS periodicity 10.24 sec, and I-DRX periodicity 1.28 sec for UE-based DL positioning

PRS periodicity 10.24 sec, and I-DRX periodicity 10.24 sec for UE-based DL positioning

Table 9 Power consumption for Rel-17 DL positioning in RRC_INACTIVE state

| INACTIVE state DL every 1.28s (PRS 1.28s, paging 1.28s) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Power state | Power unit | Duration | Instances | Sum power | Sum duration | Power ratio |
| Ramp up + down | 450 | 20 | 8 | 3600 | 160 | 10.2576% |
| PRS Rx + Paging | 1472 | 27.5 | 8 | 11776 | 220 | 33.5537% |
| Deep sleep | 2 | 1 | 9860 | 19720 | 9860 | 56.1887% |
| Total |  |  |  | 35096 | 10240 | 100.0000% |
| INACTIVE state DL every 10.24s (PRS 10.24s, paging 1.28s) |  |  |  |  |  |  |
| Power state | Power unit | Duration | Instances | Sum power | Sum duration | Power ratio |
| Ramp up + down | 450 | 20 | 8 | 3600 | 160 | 11.9776% |
| PRS Rx + paging | 1472 | 27.5 | 1 | 1472 | 27.5 | 4.8975% |
| Paging | 712 | 7.5 | 7 | 4984 | 52.5 | 16.5824% |
| Deep sleep | 2 | 1 | 10000 | 20000 | 10000 | 66.5425% |
| Total |  |  |  | 30056 | 10240 | 100.0000% |
| INACTIVE state DL every 10.24s (PRS 10.24s, paging 10.24s) |  |  |  |  |  |  |
| Power state | Power unit | Duration | Instances | Power | Sum duration | Power ratio |
| Ramp up + down | 450 | 20 | 1 | 450 | 20 | 2.0173% |
| PRS Rx + Paging | 1472 | 27.5 | 1 | 1472 | 27.5 | 6.5988% |
| Deep sleep | 2 | 1 | 10192.5 | 20385 | 10192.5 | 91.3839% |
| Total |  |  |  | 22307 | 10240 | 100.0000% |

It is worth noting that for DL positioning, we only considered UE based positioning methods which seems to approach the low power consumption target, and for UE-assisted positioning, additional SDT procedure may be required to report the PRS measurement. However, SDT procedure usually lasts longer than a single UL transmission, because UE needs to wait for the RRCRelease message to enter the sleep mode, prior to which UE monitors PDCCH as in the RRC_CONNECTED state.

### Summary of Rel-17 baseline

With the evaluation results shown in Table 3, Table 6, and Table 9, the battery life estimation for Rel-17 baseline UL and DL positioning in RRC_INACTIVE state can be found in Table 10.

Table 10 Rel-17 baseline power consumption

| Case | P (PU/s) | C (mAh) | X | T (hour) |
| --- | --- | --- | --- | --- |
| Ref | 74000 | 4500 | 20% | 12 |
| UL (1.28/1.28) | 2993.75 | 800 | 100% | 264 |
| UL (10.24/1.28) | 2880.957031 | 800 | 100% | 274 |
| UL (10.24/10.24) | 2124.21875 | 800 | 100% | 372 |
| DL (1.28/1.28) | 3427.34375 | 800 | 100% | 230 |
| DL (10.24/1.28) | 2935.15625 | 800 | 100% | 269 |
| DL (10.24/10.24) | 2178.417969 | 800 | 100% | 362 |

- Observation 2: Rel-17 baseline UL and DL positioning in RRC_INACTIVE state can achieve 200 – 300 hours battery life, which cannot meet the LPHAP requirements.

## Rel-18 enhancements for evaluation

### Ultra-deep sleep

It can be observed that the percentage of power consumption contribution of deep sleep is more than 50%, and in some extreme cases, more than 90%.

Table 11 Contribution of deep sleep power to the overall power consumption

| Case | Percentage of power consumption for deep sleep |
| --- | --- |
| UL (1.28/1.28) | 65.3706% |
| UL (10.24/1.28) | 67.9299% |
| UL (10.24/10.24) | 93.8994% |
| DL (1.28/1.28) | 56.1887% |
| DL (10.24/1.28) | 66.5425% |
| DL (10.24/10.24) | 91.3839% |

It should be noted that even if UE is only staying in the deep sleep mode, the battery life cannot meet LPHAP requirement, with 320 hours.

|  | P (PU/s) | C (mAh) | X | T (hour) |
| --- | --- | --- | --- | --- |
| Ref | 74000 | 4500 | 20% | 10 |
| Sleep only | 2000 | 800 | 100% | 328.8888889 |

Observation 3: The power consumption of deep sleep is the bottleneck for further extending the battery life for LPHAP.

By adopting the following ultra-deep sleep assumption, the power evaluation for 10.24 positioning interval with 10.24s I-DRX period is shown in Table 12.

| Sleep type | Relative power | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- | --- |
| Ultra-deep sleep | 0.01 | 450 | 25 ms |
| Deep sleep | 1 | 450 | 20 ms |
| Light sleep | 20 | 100 | 6 ms |
| Micro sleep | 45 | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |  |

Table 12 Power consumption for UL and DL positioning with ultra-deep sleep

| INACTIVE state UL every 10.24s (SRS 10.24s, paging 10.24s) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Power state | Power unit | Duration | Instances | Power | Sum duration | Power ratio |
| Ramp up + down | 450 | 25 | 1 | 450 | 25 | 29.3897% |
| SRS Tx + Paging | 877 | 7.5 | 1 | 877 | 7.5 | 57.2772% |
| Ultra-deep sleep | 0.02 | 1 | 10207.5 | 204.15 | 10207.5 | 13.3331% |
| Total |  |  |  | 1531.15 | 10240 | 100.0000% |
| INACTIVE state DL every 10.24s (PRS 10.24s, paging 10.24s) |  |  |  |  |  |  |
| Power state | Power unit | Duration | Instances | Power | Sum duration | Power ratio |
| Ramp up + down | 450 | 25 | 1 | 450 | 25 | 21.1690% |
| PRS Rx + Paging | 1472 | 27.5 | 1 | 1472 | 27.5 | 69.2461% |
| Ultra-deep sleep | 0.02 | 1 | 10187.5 | 203.75 | 10187.5 | 9.5849% |
| Total |  |  |  | 2125.75 | 10240 | 100.0000% |

### eDRX or no paging reception

It can be seen from Table 12 that SRS Tx + paging and PRS Rx + paging contribute to most of the power consumption. The paging is generally considered useful when network wants to reach UE for the case of MT-data.

LPHAP service type is considered with no or little such requirements, especially for the duration when UE is under “positioning power saving mode”, which can benefit from eDRX or no paging reception in terms of power efficiency improvements.

The timeline for DL and UL positioning without paging is shown in Figure 5.

(Ultra-)Deep sleep

Ramp down

SRS [23dBm]

SSB

Micro sleep

Ramp up

(Ultra-)Deep sleep

2ms

1.5ms

0.5ms

(Ultra-)Deep sleep

Ramp down

Ramp up

(Ultra-)Deep sleep

PRS

Light sleep

19.5ms

Figure 5 UL and DL positioning timeline without paging

The power consumption for SRS only and PRS only without paging is shown in Table 13.

Table 13 Power consumption for SRS only and PRS only

| SRS Tx | Duration (ms) | Power |
| --- | --- | --- |
| Micro sleep | 1.5 | 135 |
| SSB Sync | 2 | 200 |
| SRS Tx (23dBm) | 0.5 | 210 |
| Total | 4 | 545 |
| PRS Rx | Duration (ms) | Power |
| PRS Rx | 0.5 | 120 |
| Light sleep | 19.5 | 780 |
| Total | 20 | 900 |

The power evaluation for 10.24 positioning interval without paging is shown in Table 14.

Table 14 Power consumption with ultra-deep sleep and no paging

| INACTIVE state UL every 10.24s (SRS 10.24s, no paging) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Power state | Power unit | Duration | Instances | Power | Sum duration | Power ratio |
| Ramp up + down | 450 | 25 | 1 | 450 | 25 | 37.5244% |
| SRS Tx | 545 | 4 | 1 | 545 | 4 | 45.4462% |
| Ultra-deep sleep | 0.02 | 1 | 10211 | 204.22 | 10211 | 17.0294% |
| Total |  |  |  | 1199.22 | 10240 | 100.0000% |
| INACTIVE state DL every 10.24s (PRS 10.24s, no paging) |  |  |  |  |  |  |
| Power state | Power unit | Duration | Instances | Power | Sum duration | Power ratio |
| Ramp up + down | 450 | 25 | 1 | 450 | 25 | 28.9594% |
| PRS Rx | 900 | 20 | 1 | 900 | 20 | 57.9188% |
| Ultra-deep sleep | 0.02 | 1 | 10195 | 203.9 | 10195 | 13.1218% |
| Total |  |  |  | 1553.9 | 10240 | 100.0000% |

### TRS-based synchronization

The power for UL positioning can be further reduced if the synchronization is close to SRS transmission, e.g. in adjacent slots. Usually this is not possible for SSB, since SSB is cell-specifically deployed and its transmission occasion is fixed according to TS 38.213; we could consider TRS, which also serves the time/frequency synchronization purpose. There is no such requirement to have two slots TRS transmission/reception because the synchronization requirement to transmit SRS is not so stringent as PDSCH reception.

The timeline for UL positioning with TRS-based synchronization can be found in Figure 6.

(Ultra-)Deep sleep

Ramp down

SRS [23dBm]

Ramp up

(Ultra-)Deep sleep

1ms

TRS

Figure 6 UL positioning timeline without paging for TRS-based synchronization

The power consumption for SRS only without paging synchronized by TRS is shown in Table 15.

Table 15 Power consumption for SRS only without paging synchronized by TRS

| SRS Tx TRS | Duration (ms) | Power |
| --- | --- | --- |
| TRS sync | 0.5 | 50 |
| SRS Tx (23dBm) | 0.5 | 210 |
| Total | 1 | 260 |

The power evaluation for 10.24 positioning interval without paging synchronized by TRS is shown in Table 16.

Table 16 Power consumption with ultra-deep sleep and no paging synchronized by TRS

| INACTIVE state UL every 10.24s (SRS 10.24s, no paging) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Power state | Power unit | Duration | Instances | Power | Sum duration | Power ratio |
| Ramp up + down | 450 | 25 | 1 | 450 | 25 | 49.2191% |
| SRS Tx TRS | 260 | 1 | 1 | 260 | 1 | 28.4377% |
| Ultra-deep sleep | 0.02 | 1 | 10214 | 204.28 | 10214 | 22.3433% |
| Total |  |  |  | 914.28 | 10240 | 100.0000% |

### Summary of Rel-18 enhancements

With the evaluation results shown in Table 12, Table 14, and Table 16, the battery life estimation for Rel-18 LPHAP UL and DL positioning in RRC_INACTIVE state can be found in Table 17.

Table 17 Rel-18 LPHAP enhancements power consumption

| Case | P (PU/s) | C (mAh) | X | T (hour) | LPHAP Targets |
| --- | --- | --- | --- | --- | --- |
| Ref | 74000 | 4500 | 20% | 12 | <6 months |
| UL (10.24/10.24, ultra-deep sleep) | 149.5263672 | 800 | 100% | 5279 | 6 months |
| DL (10.24/10.24, ultra-deep sleep) | 207.5927734 | 800 | 100% | 3802 | <6 months |
| UL (10.24/no paging, ultra-deep sleep) | 117.1113281 | 800 | 100% | 6740 | 6 months |
| DL (10.24/no paging, ultra-deep sleep) | 151.7480469 | 800 | 100% | 5202 | 6 months |
| UL (10.24/no paging, ultra-deep sleep, TRS) | 89.28515625 | 800 | 100% | 8841 | 12 months |

- Observation 4:

With ultra-deep sleep

  - UL positioning can meet the requirement of 6 months

By further removing paging reception

  - UL and DL positioning can meet the requirement of 6 months

By further replacing SSB-based synchronization with TRS-based synchronization

  - UL positioning can meet the requirement of one year

# Considered enhancements

## Ultra-deep sleep

As discussed, ultra-deep sleep is essential to LPHAP. In general, this is considered as a UE implementation; however, we think the following factors need to be additionally considered.

No UE will implement ultra-deep sleep unless the sleep state can be acknowledged and coordinated by the network, e.g. avoiding configuring I-DRX with very short periodicity.

In order to wake up from such an ultra-deep sleep state in a very short time as 25 msec (i.e. 25 msec ramp-up time), UE may not be fully functional, and may only perform very simple operations, such as synchronization and SRS transmission or PRS reception only.

- Proposal 5: Ultra-deep sleep should be considered to achieve the battery life target for LPHAP.

RAN1 should further study the impact to support ultra-deep sleep.

## DRX enhancements

Rel-17 already supports e-DRX for RedCap, and longer DRX cycle than 10.24 sec is supported only for RRC_IDLE state that corresponds to CN paging. In RRC_INACTIVE state, such extension may be helpful for LPHAP to preserve power and also facilitate ultra-deep sleep at the UE.

In addition, considering LPHAP device is a device that is only tasked to help network track its location, in general there is little requirement for the network to send MT-data to the UE, and the necessity of receiving paging for LPHAP may be revisited.

- Proposal 6: RAN1 acknowledges the benefit of DRX enhancement (e.g. no paging reception) to reduce the power consumption for LPHAP.

## Decouple of communication bandwidth and positioning bandwidth

Although we concluded in RAN1#109-e that 100MHz bandwidth for FR1 is considered for LPHAP to meet the LPHAP accuracy requirement, this does not mean that LPHAP should be treated as an eMBB UE, which is not cost friendly.

Conclusion - At least when the positioning accuracy is evaluated without jointly evaluating the associated power consumption, the target horizontal positioning accuracy requirement on LPHAP of <1m can be achieved by Rel-16/17 positioning techniques with a positioning bandwidth of at least 100MHz. - The main aspect of RAN1 evaluation is on power consumption. - Note: This does not preclude the case that the positioning accuracy can be revisited, if found necessary at later stage.

From the service type that a LPHAP device supports, there is little communication requirement, and all the communication capability is only serving positioning functionality, including initial access in RAN, registration in CN, positioning reference signals (PRS, SRS) configuration, location information reporting, and etc. It should be considered as a sensor, but not as a regular cell-phone, nor be assumed as a UE that tasks all data-driven services.

Therefore, on one hand, a LPHAP device is not required to support high data rate, assuming some basic communication capability is required to access the network, and on the other hand, a LPHAP device should be able to process up to 100MHz positioning reference signal transmission or reception, which motivates the decoupling of positioning bandwidth and communication bandwidth.

With the decoupling, UE RF bandwidth should cover the bandwidth required for positioning, but its baseband processing capability is only limited to the communication bandwidth, which is similar to UWB.

- Proposal 7: RAN1 should further study the decoupling of bandwidth of communication and positioning for LPHAP.

## UL enhancements

As shown in the evaluation, TRS based synchronization may shorten the active time when UE wakes up for the purpose of transmitting SRS compared with SSB based.

Rel-17 already introduced TRS configuration for RRC_INACTIVE, but the presence of TRS may not be guaranteed. For SRS transmission in RRC_INACTIVE, network could ensure the transmission by associating the SRS configuration with the TRS configuration.

- Proposal 8: RAN1 should further study the configuration of TRS for synchronization before the SRS transmission.

Another aspect of UL positioning is that the SRS configuration is only valid within the cell that configures the SRS to the UE for RRC_INACTIVE in Rel-17. During the previous evaluation, it is assumed that there is no cell change at all, which may not be likely for a real deployment where multiple cells are present in a warehouse.

The principle of predefined assistance data for DL can be considered for UL, where the SRS configuration could be applied to an area consisting of multiple cells. As long as UE is still within the coverage of the multiple cells, there is no requirement for the UE to release the SRS configuration or to notify network (except for the general mobility management in RRC_INACTIVE, e.g. RNA update or periodic registration if configured by the network).

- Proposal 9: RAN1 should further study the configuration of SRS that is applicable to an area consisting of multiple cells.

# Conclusion

In this contribution, we discussed the evaluation and enhancements to support LPHAP functionality, and have the following observations and proposals.

- Observation 1: Both CIoT and UWB considered a very deep sleep mode with current in the order of uA, which is much lower than the assumption of the current in the deep sleep in TR 38.840.

- Observation 2: Rel-17 baseline UL and DL positioning in RRC_INACTIVE state can achieve 200 – 300 hours battery life, which cannot meet the LPHAP requirements.

Observation 3: The power consumption of deep sleep is the bottleneck for further extending the battery life for LPHAP.

- Observation 4:

With ultra-deep sleep

  - UL positioning can meet the requirement of 6 months

By further removing paging reception

  - UL and DL positioning can meet the requirement of 6 months

By further replacing SSB-based synchronization with TRS-based synchronization

  - UL positioning can meet the requirement of one year

- Proposal 1: Adopt Alt.1 to use the battery life as the metric to identify the gap.

Alt.2 can be considered to evaluate the maximum allowed power unit for any “ultra-deep sleep” to meet the battery life target.

Adopt the following table as the starting point for evaluating the battery life of LPHAP.

| C1 | T1 | X | reference traffic type | C2 | T2req |
| --- | --- | --- | --- | --- | --- |
| 4500 mAh | [8~12] hours under the reference C-DRX configuration of 160ms DRX cycle 10ms on duration 100ms inactivity timer | 20 % | FTP (model 3) | 800 mAh | 12 months |

- Proposal 2: Define a new ultra-deep sleep type for the evaluation of LPHAP.

The power unit of ultra-deep sleep can take 0.01 power unit as the starting point.

The transition energy is 450 power unit.

The transition time is 25ms.

Proposal 3: For paging reception, update the previous assumption that

UE attempts to receive paging DCI in two consecutive slots associated with the SSB.

One slot is treated as PDCCH-only.

The other slot is treated as 90% PDCCH-only and 10% PDCCH+PDSCH.

Note: This reverts the previous agreement of paging receive duration of 2ms.

Proposal 4: For the two slots associated with paging reception, the mean power unit per slot is 53.5.

- Proposal 5: Ultra-deep sleep should be considered to achieve the battery life target for LPHAP.

RAN1 should further study the impact to support ultra-deep sleep.

- Proposal 6: RAN1 acknowledges the benefit of DRX enhancement (e.g. no paging reception) to reduce the power consumption for LPHAP.

- Proposal 7: RAN1 should further study the decoupling of bandwidth of communication and positioning for LPHAP.

- Proposal 8: RAN1 should further study the configuration of TRS for synchronization before the SRS transmission.

- Proposal 9: RAN1 should further study the configuration of SRS that is applicable to an area consisting of multiple cells.

# Reference

- RAN1, Chair’s Notes RAN1#109-e v19, RAN1#109-e, e-Meeting, 9th – 20th May, 2022.

- TR 38.840, NR; Study on User Equipment (UE) power saving in NR.

- TR 45.820, Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT).

- https://www.decawave.com/wp-content/uploads/2020/04/DW1000_Datasheet.pdf