3GPP TSG-RAN WG1 Meeting #110	R1-2205906

# Toulouse, France, August 22nd –26th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on DL-AOD positioning measurement for 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN1#106bis meeting, the following agreement was achieved for UE assisted DL-AOD measurement, where up to 24 PRS RSRP and RSRPP measurements per TRP were agreed. In the first note of the agreement, it is clarified that multiple RSRPs can be reported for a given PRS resource. That is, multiple RSRP or RSRPP measurements can be reported for the same PRS resource. However, in the current TS 38.214, the up to 24 PRS RSRP or RSRPP measurements should be always on different PRS resoruces. In other words, multiple RSRP or RSRPP measurement report are not allowed for the same PRS resource. It is not aligned with the agreement. Agreement in RAN1#106bis-e: The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. - For UE-A DL-AOD, support reporting more than 8 up to 16 M first path PRS RSRP measurements per TRP, where M is a UE capability - FFS: Values of M. Candidate values include {2,4,8,16,24}. - FFS: Whether M is always equal to N - Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps. - Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the restriction to allow multiple RSRP or RSRPP measurements on the same PRS resource |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not aligned with the agreement, and multiple RSRP or RSRPP measurement report are not allowed for the same PRS resource |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This is to correct the specification to align the previous agreement. No isolated impact is expected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unrelated part omitted>

5.1.6.5 PRS reception procedure

The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of dl-PRS-ID with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those dl-PRS-ID. The up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same report use a single reference timing.

The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on DL PRS resources associated with the same dl-PRS-ID.

The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement].

For each PRS resource, the UE may be configured, subject to UE capability, with [DL-AOD-PRS resource-Subset] that is associated with this PRS resource, where the subset of PRS resources associated with the PRS resource can be in the same or different PRS resource set than the PRS resource. The UE may include UE measurements for the subset of PRS resources in [NR-DL-AoD-AdditionalMeasurementElement] if the UE measurements of the associated PRS resource are reported, where the UE measurement can be DL PRS-RSRP and/or DL PRS-RSRPP. The UE may report DL PRS-RSRP and/or DL PRS-RSRPP measurements only for the subset of PRS resources. Subject to UE capability, the UE may be configured with higher layer parameter [DL-AOD Boresight direction] for each PRS resource.

The UE may be provided with beam/antenna information via higher layer parameter [trpBeamAntennaInformation].

The UE may request to be provided with either expected DL-AoD/ZoD and uncertainty range(s) of expected DL-AoD/ZoD, or expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA. The UE may be provided with expected DL-AoD/ZoD and uncertainty range(s) of the expected DL-AoD/ZoD. The UE may be provided with expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA. The uncertainty range(s) of the expected DL-AoD/DL-AoA may be configured within [0, 60]. The uncertainty range(s) of expected DL-ZoD/DL-ZoA may be configured within [0, 30].

<Unrelated part omitted>