3GPP TSG-RAN WG1 Meeting #110	R1-2205909

# Toulouse, France, August 22nd –26th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PRS processing sample for 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN1#106e and RAN1#106bis meeting, the following agreements were achieved for postioning latency improvement, where at least Nsample=1 is supported for PRS processing sample. Moreover, in RAN4#102 meeting, Nsample can also be equal to 2 for DL RSTD, DL PRS-RSRP, or UE Rx-Tx time difference measurements under certain conditions. However, in the current TS 38.214, only Nsample=1 or Nsample=4 is specified. In other words, Nsample=2 is not supported in the current TS 38.214 for PRS processing sample. It is not aligned with the agreements, TS 38.133 and the corresponding UE capabilities. Agreement in RAN1#106-e: Subject to UE capability, support LMF to explicitly request UE to report the measurement with either M-sample or 4-sample, if RAN4 has supported M-sample measurement. - FFS signalling details. Agreement in RAN1#106bis-e: For the PRS processing sample number M, at least M = 1 is supported. Agreement in RAN4#102-e: - Define low latency requirements the reduced number of samples: - Nsample= 1 if the condition under which AGC is not required is met - Nsample= 2 if the condition under which AGC is not required is NOT met |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the reduced number of samples as Nsample = 1 or 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not aligned with RAN4’s agreement and the corresponding UE capabilities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This is to correct the specification to align the previous agreement. No isolated impact is expected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.6.5 PRS reception procedure

<Unrelated part omitted>

The UE may be requested, subject to UE capability, to measure and report one or more of the DL RSTD, DL PRS-RSRP, or UE Rx-Tx time difference measurements with either Nsample1 or 4 samples, where Nsample = 1 or 2 as defined in [11, TS 38.133], via higher layer parameter requestedDL-PRS-ProcessingSamples  [17, TS 37.355] which applies for all DL PRS positioning frequency layers.

<Unrelated part omitted>