3GPP TSG RAN WG1 Meeting # 110	 R1-2205950

# Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on multiplexing PUCCH carrying HP SR and HP HARQ with PUCCH carrying LP HARQ-ACK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-8-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To avoid unnecessary dropping of LP HARQ-ACK when UE transmits overlapping PUCCHs carrying SR and HP HARQ with PUCCH format 0/1 and a LP PUCCH carrying HARQ-ACK |  |  |  |  |  |  |  |  |
| Summary of change: |  | When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK, for HP SR with F1 and HP HARQ-ACK with F0/F1, and HP SR with F0 and HP HARQ-ACK with F0 overlaps with a PUCCH carrying LP HARQ-ACK, - If the total number of LP and HP HARQ-ACK bits is 2, the multiplexing follows the legacy way for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, i.e., treat the two bits as HARQ-ACK bits with high priority and the resultant PUCCH format is 0/1. - If the total number of LP and HP HARQ-ACK bits is larger than 2, - If the resultant PUCCH after multiplexing between HP SR and HP HARQ-ACK doesn't use the PUCCH resource with n1PUCCH-AN or SR resource with PF1, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is O_"UCI" =O_"ACK,1" +1. The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. - Otherwise, LP HARQ-ACK is dropped. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary dropping of LP HARQ-ACK. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.2.5.3	UE procedure for reporting UCI of different priorities

If a UE

-	is provided PUCCH-ConfigurationList for PUCCH transmissions with priority 0 and 1,

-	is provided UCI-MuxWithDifferentPriority, and

-	would transmit overlapping PUCCHs that include a first PUCCH with OACK,0 HARQ-ACK information bits of priority 0 and a second PUCCH with OACK,1 HARQ-ACK information bits of priority 1

-	if the PUCCH resource for the second PUCCH includes PUCCH format 2, 3, or 4 and additionally includes OSR,1 SR bits of priority 1, OACK,1 is replaced by OACK,1+OSR,1 where OSR,1 is determined according to clause 9.2.5.1

-	if the PUCCH resource for the second PUCCH includes PUCCH format 0 or 1 and overlaps with PUCCH includes PUCCH format 1 with 1 SR bit of priority 1, or the PUCCH resource for the second PUCCH includes PUCCH format 0 and overlaps with PUCCH includes PUCCH format 0 with 1 SR bit of priority 1

-	and if OACK,0+OACK,1>2, OACK,1 is replaced by OACK,1+1

the UE

-	determines

-	a PUCCH resource set from the second PUCCH-Config using OUCI=OACK,0+OACK,1 as described in clause 9.2.1, and a PUCCH resource from the PUCCH resource set as described in clause 9.2.3 where a DCI format, if any, triggers PUCCH transmission of priority 1, or

-	a PUCCH resource from the second sps-PUCCH-AN-List using OUCI=OACK,0+OACK,1 as described in clause 9.2.1, and

-	multiplexes the OACK,0 and OACK,1 HARQ-ACK information bits in a same PUCCH using the PUCCH resource.

If a UE would transmit overlapping PUCCHs that include a first PUCCH with 1 HARQ-ACK information bits of priority 0 and a second PUCCH with 1 HARQ-ACK information bits of priority 1 includes PUCCH format 0 or 1 and the third PUCCH with SR information of priority 1 includes PUCCH format 1, or would transmit overlapping PUCCHs that include a first PUCCH with 1 HARQ-ACK information bits of priority 0 and a second PUCCH with 1 HARQ-ACK information bits of priority 1 includes PUCCH format 0 and the third PUCCH with SR information of priority 1 includes PUCCH format 0

-	if the resultant PUCCH transmission uses a resource that includes PUCCH format 0, the HARQ-ACK information bits of priority 1 and priority 0 are set as the first and second bits in Table 9.2.5-2, respectively, to derive the mCS of the PUCCH transmission

-	if the resultant PUCCH transmission uses a resource that includes PUCCH format 1 as described in clause 9.2.5.1, the HARQ-ACK information bits of priority 1 and priority 0 are the first and second bits, respectively, of the QPSK modulated symbol for the PUCCH transmission

If the PUCCH resource includes PUCCH format 2 or PUCCH format 3 and MRBPUCCH PRBs, the UE determines a number of MRB,minPUCCH≤MRBPUCCH PRBs for the PUCCH transmission to be the minimum number of PRBs that starts from the first PRB from the MRBPUCCH PRBs and results to

OACK,0+OCRC,0⋅r1+OACK,1+OCRC,1⋅r0≤MRB,minPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r0⋅r1

where OCRC,0 or OCRC,1 is a number of CRC bits, if any, for encoding the OACK,0 or the OACK,1 HARQ-ACK information bits, respectively, r0 is provided by maxCodeRateLP, and the remaining parameters are as defined in clause 9.2.5.2 with r1=r. For PUCCH format 3, if MRB,minPUCCH is not equal to 2α2⋅3α3⋅5α5 [4, TS 38.211], MRB,minPUCCH is increased to a nearest value that is equal to 2α2⋅3α3⋅5α5 and does not exceed nrofPRBs.

If OACK,0+OCRC,0⋅r1+OACK,1+OCRC,1⋅r0>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r0⋅r1, the UE transmits the PUCCH over the MRBPUCCH PRBs.

If a UE transmits a PUCCH that includes HARQ-ACK information bits of priority 0 and 1 using a PUCCH resource that includes PUCCH format 2, 3 or 4, the UE determines a power for the PUCCH transmission as described in clause 7.2.1 assuming that the PUCCH includes only UCI bits of priority 1, where NRE(i)=minMRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,OUCI,1+OCRC,1Qm⋅r1.

If a UE transmits a PUCCH that includes one HARQ-ACK information bit of priority 0 and one HARQ-ACK information bit of priority 1

-	if the PUCCH transmission uses a resource that includes PUCCH format 0, the HARQ-ACK information bits of priority 1 and priority 0 are set as the first and second bits in Table 9.2.3-4, respectively, to derive the mCS of the PUCCH transmission

-	if the PUCCH transmission uses a resource that includes PUCCH format 1, the HARQ-ACK information bits of priority 1 and priority 0 are the first and second bits, respectively, of the QPSK modulated symbol for the PUCCH transmission

If a UE transmits a PUCCH that includes HARQ-ACK information bits of priority 0 and 1 using PUCCH format 1, the UE determines a power for the PUCCH transmission as described in clause 7.2.1 assuming that all HARQ-ACK information bits have priority 1.

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation

-	if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 in InterlaceAllocation

-	if OACK,0+OCRC,0⋅r1+OACK,1+OCRC,1⋅r0≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r0⋅r1, the UE transmits the PUCCH over the first interlace

-	else, the UE transmits the PUCCH over both the first and second interlaces

-	else the UE transmits the PUCCH over the first interlace

If the UE transmits a PUCCH that includes HARQ-ACK information bits of priority 0 and 1 over interlaces, the UE determines a power for the PUCCH transmission as described in clause 7.2.1 assuming that the PUCCH includes only UCI bits of priority 1.

< Unchanged parts are omitted >