3GPP TSG RAN WG1 #110                                                                               R1- 2206106

Toulouse, France, August 22nd – 26th, 2022

Source:	New H3C

Title:	Discussions on increased number of orthogonal DMRS ports

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

- Introduction

In the RAN1 #109-e meeting, increased number of orthogonal DMRS ports in Rel.18 NR MIMO evolution was discussed. Some agreements were made as below [1].

Agreement

- LLS is used for objective #3 (increasing DMRS ports for MU-MIMO) in Rel.18 MIMO, while SLS can be used optionally.

Agreement

- No EVM discussion is needed for objective #5 (>4 layers PUSCH DMRS) in AI 9.1.3.1 (DMRS) in Rel.18.

Agreement

- LLS for increasing DMRS ports in AI 9.1.3.1 in Rel.18:

  - Evaluated channel: PDSCH as baseline (Companies can additionally submit evaluation results of PUSCH).

  - Evaluation metric:

    - BLER for fixed MCS and rank as baseline

    - User throughput for adaptive MCS and rank as optional

    - MSE or NMSE of DMRS as optional

  - Evaluation baseline (i.e. compared with):

    - For evaluation of enhanced single-symbol DMRS, baseline refers to Rel.15 single-symbol DMRS or Rel.15 double-symbol DMRS.

    - For evaluation of enhanced double-symbol DMRS, baseline refers to Rel.15 double-symbol DMRS.

Agreement

- Specify to increase the max. number of DMRS ports for PDSCH/PUSCH larger than Rel.15 for CP-OFDM without increasing the DMRS overhead.

  - Strive to have common design of DMRS enhancement for PDSCH and PUSCH for a given DMRS Type.

Agreement

- The maximum number of enhanced DMRS ports in Rel.18 is doubled from Rel.15 DMRS ports:

  - For DMRS type 1, the max. number of enhanced DMRS ports in Rel.18 for PDSCH/PUSCH is

    - Single symbol DMRS: 8 DMRS ports.

    - Double symbol DMRS: 16 DMRS ports.

  - For DMRS type 2, the max. number of enhanced DMRS ports in Rel.18 for PDSCH/PUSCH is

    - Single symbol DMRS: 12 DMRS ports.

    - Double symbol DMRS: 24 DMRS ports.

Agreement

- To increase the number of DMRS ports for PDSCH/PUSCH, evaluate and, if needed, specify one or more from the following options:

  - Opt.1 (enhance FD-OCC): Introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6).

    - Study aspect includes potential performance degradation in large delay spread, potential scheduling restriction, backward compatibility.

  - Opt.2 (enhance TD-OCC): Utilize TD-OCC over non-contiguous DMRS symbols (e.g. TD-OCC across front/additional DMRS symbols)

    - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility.

  - Opt.3 (Sparser frequency allocation): increase the number of CDM groups (e.g. larger number of comb/FDM).

    - Study aspect includes potential performance degradation in large delay spread, backward compatibility.

  - Opt.4 (using TDMed DMRS symbol): reusing additional DMRS symbols to increase orthogonal DMRS ports

    - Study aspect includes potential performance degradation in high UE velocity, potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility.

  - Opt.5 TD-OCC over non-contiguous DMRS symbols combined with FD-OCC or FDM: reusing additional DMRS symbol(s) to improve channel estimation performance.

    - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility.

  - The same option can be applied to both single symbol DMRS and double symbol DMRS.

Agreement

- To increase the max. number of DMRS ports for PDSCH/PUSCH compared to Rel.15 DMRS for CP-OFDM without increasing the DMRS overhead,

  - Study whether/how to enable MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports, as well as whether/how to enable MU-MIMO among Rel.18 DMRS ports, in the same or different CDM group.

Agreement

- To increase the max. number of orthogonal DMRS ports for PDSCH/PUSCH larger than Rel.15

  - Study whether/how to support DCI-based dynamic antenna ports indication of Rel.18 DMRS ports and/or Rel.15 DMRS ports.

  - Study whether/how to reuse the antenna port indication table in 38.212 as much as possible for both PDSCH and PUSCH

  - Study the potential need for MU scheduling restrictions in the design of the enhanced antenna port indication table in 38.212 for DL PDSCH.

This contribution focuses on further discussion of increased number of orthogonal DMRS ports.

- Discussion

Based on the agreement in last meeting[1], we have 5 options on increasing DMRS ports as follows:

Agreement - To increase the number of DMRS ports for PDSCH/PUSCH, evaluate and, if needed, specify one or more from the following options: - Opt.1 (enhance FD-OCC): Introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6). - Study aspect includes potential performance degradation in large delay spread, potential scheduling restriction, backward compatibility. - Opt.2 (enhance TD-OCC): Utilize TD-OCC over non-contiguous DMRS symbols (e.g. TD-OCC across front/additional DMRS symbols) - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - Opt.3 (Sparser frequency allocation): increase the number of CDM groups (e.g. larger number of comb/FDM). - Study aspect includes potential performance degradation in large delay spread, backward compatibility. - Opt.4 (using TDMed DMRS symbol): reusing additional DMRS symbols to increase orthogonal DMRS ports - Study aspect includes potential performance degradation in high UE velocity, potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - Opt.5 TD-OCC over non-contiguous DMRS symbols combined with FD-OCC or FDM: reusing additional DMRS symbol(s) to improve channel estimation performance. - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - The same option can be applied to both single symbol DMRS and double symbol DMRS.

For Option 1 and Option 3 as shown in Figure 1 and Figure 2, these two options methods are simple method to support increasing antenna ports.

For Option 2&Option 5, gNB scheduling restriction of frequency hopping and additional DMRS symbol can be introduced.

For Option 4 shown in Figure 1 and Figure 2, for supporting high UE velocity, Option 4 need be considered.

Therefore, in order to reduce the work load of RAN1, option 1 and 3 can be supported with high priority. Considering high UE velocity case, option 4 should be considered.

Fig.1. Example of Option 1 (enhanced FD-OCC)

- Fig.2. Example of Option 3 (Sparser frequency allocation)

Proposal 1: For R18 increasing antenna ports, Option 1 (enhanced FD-OCC) and Option 3. (Sparser frequency allocation) should be considered as baseline.

Proposal 2: Considering the case of high UE velocity, option 4 should be supported.

Conclusions

In this contribution, we discussed about increased number of orthogonal DMRS ports. The proposals are as follows:

Proposal 1: For R18 increasing antenna ports, Option 1 (enhanced FD-OCC) and Option 3. (Sparser frequency allocation) should be considered as baseline.

Proposal 2: Considering the case of high UE velocity, option 4 should be supported.

- Reference

- Chairman’s Notes, RAN1#109-e, final, May 9th – 20th, 2022