3GPP TSG RAN WG1 #110			R1-2206182

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.2.3.2

Source:	InterDigital, Inc.

Title:	Discussion for other aspects on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

In RAN#94-e [1], the study item for AI/ML has been approved for NR Air Interface. In this contribution, we discuss potential issues and associated standard impacts to support AI/ML for beam management in NR air interface based on BM-Case1, BM-Case2 and other possible cases.

# Discussions

In RAN1#109-e [2], the following agreements on BM-Case1 and BM-Case2 were made:

Agreement For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations - BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams - BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams - FFS: details of BM-Case1 and BM-Case2 - FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

In Section 2.1, 2.2 and 2.3, we provide discussion on potential issues and further details for BM-Case1 and BM-Case2 in. In Section 2.4, we provide our view on potential specification enhancements.

## Location of AI/ML Inference

In RAN1#109-e [2], the location of AI/ML inference was discussed and two alternatives are provided for each scenario as shown in the below:

Agreement For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side Agreement For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side

AI/ML inference at NW side (Alt.1) could be a good implementation option as UE implementation is generally limited due to computational power and battery consumption than gNB implementation. However, AI/ML inference generally requires more detailed explicit information which leads to significant reporting overhead. For a pair of gNB and UE, the increased reporting overhead may not be a critical issue for the network, however, if we consider all available UEs in a cell, acquiring information for the inference of multiple UEs requires huge overheads. On the other hand, AI/ML inference at UE side (Alt.2) can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead.

Observation 1: AI/ML inference at NW side (Alt.1) could be a good implementation option as UE implementation is generally limited due to computational power and battery consumption than gNB implementation. However, AI/ML inference generally requires more detailed explicit information which leads significant reporting overhead.

Observation 2: AI/ML inference at UE side (Alt.2) can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead.

Proposal 1: Consider both AI/ML inference at NW side (Alt.1) and UE side (Alt.2) for both BM-Case1 and BM-Case2.

## Relationship between Set A and B

In RAN1#109-e [2], the following alternatives were agreed for BM-Case 1 and BM-Case 2.

Conclusion: For the sub use case BM-Case1, consider the following alternatives for further study: - Alt.1: Set B is a subset of Set A o FFS: the number of beams in Set A and B o FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …) - Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams) o FFS: the number of beams in Set A and B o FFS: QCL relation between beams in Set A and beams in Set B - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact - Note3: The codebook constructions of Set A and Set B can be clarified by the companies. Conclusion For the sub use case BM-Case2, further study the following alternatives with potential down-selection: - Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams) - FFS: QCL relation between beams in Set A and beams in Set B - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …) - Alt.3: Set A and Set B are the same - Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B. - Note2: It is up to companies to provide other alternative(s) Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact

- Set B is a subset of Set A

  - As using same beamwidth for all channels and signals is a general implementation within a frequency range, using a subset of Set A as Set B is a reasonable option if Set A and Set B are utilized in a same frequency range. This is especially beneficial when Set A and Set B are in an identical frequency range. However, it is difficult to use a subset of Set A considering different beamwidths in different frequency ranges.

Figure 1. Example of ‘Set B is a subset of Set A’ for BM-Case1

- Alt.2: Set A and Set B are different

  - As discussed, utilizing different beams is not a general option for different FRs. However, it is difficult to apply Alt.1 considering different beamwidths in different frequency ranges. In addition, it should be noted that utilization of wide beam information from a low frequency range has great potential as a low frequency range is more reliable and utilization of wide beam requires much less time and frequency resources for beam management. In our view, association between different frequency ranges should be supported for both between FR1 and FR2-1 and between FR2-1 and FR2-2.

Observation 3: As using same beamwidth for all channels and signals is a general implementation within a frequency range, using a subset of Set A as Set B is a reasonable option if Set A and Set B are utilized in a same frequency range.

Observation 4: It is difficult to use a subset of Set A considering different beamwidths for beam management between different frequency ranges.

Observation 5: Utilization of wide beam information from a low frequency range has great potential as a low frequency range is more reliable and utilization of wide beam requires much less time and frequency resources for beam management.

Proposal 2: Support ‘Set B is a subset of Set A’ when Set A and Set B are utilized in a same frequency range for both BM-Case1 and BM-Case2.

Proposal 3: Support ‘Set A and Set B are different’ when Set A and Set B are utilized in different frequency ranges for both BM-Case1 and BM-Case2.

Proposal 4: AI/ML based beam management based on association between different frequency ranges should supported for both between FR1 and FR2-1 and between FR2-1 and FR2-2.

## AI/ML input

In RAN1#109-e [2], the following alternatives were discussed for AI/ML input:

Conclusion Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose. Conclusion Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance): - Alt.1: Only L1-RSRP measurement based on Set B - Alt 2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives Note2: All the inputs are “nominal” and only for discussion purpose.

For us, ‘Only L1-RSRP measurement based on Set B’ is not clear enough as the alternative does not provide any beam related information. If ‘Only L1-RSRP measurement based on Set B’ means that L1-RSRP measurements are provided in a fixed order, in our view, the input is not ‘Only L1-RSRP measurement based on Set B’. Reporting L1-RSRP measurements in a fixed order is indicating L1-RSRP measurement with implicit beam related information. Having said that, companies supporting the alternative should provide more details for predicting L1-RSRP values without any beam information. On the other hand, ‘L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID’ can be a baseline option for studying AI/ML based beam management. By providing L1-RSRP measurements with Tx and Rx beam ID, AI/ML model can predict RSRP measurements with Tx and Rx beam IDs which are not provided. In addition, some additional assistance information can be considered as discussed in ‘ L1-RSRP measurement based on Set B and assistance information’. For example, TRP IDs and/or panel IDs can be used for multi-TRP/panel operation. ‘CIR based on Set B’ can be considered as an alternative only for beam management based on FR1 information.

Observation 6: ‘Only L1-RSRP measurement based on Set B’ is not clear enough as the alternative does not provide any beam related information.

- If ‘Only L1-RSRP measurement based on Set B’ means that L1-RSRP measurements are provided in a fixed order, in our view, the input is not ‘Only L1-RSRP measurement based on Set B’.

- Reporting L1-RSRP measurements in a fixed order is indicating L1-RSRP measurement with implicit beam related information.

Proposal 5: Companies supporting the alternative should provide more details for predicting L1-RSRP values without any beam information.

Observation 7: ‘L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID’ can be a baseline option as AI/ML model can predict RSRP measurements with Tx and Rx beam IDs which are not provided.

Proposal 6: Support ‘L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID’ as a baseline.

Proposal 7: Additional information such as TRP IDs and Panels IDs should be considered.

Proposal 8: ‘CIR based on Set B’ can be considered as an alternative only for beam management based on FR1 information.

## Possible specification impacts

In Rel-15, DL beam management procedures for P1 (initial beam selection), P2 (beam refinement for gNB Tx beam) and P3 (beam refinement for UE Rx beam) were defined. Although actual beam selection for beam management is up to gNB implementation, Rel-15 beam management procedures are basically designed for exhaustive searching of gNB transmission beams and/or UE reception beams. As analog beams cannot be multiplexed in time domain, the exhaustive searching requires enormous amount of resource overheads as well as large latency. In Rel-16/17, non-coherent JT based on multi-TRP, unified TCI states and other specification enhancements were introduced, however, the basic principle of exhaustive searching for beam measurement did not change. Especially, the overhead and the latency issues are more serious for FR2-2 as utilization of narrower beams than FR2-1 is expected to compensate the increased pathloss. Given the situation, the following specification enhancements should be considered for Rel-18 AI/ML beam management.

Simple specification extension of UE reporting which enables AI/ML beam prediction

To enable AI/ML beam prediction based on gNB implementation, simple extension of the current UE reporting should be considered for both BM-Case1 and BM-Case2. As the current NR specification, which supports UE reporting with up to 4 best CRIs/SSBRIs with L1-RSRP or L1-SINR, can be very limited for training AI/ML model and predicting spatial/temporal beam information, the following specification enhancements can be considered.

Observation 8: The current NR specification supporting UE reporting with up to 4 best CRIs/SSBRIs with L1-RSRP or L1-SINR can be very limited for gNB estimation.

Proposal 9: Study benefits of simple specification extension of UE reporting.

UE reporting beam information with associated time domain information

In Rel-15 beam management, if measurement restriction for channel and interference is not configured, how to handle multiple measurements in multiple transmission occasions are totally up to UE implementation. If measurement restriction for channel and interference is configured, UE should consider only latest measurement for UE reporting. The measurement restriction in the current specification is not to consider time domain characteristics, but to efficiently utilize RS transmissions for multiple analog/digital beams. However, for gNB which predicts beams by using AI/ML, time domain characteristics of beam measurements are essential as well as spatial domain characteristics of beam measurements. Especially, such time domain information will be crucial for certain predictable scenarios such as highway or HST. Given that, specification enhancements such as UE reporting with associated time slot information should be considered.

Observation 9: The current NR specification supports measurement restriction to limit UE measurement, however, measurement restriction is to efficiently utilize RS transmissions for multiple beams not to consider time domain characteristics of beam measurement.

Observation 10: For gNB which predicts beams by using AI/ML, time domain characteristics of beam measurements are essential as well as spatial domain characteristics.

Proposal 10: Study benefits of specification enhancements such as UE reporting with associated time domain information.

Association between beams with different beam widths

Although wide beamwidth for SSB and narrow beamwidth for CSI-RS were considered for the initial design of NR from Rel-15, association between beams with different beam widths was not considered in efficient ways. For AI/ML based beam prediction, such association could be utilized to achieve better prediction accuracy. For example, robust estimation/identification of whole spatial characteristics could be done by utilizing wide beams and accurate beam identification could be done based on the acquired wide beam information by utilizing narrow beams.

Observation 11: The current NR specification does not consider association between beams with different beam widths.

Observation 12: Utilizing association between beams with different beam widths can provide benefits for prediction accuracy e.g., robust estimation/identification of whole spatial characteristics with wide beams and accurate beam identification with narrow beams.

Proposal 11: Study benefits of specification enhancements on association between beams with different beam widths.

Figure 2. Example of association between beams with different beam widths

Handling of UE Rx beams

For Rel-15 beam management, actual mapping between DL Tx beam and UE Rx beam is totally based on UE implementation and there’s no way to identify actual UE beam information for a DL Tx beam by gNB. The implementation-based UE Rx beam selection works for Rel-15 as the gNB needs to know only a beam index for actual transmission. However, for AI/ML based beam prediction, UE Rx beam information such as beam identity and beam direction is crucial to accurately predict beam qualities.

Observation 13: For Rel-15 beam management, actual mapping between DL Tx beam and UE Rx beam is totally based on UE implementation.

Observation 14: The implementation-based UE Rx beam selection works for Rel-15, however, UE Rx beam information is crucial to accurately predict beam qualities for AI/ML based beam prediction.

Proposal 12: Study benefits of specification enhancements on acquiring UE Rx beam information.

# Summary

In this contribution, we discuss potential issues and associated standard impacts to support AI/ML for beam management. From the discussions, we made the following observations and proposals:

Observation 1: AI/ML inference at NW side (Alt.1) could be a good implementation option as UE implementation is generally limited due to computational power and battery consumption than gNB implementation. However, AI/ML inference generally requires more detailed explicit information which leads significant reporting overhead.

Observation 2: AI/ML inference at UE side (Alt.2) can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead.

Observation 3: As using same beamwidth for all channels and signals is a general implementation within a frequency range, using a subset of Set A as Set B is a reasonable option if Set A and Set B are utilized in a same frequency range.

Observation 4: It is difficult to use a subset of Set A considering different beamwidths for beam management between different frequency ranges.

Observation 5: Utilization of wide beam information from a low frequency range has great potential as a low frequency range is more reliable and utilization of wide beam requires much less time and frequency resources for beam management.

Observation 6: ‘Only L1-RSRP measurement based on Set B’ is not clear enough as the alternative does not provide any beam related information.

- If ‘Only L1-RSRP measurement based on Set B’ means that L1-RSRP measurements are provided in a fixed order, in our view, the input is not ‘Only L1-RSRP measurement based on Set B’.

- Reporting L1-RSRP measurements in a fixed order is indicating L1-RSRP measurement with implicit beam related information.

Observation 7: ‘L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID’ can be a baseline option as AI/ML model can predict RSRP measurements with Tx and Rx beam IDs which are not provided.

Observation 8: The current NR specification supporting UE reporting with up to 4 best CRIs/SSBRIs with L1-RSRP or L1-SINR can be very limited for gNB estimation.

Observation 9: The current NR specification supports measurement restriction to limit UE measurement, however, measurement restriction is to efficiently utilize RS transmissions for multiple beams not to consider time domain characteristics of beam measurement.

Observation 10: For gNB which predicts beams by using AI/ML, time domain characteristics of beam measurements are essential as well as spatial domain characteristics.

Observation 11: The current NR specification does not consider association between beams with different beam widths.

Observation 12: Utilizing association between beams with different beam widths can provide benefits for prediction accuracy e.g., robust estimation/identification of whole spatial characteristics with wide beams and accurate beam identification with narrow beams.

Observation 13: For Rel-15 beam management, actual mapping between DL Tx beam and UE Rx beam is totally based on UE implementation.

Observation 14: The implementation-based UE Rx beam selection works for Rel-15, however, UE Rx beam information is crucial to accurately predict beam qualities for AI/ML based beam prediction.

Proposal 1: Consider both AI/ML inference at NW side (Alt.1) and UE side (Alt.2) for both BM-Case1 and BM-Case2.

Proposal 2: Support ‘Set B is a subset of Set A’ when Set A and Set B are utilized in a same frequency range for both BM-Case1 and BM-Case2.

Proposal 3: Support ‘Set A and Set B are different’ when Set A and Set B are utilized in different frequency ranges for both BM-Case1 and BM-Case2.

Proposal 4: AI/ML based beam management based on association between different frequency ranges should supported for both between FR1 and FR2-1 and between FR2-1 and FR2-2.

Proposal 5: Companies supporting the alternative should provide more details for predicting L1-RSRP values without any beam information.

Proposal 6: Support ‘L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID’ as a baseline.

Proposal 7: Additional information such as TRP IDs and Panels IDs should be considered.

Proposal 8: ‘CIR based on Set B’ can be considered as an alternative only for beam management based on FR1 information.

Proposal 9: Study benefits of simple specification extension of UE reporting.

Proposal 10: Study benefits of specification enhancements such as UE reporting with associated time domain information.

Proposal 11: Study benefits of specification enhancements on association between beams with different beam widths.

Proposal 12: Study benefits of specification enhancements on acquiring UE Rx beam information.

# References

- RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”,

- “Final Report of 3GPP TSG RAN WG1 #109-e”, 3GPP RAN1 Meeting #109-e, e-Meeting, May 9th – 20th, 2022.