3GPP TSG RAN WG1 #110                                                                       R1-2206263

Toulouse, France, August 22nd – 26th, 2022

Source:	OPPO

Title:	Unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for:	Discussion

# Introduction

The WID [1] of MIMO evolution for downlink and uplink was agreed in RAN#94e meeting. According to the arrangement, the objectives related to this agenda item are collected and highlighted as below

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we present our initial view on the aspects of a) extending unified TCI state for multi-TRP operation, b) UL beam indication for simultaneous multi-panel transmission (STxMP), c) power control for sDCI based multi-TRP operation and d) other aspects for multi-TRP operation with low priority.

# Unified approach for extending unified TCI state for mTRP

From Rel.16 to Rel.17, the multi-TRP transmission schemes have been studied and continuously specified. In Rel.18, the enhancement on multi-TRP has been ongoing to facilitate STxMP in UL, if verified and supported. In a short summary, these transmission schemes can be listed for DL and UL separately as below.

- DL transmission

  - Rel.16 mTRP PDSCH (sDCI-mPDSCH, mDCI-mPDSCH, 1a/2a/2b/3/4)

  - Rel.17 inter-cell multi-TRP (mDCI-mPDSCH)

  - Rel.17 mTRP PDCCH repetition

  - Rel.17 SFN PDCCH and SFN PDSCH (for HST)

- UL transmission

  - Rel.17 mTRP PUCCH repetition

  - Rel.17 mTRP PUSCH repetition

  - Rel.18 simultaneous multi-panel transmission (to be specified, if needed)

In RAN1#109e (the very 1st meeting) of Rel.18 MIMO, though not clearly listing above-mentioned intra- and inter-cell MTRP schemes specified in Rel.16 and Rel.17 in the following agreement, we believe there is no ambiguity on which multi-TRP schemes the agreement refers to. Apparently, the coherent joint transmission (CJT) to be specified in Rel.18 was not agreed to be considered with unified TCI framework extension.

Agreement

On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17

- Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP

One of the key features of unified TCI state for MTRP operation is the shared beam indication/updating among all channels/signals (except some special cases). Note that some particular channels (e.g. non-UE-dedicated channel for receipt of paging) and signals (P/SP CSI-RS) can be indicated with another unified TCI state(s), rather than the shared one(s).

From this sense, the unified TCI framework can unify the indicated/updated beam in DL and/or UL. However, for multi-TRP operation (up to 2 TRPs @FR2), this unified feature on beam indication/update should be properly split in a TRP-specific way. Specifically, if one unified TCI state is indicated for one TRP, it is only applicable to the DL and/or UL channel/signal associated with the TRP, rather than for the other TRP. Illustration can be found in Figure 1.

- : MTRP operation with unified TCI framework extension

- Study how to split the unified TCI framework as TRP-specific for beam indication/updating with high priority.

## Foundation of extending unified TCI state framework

For MTRP operation, up to 2 TRPs transmission are supported until Rel.17. For Rel.17 STRP operation, unified TCI state should be shared among DL channels/signals (UE-dedicated PDCCH/PDSCH and AP-CSI-RS) and UL channels/signals (PUCCH/PUSCH/SRS (except SRS for aperiodic beam management purpose)). For MTRP operation, each unified TCI state should be shared within its associated TRP.

Consider that two types of unified TCI state defined in Rel.17, i.e. joint TCI state and separate DL/UL TCI state. Each TRP can be associated with a set of unified TCI state(s). The set of unified TCI state(s) can be either 1 joint TCI state or 2 separate DL/UL TCI states. To differentiate two TRPs in specification, beam indication can be carried by the 1st set of indicated TCI state(s) and the 2nd set of indicated TCI states. In our view, that’s the foundation for the extension of unified TCI framework to MTRP.

- Support the 1st set of indicated TCI state(s) for one TRP and the 2nd set of indicated TCI state(s) for the other TRP

- One set of indicated TCI state(s) can be either 1 joint TCI state or 2 separate DL/UL TCI states

- At least the set of indicated TCI state(s) can be used for discussion purpose

## Signaling aspects on indicated TCI state(s)

### Maximum number of indicated TCI state(s)

For the maximum number of indicated TCI states, i.e. M (for DL) and/or N (for UL), it is natural to align with single beam operation of STRP, i.e. limiting M<=2 and/or N<=2. Each TRP can be indicated either 2 separate DL/UL TCI states (1 for DL and 1 for UL) or 1 joint TCI state. As for N<=2, it is also aligned with the WID on STxMP (if supported).

- For MTRP operation, support up to 2 joint TCI states and 4 separate DL/UL TCI states.

### Mode of unified TCI state configuration

Regarding the configuration of unified TCI states in Rel.17, either joint or separate DL/UL TCI state can be configured in a CC/BWP, but not both. In Rel.18, for MTRP, one may argue the flexibility of configuring the mixed mode of joint and separate TCI states and concerns over the issue of MPE.

But the reason on MPE issue seems not a valid, since whether the MPE event will occur on which panel (linked to one TRP) cannot be known before configuration. And NW has to configure both joint and separate TCI states to UE, therefore the storage burden at UE would increased correspondingly. If the MPE event will highly likely to happen at any UE panel and NW would like to avoid such negative impact, then one reasonable way would be to configure only separate DL/UL TCI states for both TRPs.

- Following Rel.17 unified TCI state configuration, either joint TCI states or separate DL/UL TCI states can be configured in one CC/BWP for MTRP operation, but not both.

### Signaling medium for TCI state indication

In RAN1 #109e, one agreement for at least S-DCI MTRP was achieved as below.

Agreement

On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list

- FFS: Detail of mapping joint/DL/UL TCI state ID(s) to a TCI codepoint, e.g., possible combinations of joint, DL, and/or UL TCI state IDs that can be mapped to a TCI codepoint

- FFS: Whether to increase the max number of MAC CE activated TCI codepoints, i.e., more than 8 codepoints

- FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits

- Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded

Note: The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS

From signaling perspective, there could be up to 4 (M = 2, N = 2) separate DL/UL TCI states or 2 (M = N = 2) joint TCI states to be carried by MAC CE or MAC CE + DCI. Assuming above are agreeable, then the total number of TCI states in one TCI codepoint reaches up to 4 for separate DL/UL TCI states and up to 2 for joint TCI states. In addition, assume either joint or separate TCI states are configured. Correspondingly, MAC CE should be extended to facilitate up to 4 separate DL/UL TCI states or 2 joint TCI states per codepoint.

- Enhance MAC CE to facilitate up to 4 separate DL/UL TCI states or 2 joint TCI states per codepoint.

If only single mode of unified TCI states can be configured, then at least for joint TCI states, there seems no need to increase the maximum number of TCI codepoints. Recall the MAC CE design for S-DCI based M-PDSCH in Rel.16, there are also 8 codepoints and each codepoint may include one or two legacy TCI state(s).

- At least for joint TCI states, it’s unnecessary to increase the maximum number of codepoints (i.e. 8 codepoints in legacy MAC CE).

Correspondingly, for DCI format 1_1 and 1_2, it seems enough to reuse the maximum number of TCI field bits (i.e. 3 bits). Moreover, we didn’t see any strong motivation to insert additional TCI-related field in these DCI. On the other hand, we have to consider the UE decoding complexity on PDCCH when more DCI formats are introduced.

- For beam indication of MTRP operation, reuse Rel.17 DCI format 1_1/1_2, i.e. adding no additional TCI related field and TCI field bits.

### TCI state updating mechanism

In Rel.15/16, beam indication for PDSCH (either STRP or MTRP) can be based on dynamic TCI state indication in DL DCI. The indicated TCI state(s) is (are) applicable for the scheduled PDSCH on its occasion(s). The indicated TCI state will not last for the upcoming slots.

Different from the legacy TCI indication, the unified TCI state in Rel.17 is applicable after BAT until next round of unified TCI state indication. The indicated unified TCI state will last for an uncertain period until next update from NW. For S-DCI, one set or two sets of unified TCI state(s) can be indicated to update previously applicable ones. For the case (shown in Figure 2) when one set of TCI state is indicated and previously two sets of indicated TCI states are applicable, one issue is whether UE should maintain the other set of TCI state(s) to facilitate MTRP operation as configured. In our view, this issue should be studied and given considerable attention.

- : DCI partially updating the set of unified TCI state(s)

- For the case when two sets of indicated TCI states are updated by only one set, study unified TCI state updating mechanism, i.e. whether UE should maintain two sets of indicated TCI states or fallback to STRP operation.

## Association/mapping between indicated TCI state(s) and channels

Since we have defined the 1st set and 2nd set of indicated TCI states, next it’s necessary to associate/map the sets of indicated TCI states with/to DL or UL channels. Let’s next present our view per each channel when considering previous agreements in RAN1#109e.

### MTRP PDCCH

In RAN1 #109e, the following agreement on TCI state indication for PDCCH was achieved. A few of alternatives on how to split the indicated TCI state(s) for TRP-specific PDCCH were listed and potentially to be narrowed down.

Agreement

On unified TCI framework extension for S-DCI based MTRP, consider at least the following alternatives to map/associate a joint/DL TCI state to PDCCH reception(s)

- Atl1: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET or a CORESET group

- Alt2: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a search space set

- Alt3: Use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET or a CORESET group

- Alt4: Use DCI to inform the mapping/association between an indicated joint/DL TCI state and a CORESET or a CORESET group

- Alt5: Based on a fixed mapping/association rule, e.g., the first indicated joint/DL TCI state always applies to PDCCH receptions

Consider above alternatives for PDCCH repetition, PDCCH-SFN, PDCCH w/o repetition/SFN, and potential support of dynamic switching between S-TRP and M-TRP for PDCCH. It is not precluded to adopt one single alternative or multiple alternatives to support these cases.

Consider the case that when S-DCI schedules M-PDSCH (MTRP). Two sets of TCI states in the S-DCI are indicated for two PDSCHs. Then which one of the two sets of indicated TCI states would be applied to PDCCH itself seems unclear. One simple way to handle this issue is to associate all CORESETs with either 1st set of indicated TCI state or the 2nd set of indicated TCI state. Such association can be done via RRC signaling or via a fixed mapping rule, e.g. the 1st set of indicated TCI state always applies to the CORESET in which the scheduling PDCCH is transmitted.

- For S-DCI MTRP, associate per each CORESET with either 1st set or 2nd set of indicated TCI state

- Via RRC signaling (Alt1) or

- Apply a pre-defined and fixed for the scheduling CORESET (Alt5).

### MTRP PDSCH

Regarding on whether the RRC parameter CORESETPoolIndex is presented or absent, the difference between S-DCI and M-DCI based MTRP operation can be identified. Given the current available solutions for both operations, it seems unified solutions for S-DCI and M-DCI MTRP cannot be easily achieved. The reason lies in the fact that for both operations, we cannot expect RRC signaling to configure all the same association/mapping for each channel and each S-DCI/M-DCI MTRP operation. Not to mention the fact that the association/mapping for S-DCI MTRP is still not widely discussed yet.

- Unified solution for S-DCI and M-DCI MTRP beam indication cannot be easily achieved.

In RAN1#109e, the following alternatives for TCI state updating mechanism for M-DCI based MTRP was achieved.

Agreement

On unified TCI framework extension for M-DCI based MTRP, consider the following alternatives for TCI state update:

- Alt1: Reuse the same TCI state update scheme for S-DCI based MTRP

- Atl2: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate the joint/DL/UL TCI state(s) corresponding to the same CORESETPoolIndex value

- Alt3: Use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) to indicate all joint/DL/UL TCI states corresponding to both CORESETPoolIndex values

  - Study the association between the indicated joint/DL/UL TCI state(s) and a CORESETPoolIndex value

- Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value.

  - Study whether the indicated joint/DL/UL TCI state(s) applies to the channels/signals associated with the same CORESETPoolIndex value or different CORESETPoolIndex value is indicated by DCI

Normally, the M-DCI based MTRP is based on non-ideal backhaul. Each TRP should at least update the set of TCI state for itself, rather than for the other TRP. Hence at least Alt.2 should be supported.

- For M-DCI based MTRP, support to use existing TCI field in DCI associated with one CORESETPoolIndex to indicate unified TCI state(s) for channel/signal corresponding to the same CORESETPoolIndex.

There could be issue for dynamic switch between STRP and MTRP PDSCH. In Rel.15/16, the number of indicated TCI state(s) implies either STRP or MTRP PDSCH transmission. But as for DL/joint TCI state, since there is HARQ feedback for the DCI indicating unified TCI states and BAT, the indicated DL/joint TCI state(s) is (are) not applicable when receiving PDSCH depicted in Figure 3. The number of indicated DL/joint TCI state(s) cannot be used by UE to identify STRP or MTRP transmission.

- : STRP/MTRP PDSCH dynamic switch

And we think to enable the dynamic switch between STRP and MTRP PDSCH as in legacy release, this issue should be studied by the group and solutions should be provided and selected.

- To enable dynamic switch between STRP and MTRP PDSCH using unified TCI state(s), study on how to differentiate STRP and MTRP PDSCH.

### MTRP PUCCH

In Rel.17, either STRP or MTRP PUCCH depends on how many spatial relation information(s) is (are) activated by MAC CE per PUCCH resource. Specifically, if two spatial relation information are activated, it’s MTRP PUCCH repetition; otherwise the STRP PUCCH should be transmitted by UE.

It seems straightforward to follow the same principle for UL/joint TCI state(s). That is if one set of UL/joint TCI state is indicated, the all PUCCH resource (or PUCCH resource group depending on RRC configuration) on the active BWP/CC should apply STRP transmission; otherwise (two sets of UL/joint TCI states are indicated), all PUCCH resources (or PUCCH resource group depending on RRC configuration) on the active BWP/CC should apply MTRP PUCCH repetition in time domain.

- All PUCCH resources or PUCCH resource group on a BWP/CC can be indicated with one set or two sets of UL/joint TCI states to differentiate STRP transmission or MTRP repetition.

### MTRP PUSCH

In Rel.17, the TDM-based PUSCH repetition toward different TRPs was specified to enhance UL data reliability. Specifically, two SRS resource sets can be signaled with different orders for MTRP UL transmission. The association between TRPs and UL scheduling information (e.g. TPMIs, SRIs) is signaled to UE as well. STRP transmission can also be dynamically switched ON by indicating one SRS resource set. The beam indication for PUSCH repetition is based on legacy spatial relation information. The mapping between SRS resource set(s) and SRI (carrying UL Tx beam information) is indicated.

In Rel.18, the enhancement of beam indication is to be conducted via UL/joint TCI state. Hence it is reasonable to build similar mapping between SRS resource sets and the sets of indicated UL/joint TCI states. Specifically, the 1st set and 2nd set of indicated UL/joint states are associated with the 1st and 2nd SRS resource sets respectively. Of course, the association relation can be switched (e.g. 1st set and 2nd set of indicated UL/joint TCI state associated with 2nd and 1st SRS resource set, respectively) if necessary.

- For MTRP PUSCH repetition and STxMP (if supported), associate/map the sets of indicated UL/joint TCI states to the SRS resource sets for beam indication.

# Unified TCI state for UL power control

In Rel.17, the UL power control parameters, i.e. PL-RS, and the set of alpha, P0 and CLI, can be optionally associated with UL/joint TCI state. This association can be considered as beam-level power control, which can be simply extended from STRP to MTRP by reusing similar approach as in Rel.17 unified TCI state design but on a per TRP basis. In RAN #109e, the following agreement was reached to associate the indicated UL/joint TCI state with UL PC parameters.

Agreement

On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion.

- FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported

- FFS: UL PC enhancement for CB and non-CB SRS in above case

FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

In Rel.17 for MTRP PUSCH repetition, two SRS resource sets can be mapped to two SRIs in UL scheduling DCI. The SRI field in DCI format 0_1/0_2 can dynamically indicate the UL PC parameters. The SRS resource set(s) can also be associated with UL/joint TCI state(s) to determine the indicated/updated UL beam(s) for transmission. Hence, the power control association chains can be descripted as in Figure 4 where the UL PC parameter set associated with UL/joint TCI state could be different from the one indicated by SRI. Since at each end of this chain, the UL PC parameter set is optionally managed (configured/activated) by NW. This potential power control parameter collision can be handled by NW via implementation.

- : UL power control parameter set chains

- Potential UL PC parameter set collision (one set from associated UL/joint TCI state and another set from indicated SRI) can be and should be addressed by NW via implementation.

For the case when both indicated UL/joint TCI states are not associated with UL PC for PUCCH/PUSCH, it is still applicable to reuse legacy approach for power control, i.e. in Rel.15/16 the UL PC parameters associated with PUCCH/PUSCH (on a per channel basis).

For the case when one or two indicated UL/joint TCI state(s) is (are) not associated with UL PC parameters for PUCCH/PUSCH, it’s reasonable to apply the Rel.17 method, i.e. UL PC parameters associated with each TRP for PUCCH/PUSCH repetition. Specifically, for each PUCCH resource, the UL PC parameter set can be associated with each PUCCH-SpatialRelationInfo and one PUCCH resource can be activated with 1 or 2 PUCCH-SpatialRelationInfo. As for PUSCH, 1 or 2 UL PC parameter set(s) can be associated with the indicated SRS resource set(s) with usage of CB/NCB. Hence, for MTRP power control in Rel.18, it would be good to reuse legacy scheme to handle the MTRP power control in Rel.17.

- For the cases when 1 or 2 indicated UL/joint TCI state(s) is (are) not associated with UL PC parameter set(s), reuse the Rel.17 UL PC parameter sets association for each TRP.

# Beam reporting enhancement for STxMP

In Rel.17, the fast panel selection was intensively discussed and finally specified with introduced UE capability set. Specifically, UE reports the selected panel type(s) (rather than the panel entity ID) associated with beam index(es) and performance metric, e.g. L1-RSRP. With these panel related beam reporting, NW indicates UL/joint TCI state which may contain the DL RS associated with the reported CRI/SSBRI. With such beam indication, single panel can be selected by UE for UL transmission, if symmetric panels deployed at UE.

In UE capability set, Rel.17 only specifies the number of SRS antenna port(s) to represent a panel type. However, there could be cases that two different panels (panel #A with maximum rank 4 and panel #B with maximum rank 2) have the same number of SRS ports (e.g. 4 in this case). With such beam reporting, NW cannot know the rank limit for UE selected panel. To facilitate UL scheduling, it would be helpful to let NW know more on the UL transmission capability of each UE panel. Therefore, along with the number of SRS antenna port(s), we suggest to add more attribute as UE capability set.

- To enhance panel-specific beam reporting, add the maximum number of supported layer(s) into UE capability set for STxMP.

Moreover, another artificial restriction in Rel.17 should be relaxed in Rel.18, i.e. allowing the reported UE capability sets to have the same value, e.g. the same number of SRS antenna port(s) and maximum supported layer(s) (if supported). From our experience, the symmetric antenna panel implementation is very common among commercial UEs. Assuming the case that when two same panels are deployed at UE, only one panel can be selected and therefore STxMP may not be enabled, at least for the function of multi-panel beam reporting.

- For STxMP, the beam reporting with UE capability set should be relaxed to allow symmetric panel implementation reported within one beam reporting instance in Rel.18.

# BFR enhancement using unified TCI states

In Rel.17, when unified TCI states configured and activated, the procedure of beam failure recovery can recover DL and UL new beam for PDCCH/PDSCH/AP-CSI-RS and PUCCH/PUSCH/SRS respectively. That results in the unified new beam for DL and/or UL. Similarly, for extending unified TCI states to MTRP, the similar UE behavior on beam setting can be studied and investigated as well. But given current status, this issue can be with low priority.

- Study how to extend the beam resetting of STRP BFR to MTRP BFR when using unified TCI state.

# Conclusion

In this section, allow us to repeat our proposals

- Study how to split the unified TCI framework as TRP-specific for beam indication/updating with high priority.

- Support the 1st set of indicated TCI state(s) for one TRP and the 2nd set of indicated TCI state(s) for the other TRP

- One set of indicated TCI state(s) can be either 1 joint TCI state or 2 separate DL/UL TCI states

- At least the set of indicated TCI state(s) can be used for discussion purpose

- For MTRP operation, support up to 2 joint TCI states and 4 separate DL/UL TCI states.

- Following Rel.17 unified TCI state configuration, either joint TCI states or separate DL/UL TCI states can be configured in one CC/BWP for MTRP operation, but not both.

- Enhance MAC CE to facilitate up to 4 separate DL/UL TCI states or 2 joint TCI states per codepoint.

- At least for joint TCI states, it’s unnecessary to increase the maximum number of codepoints (i.e. 8 codepoints in legacy MAC CE).

- For beam indication of MTRP operation, reuse Rel.17 DCI format 1_1/1_2, i.e. adding no additional TCI related field and TCI field bits.

- For the case when two sets of indicated TCI states are updated by only one set, study unified TCI state updating mechanism, i.e. whether UE should maintain two sets of indicated TCI states or fallback to STRP operation.

- For S-DCI MTRP, associate per each CORESET with either 1st set or 2nd set of indicated TCI state

- Via RRC signaling (Alt1) or

- Apply a pre-defined and fixed for the scheduling CORESET (Alt5).

- For M-DCI based MTRP, support to use existing TCI field in DCI associated with one CORESETPoolIndex to indicate unified TCI state(s) for channel/signal corresponding to the same CORESETPoolIndex.

- To enable dynamic switch between STRP and MTRP PDSCH using unified TCI state(s), study on how to differentiate STRP and MTRP PDSCH.

- All PUCCH resources or PUCCH resource group on a BWP/CC can be indicated with one set or two sets of UL/joint TCI states to differentiate STRP transmission or MTRP repetition.

- For MTRP PUSCH repetition and STxMP (if supported), associate/map the sets of indicated UL/joint TCI states to the SRS resource sets for beam indication.

- For the cases when 1 or 2 indicated UL/joint TCI state(s) is (are) not associated with UL PC parameter set(s), reuse the Rel.17 UL PC parameter sets association for each TRP.

- To enhance panel-specific beam reporting, add the maximum number of supported layer(s) into UE capability set for STxMP.

- For STxMP, the beam reporting with UE capability set should be relaxed to allow symmetric panel implementation reported within one beam reporting instance in Rel.18.

- Study how to extend the beam resetting of STRP BFR to MTRP BFR when using unified TCI state.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN#94e, Dec. 2021

- R1-2007179 Summary of EVM discussion for mTRP PDCCH reliability enhancements

- R1-2007182 Summary of AI:8.1.2.1 Enhancements for Multi-TRP URLLC for PUCCH and PUSCH