3GPP TSG RAN WG1 #110		R1-2206281

Toulouse, France, August 22th – 26th, 2022

Source:	OPPO

Title:	Discussion on the LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

RAN1 received a LS from RAN2 with questions on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig [1], in this contribution, we will discuss the questions and suggest our reply to the questions.

Discussion

- In the LS, RAN2 expressed their understanding of UE’s behaviour in the case when RRC parameter “sl-AllowedResourceSelectionConfig” is absent for a resource pool and asked RAN1 to confirm our understanding.

When the sl-AllowedResourceSelectionConfig is absent for a mode-2 transmission resource pool indicated by sl-TxPoolSelectedNormal (which can be included within sl-BWP-PoolConfig-r16/sl-BWP-PoolConfigCommon-r16 and within sl-BWP-PoolConfigPS-r17/sl-BWP-PoolConfigCommonPS-r17, as specified in TS 38.331), the UE is only allowed to use full sensing based resource selection scheme for transmission in this resource pool.

- In general, we share the same understanding with RAN2 on UE’s behaviour when RRC parameter “sl-AllowedResourceSelectionConfig” is absent. However, it should be pointed out that this is already the behaviour for all Rel-16 UEs since this RRC parameter is only introduced from Rel-17. Therefore, in our view, Rel-16 UE behaviour does not change. Therefore, the question from RAN2 targets only Rel-17 UEs that support full sensing, partial sensing and random resource selection. Hence, in the absence of the parameter “sl-AllowedResourceSelectionConfig”, Rel-17 UE’s behaviour should fall back to Rel-16. That is, full sensing based RA. RAN1 should make this point clear to RAN2 and reply to RAN2 accordingly.

Proposed reply:

RAN1 shares the same understanding in general with the above expressed by RAN2. In order to be clear, since the design of NR sidelink in Rel-17 is backward compatible to Rel-16 UEs operating in the same resource pool, the statement on “the UE is only allowed to use full sensing based resource selection scheme …” is meant for Rel-17 UEs capable of at least full sensing. Since Rel-16 UEs would not understand RRC parameter “sl-AllowedResourceSelectionConfig”, so they will always perform full sensing based RA, except in the exceptional pool.

Conclusion

In this contribution, we discussed the question of RAN2 LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig and suggested reply to question as below:

Proposed reply:

RAN1 shares the same understanding in general with the above expressed by RAN2. In order to be clear, since the design of NR sidelink in Rel-17 is backward compatible to Rel-16 UEs operating in the same resource pool, the statement on “the UE is only allowed to use full sensing based resource selection scheme …” is meant for Rel-17 UEs capable of at least full sensing. Since Rel-16 UEs would not understand RRC parameter “sl-AllowedResourceSelectionConfig”, so they will always perform full sensing based RA, except in the exceptional pool.

References

- R1-2205735 (R2-2206316) LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig