3GPP TSG RAN WG1 #110 R1-2206294 August 22nd – 26th, 2022, Toulouse, France CR-Form-v12.0. CHANGE REQUEST. 37.213; CR; rev; -; Current version:; 17.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on resolving issue for short control signaling. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_ext_to_71GHz-Core; Date:; 2022-08-22. Category:; C; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; Currently it is unclear from the specification text, whether the short control signaling exemption should apply per transmitting device or per cell, and also how to disable this exemption when this is not supported based on regional regulatory requirements. In order to solve this issue, the following CR is proposed.. -. Summary of change:; To specify that the short control signaling exemption applies per UE and to indicate to the UE when this exemption should be applied.. Consequences if not approved:; unclear specifications. Clauses affected:; 4.4.5. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR..

- 4.4.5	Exempted transmissions from sensing

In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, a gNB/UE may transmit the following transmission(s) on a channel without sensing the channel:

-	Transmission(s) of the discovery burst by the gNB;

-	If the higher layer parameter ShortControlSignalling-r17 is configured, Transmission(s) of the first message in a random access procedure by the UE.