3GPP TSG RAN WG1 #110						          	            R1-2206322

Toulouse, France, August 22nd – 26th, 2022

Source:	OPPO

Title:	Discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

# Introduction

In RAN1 109e meeting, some agreements and conclusion on subband non-overlapping full duplex was approved [1], as shown in the following:

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

At least study SBFD operation within a TDD carrier

Conclusion

For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

In this document, we focus on potential enhancements on subband non-overlapping full duplex.

# Discussion

First of all, because subband has been a long-time used terminology in RAN1/RAN2/RAN4 for resource allocation and MIMO functionalities, the simple plain use of “subband” in SBFD discussion could be confusing and misleading regarding to the configuration, granularity, applicable functionality and etc, especially for people not involving in RAN1 SBFD discussion and in case where the SBFD becomes non-transparent to resource allocation and MIMO. So we prefer to use in future SBFD discussion a new terminology that can hold what the current “subband” in SBFD discussion stands for (hopefully not to cause the revision of WID) and meanwhile avoid confusion from what the name of “subband” can represent in existing spec for resource allocation and MIMO. “Duplexing subband” seems a good candidate to meet the target.

Proposal 1: Use a new terminology such as “duplexing subband” to replace “subband” in SBFD discussion.

# Interference management

In RAN1 109e meeting, several interference types were summarized as following:

Agreement For discussion for duplex evolution study (all agenda items), consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation: - gNB self-interference (SI): Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency. - gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. - UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. - (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier. - (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier. - (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier. - (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. - (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. - (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency. - gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier. - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI. - UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier. Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs. Note: This does not imply we need to consider all the above interference types in evaluation for SBFD.

On gNB side, there are gNB self-interference and inter-gNB CLI including co-channel and adjacent-channel. The gNB self-interference is a new type of CLI, for which the mitigation methods can include antenna isolation, TX IM suppression in the RX part, filtering, digital interference suppression and guard band allocation. For inter-gNB CLI, mitigation techniques can have coordinated scheduling/beamforming and power control.

On UE side, the CLI is only the inter-UE CLI that also can be co-channel and adjacent channel. UE TX power is rather limited and interfering range is very limit, so gNB scheduling is the most effective and feasible solution to mitigate inter-UE CLI. Moreover, R16 WI of NR_CLI_RIM introduces some inter-UE CLI handling, which can be a baseline reusable technique. In addition, due to inter-UE CLI can be limited to a bandwidth matching the subband, so subband-alike based interference measurement in UE side can be considered.

Observation 1: For gNB-side CLI,

- The inter-gNB CLI can have mitigation techniques such as coordinated scheduling/beamforming and power control.

- The gNB self-interference can have mitigation techniques such as antenna isolation, TX IM suppression in the RX part, filtering, digital interference suppression and guard band allocation.

Observation 2: For inter-UE CLI, R16 CLI handling can be reused. The enhancement of interference measurement by UE on a subband-equivalent bandwidth can be considered.

# SBFD notification and operation

## Transparency of duplexing subband to UE

It is an open issue whether/how to notify a UE the time and/or frequency locations of a subband that gNB would use for SBFD operation. Generally, there are three alternatives for such notification, as listed in Table 1:

- Alt-1: Both time and frequency locations of duplexing subband are kept unknown to UE. The legacy UE certainly falls into this alternative. Besides, it is also feasible to have full SBFD transparency to Rel-18 UE.

- Alt-2: The time allocation of duplexing subband is made known to UE but the frequency domain allocation is not.

- Alt-3: Both time and frequency locations of duplexing subband are made known to UE.

Table 1 Alternatives of SBFD notification to UE

|  | Alt-1 | Alt-2 | Alt-3 |
| --- | --- | --- | --- |
| Time-domain allocation | Transparent to UE | Non-transparent to UE |  |
| Frequency-domain allocation | Transparent to UE |  | Non-transparent to UE |

It seems not necessary for RAN1 to consider the case where the time domain allocation of SBFD is transparent to UE but the frequency domain allocation is not, because the UL subband is not supposed to occur in UL symbol and therefore certain time domain boundary should exist in duplexing subband notification.

Proposal 2: Rel-18 duplexing SI should study the following alternatives for a duplexing subband allocated at gNB.

- Alt-1: Both time-domain allocation and frequency-domain allocation of the duplexing subband are transparent to UE.

- Alt-2: Frequency-domain allocation of the duplexing subband is transparent to UE but the time-domain allocation is not.

- Alt-3: None of time-domain allocation and frequency-domain allocation of the duplexing subband is transparent to UE.

## Time-domain transparency

For the time-domain transparency of SBFD that is only in Alt-1, a SBFD symbol from gNB perspective can contain both a group of REs actually used as DL for one UE and another group of REs actually used as UL for another UE. Then the SBFD symbol should be configured as flexible symbol by a cell-specific TDD-UL-DL-ConfigCommon, followed by either of the following configuration or indication already supported in current spec:

- The symbol is further configured as DL symbol or UL symbol by UE-dedicated TDD-UL-DL-ConfigDedicated; or

- The symbol is further indicated as DL symbol or UL symbol by dynamic SFI.

With time-domain transparency, the amount of SBFD symbols is restricted by the amount of flexible symbols in TDD-UL-DL-ConfigCommon.

For the time-domain non-transparency of SBFD that is in Alt-2 and Alt-3, there could be two logics to interpret the notification from gNB:

- Logic-1: A notification of UL subband means to add UL transmission possibility on top of the DL reception possibility that is already informed to UE, i.e., UE treats the subband-overlapping symbol as “flexible” and needs further gNB instruction to determine whether the symbol is eventually used for DL or UL, as the current spec formulates. Within this logic, it does not seem beneficial to define UL subband over a true flexible symbol.

- Logic-2: A notification of UL subband means to replace DL reception possibility that is already informed to UE with the UL transmission possibility, i.e., UE treats the subband-overlapping symbol as “uplink” and does not expect any DL reception in the subband (even when the UE has no UL transmission on the symbol).

Proposal 3: In case the time domain allocation of UL subband is non-transparent to UE, RAN1 study considers the subband-overlapping symbol as converted to “flexible symbol” or “uplink symbol” from UE perspective, and avoids defining UL operations directly on a DL symbol.

## Frequency-domain transparency

The frequency-domain transparency of an UL subband that is in Alt-1 and Alt-2 needs to meet two conditions:

- Condition-1: An UL transmission from a UE can be controlled in a transparent way to not exceed the bandwidth of a certain block of RBs in frequency domain;

- Condition-2: A DL reception by a UE can be controlled in a transparent way to not involve the REs in a certain block of RBs in frequency domain.

For Condition-1, gNB dynamic grant is the tool that can control the DG-PUSCH within certain range of RBs. It is also feasible to ensure CG-PUSCH to fully fall into a certain range of RBs by CG configuration. For PRACH, SRS and PUCCH, to keep these UL signals away from SBFD symbols can be always a last bottom line method. For example, with some UL traffic moves to SBFD symbols, additional resources in true UL symbols can be allocated for PUCCH so that PUCCH can also somehow share the SBFD benefit.

For Condition-2, it is natural to keep cell-specific and UE-group-common signals/channels away from SBFD subband or even symbols overlapping with the subband in order to avoid impacts to legacy UE. These kinds of signals at least include SSB, PRS, CSI-RS (for cell-specific or UE group common use) and PDCCH/PDSCH for broadcast purpose. For the channels/signals transmitted to a single UE, configuration and scheduling are always a last bottom line method. Take CSI-RS dedicated to UE for an example. The full-band NZP CSI-RS shown in Figure 1 is cut by UL subband into two segments: one segment above UL subband and another segment below UL subband. Because one CSI-RS resource is configured with CSI-FrequencyOccupation IE, which is able to assign starting RB position and ending RB position of the UL subband,  gNB can assign the CSI-RS segments respectively above the UL subband and below the UL subband with different resource_id (id=a and id=b in Figure 1) and include the two CSI-RS segments into one CSI-RS resource set.

CSI-FrequencyOccupation ::= SEQUENCE { startingRB INTEGER (0..maxNrofPhysicalResourceBlocks-1), nrofRBs INTEGER (24..maxNrofPhysicalResourceBlocksPlus1), ... } nrofRBs Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 24 and the width of the associated BWP. If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP.. startingRB PRB where this CSI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid. Only multiples of 4 are allowed (0, 4, ...).

Figure 1 CSI-RS configuration around UL subband

Based on above analysis for both time-domain transparency and frequency-domain transparency, the legacy UE can run with half-duplex mode in SBFD symbols without knowing the UL subband allocation at gNB side.

Observation 3: The legacy UE can run with half-duplex mode in SBFD symbols with both time-domain transparency and frequency-domain transparency to UL subband allocation at gNB.

## SBFD operation

One question for SBFD operation is whether the duplexing subband allocation modifies the BWP configuration in a carrier, for example, whether a UL subband of size of 24 RBs leads to a UL BWP size reduction to 24 RBs on the SBFD symbols. It is desirable that SBFD operation on gNB side does not increase UE implementation complexity. So, the legacy UE restriction on UL and DL BWPs should be maintained, i.e., DL and UL BWP pair with the same configured bwp-id must have the same center frequency. Moreover, up to one DL and UL BWP pair can be configured in one symbol. To avoid long BWP switching time between SBFD symbol and non-SBFD symbol with the same transmission direction, both center frequency and bandwidth of BWP should keep the same.

Proposal 4: Center frequency and bandwidth of BWP should keep the same between SBFD symbol and non-SBFD symbol with the same transmission direction.

Another issue for SBFD operation is whether different SBFD configuration is allowed between adjacent carriers. As shown in Figure 2, there are five possible SBFD configuration combinations over adjacent carriers {CC1, CC2}. Scheme A means that SBFD configuration keeps the same between two carriers. Schemes B and D mean that SBFD configuration can be different between two carriers, however, transmission directions in two adjacent edge subbands should keep the same to avoid CLI from adjacent carrier. In contrast, schemes C and E means that both the SBFD configurations and transmission directions in two adjacent edge subbands can be different between two carriers. Because adjacent-channel CLI is almost none for schemes A, B and D, there is no additional interference management task for schemes {A, B, D} comparing to interference management in a single carrier. Therefore at least schemes {A, B, D} can be assumed for SBFD subband allocation.

Figure 2 SBFD operations between adjacent carriers

Observation 4: The SBFD subband allocation does not have to be the same over adjacent carriers, but the adjacent edge subbands are desired to have the same transmission direction.

# Conclusions

In this contribution, we discuss subband non-overlapping full duplex with the following observations and proposals:

Observation 1: For gNB-side CLI,

- The inter-gNB CLI can have mitigation techniques such as coordinated scheduling/beamforming and power control.

- The gNB self-interference can have mitigation techniques such as antenna isolation, TX IM suppression in the RX part, filtering, digital interference suppression and guard band allocation.

Observation 2: For inter-UE CLI, R16 CLI handling can be reused. The enhancement of interference measurement by UE on a subband-equivalent bandwidth can be considered.

Observation 3: The legacy UE can run with half-duplex mode in SBFD symbols with both time-domain transparency and frequency-domain transparency to UL subband allocation at gNB.

Observation 4: The SBFD subband allocation does not have to be the same over adjacent carriers, but the adjacent edge subbands are desired to have the same transmission direction.

Proposal 1: Use a new terminology such as “duplexing subband” to replace “subband” in SBFD discussion.

Proposal 2: Rel-18 duplexing SI should study the following alternatives for a duplexing subband allocated at gNB.

- Alt-1: Both time-domain allocation and frequency-domain allocation of the duplexing subband are transparent to UE.

- Alt-2: Frequency-domain allocation of the duplexing subband is transparent to UE but the time-domain allocation is not.

- Alt-3: None of time-domain allocation and frequency-domain allocation of the duplexing subband is transparent to UE.

Proposal 3: In case the time domain allocation of UL subband is non-transparent to UE, RAN1 study considers the subband-overlapping symbol as converted to “flexible symbol” or “uplink symbol” from UE perspective, and avoids defining UL operations directly on a DL symbol.

Proposal 4: Center frequency and bandwidth of BWP should keep the same between SBFD symbol and non-SBFD symbol with the same transmission direction.

# Reference

# [1] RAN1 #109e meeting chairman notes