3GPP TSG RAN WG1 #110		                                                                 R1-2206344

Toulouse, France, August 22nd – 26th, 2022

Title:	Draft Reply LS on power-saving resource allocation with absent sl-AllowedResourceSelectionConfig

Response to:	R1-2205735 /R2-2206316

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	[RAN1], CATT, GOHIGH

To:	RAN2

Cc:

Attachment:	-

Contact Person:

Name:	Xiaoran Wen

E-mail Address:	wenxiaoran@gohigh.com.cn

Overall description

RAN1 thanks RAN2 for the LS on power-saving resource allocation on how to understand the intended UE behavior in a resource pool for which sl-AllowedResourceSelectionConfig is absent in RAN1. RAN1 would like to provide the response to the following understanding of RAN2.

- When the sl-AllowedResourceSelectionConfig is absent for a mode-2 transmission resource pool indicated by sl-TxPoolSelectedNormal (which can be included within sl-BWP-PoolConfig-r16/sl-BWP-PoolConfigCommon-r16 and within sl-BWP-PoolConfigPS-r17/sl-BWP-PoolConfigCommonPS-r17, as specified in TS 38.331), the UE is only allowed to use full sensing based resource selection scheme for transmission in this resource pool.

RAN1 response: RAN1 share the same view with RAN2 that when sl-AllowedResourceSelectionConfig is absent in a resource pool, the UE is only allowed to use full sensing based resource selection scheme for transmission in this resource pool. That is, the UE should follow the same behavior in R16 resource allocation mode 2. Random selection can only be performed in an exceptional resource pool in this case.

Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take the above responses into account in the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#110			22nd -26th Aug 2022				Toulouse, France

RAN1#110-bis-e			10th -19th Oct 2022				E-meeting