3GPP TSG RAN WG1 #110	                                                 R1-2206348

Toulouse, France, August 22nd – 26th, 2022

Source:	CATT

Title:	Discussion on timing advance (TADV) report mapping for NR UL E-CID positioning

Agenda item:	5

Document for:	Discussion and Decision

Introduction

RAN4 has introduced the Timing advance (TADV) measurement report mapping in TS 38.133, Rel-17, for the timing advance (TADV) measurement specified in clause 5.2.7 of TS 38.215 v17.1.0, and has sent an LS to RAN1 for information and action [1].

This contribution discusses and gives our views on the issue in the RAN4’s LS.

Discussion

In the RAN4 LS R1-2205712(R4-2211167) [1], RAN4 informs that RAN4 has introduced the following Timing advance (TADV) measurement report mapping in TS 38.133, Rel-17, for the timing advance (TADV) measurement specified in clause 5.2.7 of TS 38.215 v17.1.0:

The reporting range of Timing advance (TADV) is defined from 0 to 3150848 Tc with: - 128 Tc resolution for timing advance less or equal to 262144 Tc and - 512 Tc for timing advance greater than 262144 Tc. The mapping of measured quantity is defined in Table 13.X.1-1: Table 13.X.1-1: Timing advance (TADV) measurement report mapping Reported value; Measured quantity value; Unit. TIME_ADVANCE_00; TADV 128; Tc. TIME_ADVANCE_01; 128 TADV < 256; Tc. TIME_ADVANCE_02; 256 TADV < 384; Tc. …; …; …. TIME_ADVANCE_2046; 261888 TADV < 262016; Tc. TIME_ADVANCE_2047; 262016 TADV < 262144; Tc. TIME_ADVANCE_2048; 262144 TADV < 262656; Tc. TIME_ADVANCE_2049; 262656 TADV < 263168; Tc. …; …; …. TIME_ADVANCE_7688; 3149824 TADV < 3150336; Tc. TIME_ADVANCE_7689; 3150336 TADV < 3150848; Tc. TIME_ADVANCE_7690; 3150848 TADV; Tc. NOTE: For report mapping, the TADV equal is to (gNB Rx – Tx time difference) + NTA_offset, where NTA_offset is based on the information n-TimingAdvanceOffset as specified in TS 38.331.

The above timing advance (TADV) report mapping consists of only positive values. Therefore, RAN4 think it is up to RAN1 to judge if this has any impact on RAN1 specifications.

In our point of view, the above NOTE in the RAN4’s LS should be added into the RAN1 specification TS 38.215, in order to align with the agreement in RAN4 and make the definition of TADV more clear.

Proposal 1: The following NOTE in the RAN4’s LS R1-2205712(R4-2211167) should be added into the definition of TADV in TS 38.215:

- NOTE: For report mapping, the TADV is equal to (gNB Rx – Tx time difference) + NTA_offset, where NTA_offset is based on the information n-TimingAdvanceOffset as specified in TS 38.331.

Conclusions

In this contribution, we discuss the issue in the RAN4’s LS, and give the following proposals:

Proposal 1: The following NOTE in the RAN4’s LS R1-2205712(R4-2211167) should be added into the definition of TADV in TS 38.215:

- NOTE:For report mapping, the TADV is equal to (gNB Rx – Tx time difference) + NTA_offset, where NTA_offset is based on the information n-TimingAdvanceOffset as specified in TS 38.331.

References

- R4-2211167, “LS on timing advance (TADV) report mapping for NR UL E-CID positioning”, RAN4, RAN4#103-e.

- 3GPP TS 38.215, “NR; Physical layer measurements (Release 17)”, v17.1.0, 2022-03.