3GPP TSG RAN WG1 #110			R1-2206468

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	8.11

Source: 	NEC

Title:	Editorial correction for inter-UE coordination

Document for:	Discussion/Decision

Introduction

In this contribution, we discussed some editorial issues for IUC including missing higher layer parameters discussed in section 2.1 and a number of editorial corrections discussed in section 2.2.

Discussion

Missing RRC parameters

RAN1 had following working assumption at RAN1#197-e meeting for scheme 2:

- Working Assumption: - A resource pool level (pre-)configuration can enable one of the following options: - Option 1: - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs - For the case when UE-A is a destination UE of a TB transmitted by UE-B - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI: - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively - For the case when UE-A is a destination UE of a TB transmitted by another UE - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI: - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively - Option 4: - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs - For the case when UE-A is a destination UE of a TB transmitted by UE-B - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource. - For the case when UE-A is a destination UE of a TB transmitted by another UE - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s). - Support of Option 4 is subject to UE capability - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

In the higher layer parameter list to RAN2, RAN1 set following parameters regarding whether to select either option 1 or option 4 but without indicating that the two RSRP thresholds under option 1 and option 4 also need define new parameters.

optionForCondition2A1Scheme2 {RSRP-ThresPerPriorities, RSRP-ThresWithRsrpMeasurement}

Indicates whether additional criteria for condition 2-A-1 is considered based either on the RSRP threshold according to the priorities included in the SCI or based on a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’ or another UE's reserved resource.

As a consequence, in TS 38.213 and TS 38.331, parameters ThresPSSCH-RSRP-List and deltaRSRPThresh are still missing. Please note that "optionForCondition2A1Scheme2" in TS 38.213 corresponding to "sl-OptionForCondition2-A-1-r17" in TS 38.331, which will be shown together in section 2.2.

TS 38.213 - if optionForCondition2A1Scheme2 = 'RSRP-ThresPerPriorities', the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if optionForCondition2A1Scheme2 = 'RSRP-ThresWithRsrpMeasurement', the first UE can be provided a value Delta_Th by deltaRSRPThresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th

TS 38.331 SL-InterUE-CoordinationScheme2-r17 ::= SEQUENCE { sl-IUC-Scheme2-r17 ENUMERATED {enabled} OPTIONAL, -- Need R sl-RB-SetPSFCH-r17 BIT STRING (SIZE (10..275)) OPTIONAL, -- Need M sl-TypeUE-A-r17 ENUMERATED {enabled} OPTIONAL, -- Need R sl-PSFCH-Occasion-r17 INTEGER (0..1) OPTIONAL, -- Need M sl-SlotLevelResourceExclusion-r17 ENUMERATED {enabled} OPTIONAL, -- Need R sl-OptionForCondition2-A-1-r17 INTEGER (0..1) OPTIONAL, -- Need M sl-IndicationUE-B-r17 ENUMERATED {enabled, disabled} OPTIONAL, -- Need M ... }

Proposal 1: Send LS to RAN2 to indicate the above two RSRP thresholds in the working assumption made in RAN1#107-e meeting.

RRC parameters alignment and editorial correction

There are a slot of parameters used in TS 38.213 and TS 38.214 but can not be found in TS 38.331 due to parameter name misalignment.

In addition, we noticed that sl-TypeUE-A-r17 is configured as ENUMERATED {enabled}, so the following correction is also needed in TS 38.213 16.3.0:

-	if, for a resource pool, sl-TypeUE-A-r17 is not provided, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A-r17is provided, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception,

Please see our proposed corrections in section 5 including RRC name corrections and editorial corrections both for TS 38.213 and TS 38.214.

Proposal 2: Adopt the editorial corrections for IUC in TS 38.213 and TS 38.214 in section 5.

Conclusion

In this contribution, we discussed some issues about IUC and proposed that:

Proposal 1: Send LS to RAN2 to indicate the above two RSRP thresholds in the working assumption made in RAN1#107-e meeting.

Proposal 2: Adopt the editorial corrections for IUC in TS 38.213 and TS 38.214 in section 5.

Reference

[1] Chairman's note of RAN1#107-e.

[2] TS 38.213 V17.2.0

[3] TS 38.214 V17.2.0

[4] TS 38.331 v17.1.0

Text proposal

TS 38.213 V17.2.0

### 16.3	UE procedure for reporting and obtaining control information in PSFCH

Control information provided by a PSFCH transmission includes HARQ-ACK information or conflict information.

#### 16.3.0	UE procedure for transmitting PSFCH with control information

A UE can be indicated by an SCI format scheduling a PSSCH reception to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK.

A UE can be provided, by sl-PSFCH-Period, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled.

A UE can be enabled, by sl-InterUE-CoordinationScheme2-r17, to transmit a PSFCH with conflict information in a resource pool. The UE can determine, based on an indication by a SCI format 1-A, a set of resources that includes one or more slots and resource blocks that are reserved for PSSCH transmission. If the UE determines a conflict for a reserved resource for PSSCH transmission, the UE provides conflict information in a PSFCH.

A UE expects that a slot t'kSL (0≤k<T'max) has a PSFCH transmission occasion resource if k mod NPSSCHPSFCH=0, where t'kSL is defined in [6, TS 38.214], and T'max is a number of slots that belong to the resource pool within 10240 msec according to [6, TS 38.214], and NPSSCHPSFCH is provided by sl-PSFCH-Period.

A UE may be indicated by higher layers to not transmit a PSFCH that includes HARQ-ACK information in response to a PSSCH reception [11, TS 38.321].

If a UE receives a PSSCH in a resource pool and the HARQ feedback enabled/disabled indicator field in an associated SCI format 2-A/2-B/2-C has value 1 [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH-r16, of the resource pool after a last slot of the PSSCH reception.

A UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information in a PRB of the resource pool. A UE can be provided by sl-RB-SetPSFCH-r17 a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with conflict information in a PRB of the resource pool. A UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH.

The second OFDM symbol l' of PSFCH transmission in a slot is defined as l'=sl-StartSymbol+sl-LengthSymbols-2.

A UE determines a number of PSFCH resources available for multiplexing HARQ-ACK or conflict information in a PSFCH transmission as RPRB, CSPSFCH=Ntype PSFCH⋅Msubch, slotPSFCH⋅NCSPSFCH where NCSPSFCH is a number of cyclic shift pairs for the resource pool provided by sl-NumMuxCS-Pair and, based on an indication by sl-PSFCH-CandidateResourceType,

-	if sl-PSFCH-CandidateResourceType is configured as startSubCH, Ntype PSFCH=1 and the Msubch, slotPSFCH PRBs are associated with the starting sub-channel of the corresponding PSSCH

-	if sl-PSFCH-CandidateResourceType is configured as allocSubCH, Ntype PSFCH=Nsubch PSSCH and the Nsubch PSSCH⋅Msubch, slotPSFCH PRBs are associated with the Nsubch PSSCH sub-channels of the corresponding PSSCH

-	for conflict information, the corresponding PSSCH is determined based on sl-PSFCH-Occasion-r17

The PSFCH resources are first indexed according to an ascending order of the PRB index, from the Ntype PSFCH⋅Msubch, slotPSFCH PRBs, and then according to an ascending order of the cyclic shift pair index from the NCSPSFCH cyclic shift pairs.

A UE determines an index of a PSFCH resource for a PSFCH transmission with HARQ-ACK information in response to a PSSCH reception or with conflict information corresponding to a reserved resource as PID+MIDmodRPRB, CSPSFCH where PID is a physical layer source ID provided by SCI format 2-A/2-B/2-C [5, TS 38.212] scheduling the PSSCH reception, or by SCI format 2-A/2-B/2-C with corresponding SCI format 1-A reserving the resource from another UE to be provided with the conflict information. For HARQ-ACK information, MID is the identity of the UE receiving the PSSCH as indicated by higher layers if the UE detects a SCI format 2-A with Cast type indicator field value of "01"; otherwise, MID is zero. For conflict information, MID is zero.

For a PSFCH transmission with HARQ-ACK information or conflict information, a UE determines a m0 value, for computing a value of cyclic shift α [4, TS 38.211], from a cyclic shift pair index corresponding to a PSFCH resource index and from NCSPSFCH using Table 16.3-1.

Table 16.3-1: Set of cyclic shift pairs

| NCSPSFCH | m0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cyclic Shift Pair Index 0 | Cyclic Shift Pair Index 1 | Cyclic Shift Pair Index 2 | Cyclic Shift Pair Index 3 | Cyclic Shift Pair Index 4 | Cyclic Shift Pair Index 5 |
| 1 | 0 | - | - | - | - | - |
| 2 | 0 | 3 | - | - | - | - |
| 3 | 0 | 2 | 4 | - | - | - |
| 6 | 0 | 1 | 2 | 3 | 4 | 5 |

For a PSFCH transmission with HARQ-ACK information, a UE determines a mcs value, for computing a value of cyclic shift α [4, TS 38.211], as in Table 16.3-2 if the UE detects a SCI format 2-A with Cast type indicator field value of "01" or "10" or a SCI format 2-C, or as in Table 16.3-3 if the UE detects a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11". For a PSFCH transmission with conflict information, a UE determines a mcs value for computing a value of cyclic shift α [4, TS 38.211] as in Table 16.3-4. The UE applies one cyclic shift from a cyclic shift pair to a sequence used for the PSFCH transmission [4, TS 38.211].

Table 16.3-2: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes ACK or NACK

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | 0 | 6 |

Table 16.3-3: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes only NACK

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | 0 | N/A |

Table 16.3-4: Mapping of conflict information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission

| Conflict information | Conflict information for a next in time reserved resource indicated in SCI |
| --- | --- |
| Sequence cyclic shift | 0 |

A first UE determines a second UE for providing the conflict information to in a PSFCH as follows

-	if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot,

-	does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and

-	determines to transmit to the second UE the PSFCH with the conflict information.

A first UE determines a UE for providing the conflict information to in a PSFCH as follows

-	if, for a resource pool, sl-TypeUE-A-r17 is not provided, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A-r17is provided, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception,

-	detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE,

-	detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and

-	determines that the first and second resources overlap in time and frequency

-	the PSFCH occasions for resource conflict information of the second UE and the third UE are valid

-	the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if sl-IndicationUE-B-r17 = 'enabled'

-	determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH-r16 before the PSFCH occasion for conflict information

-	determines to transmit to the second UE the PSFCH with the conflict information

-	determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1

The first UE can be provided conditions by sl-OptionForCondition2-A-1-r17 to determine conflict of reserved resources in a resource pool

-	if sl-OptionForCondition2-A-1-r17 = 0, the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214]

-	if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1

-	if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2

-	if sl-OptionForCondition2-A-1-r17 = 1, the first UE can be provided a value Delta_Th by deltaRSRPThresh

-	if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively

-	if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th

If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion-r17

-	If sl-PSFCH-Occasion-r17 = 0, the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH-r16, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information.

-	If sl-PSFCH-Occasion-r17 = 1, the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH-r16 slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information.

#### 16.3.1	UE procedure for receiving PSFCH with control information

A UE that transmitted a PSSCH scheduled by a SCI format 2-A/2-B/2-C that indicates HARQ feedback enabled, attempts to receive associated PSFCHs with HARQ-ACK information according to PSFCH resources determined as described in clause 16.3.0. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [8-4, TS 38.101-4]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource.

For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following

-	if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" or a SCI format 2-C

-	report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception

-	if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01"

-	report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in clause 16.3; otherwise, report a NACK value to higher layers

-	if the PSFCH reception occasion is associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11"

-	report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers

A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by inter- sl-InterUE-CoordinationScheme2-r17, attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers

-	if sl-SlotLevelResourceExclusion-r17 is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A

-	if sl-SlotLevelResourceExclusion-r17 is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A

If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion-r17

-	If sl-PSFCH-Occasion-r17 = '0', the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH-r16, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not receive the PSFCH with conflict information

-	If sl-PSFCH-Occasion-r17 = '1', the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH-r16 slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not receive the PSFCH with conflict information

TS 38.214 V17.2.0

### 8.1.4A	UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission

When this procedure is triggered, the following parameters are provided by the higher layer:

-	the resource pool from which the preferred or non-preferred resources are to be determined;

-	the resource selection window [n+T1,n+T2] within which the preferred or non-preferred resources are to be determined;

-	the resource set type (either preferred or non-preferred resource set);

-	if the resource set type indicates preferred set, then the higher layer additionally provides the following parameters:

-	L1 priority, prioTX;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	the resource reservation period, Prsvp_TX, if present.

The value of Cresel is determined by the UE according to clause 8.1.5.

When this procedure is triggered by another UE's explicit request, the fields in the request are interpreted as follows:

-	The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A.

-	The field 'Resource reservation period' is encoded in the same way as the field of the same name in SCI format 1-A.

When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications:

-	Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met:

-	the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined;

-	the higher layer parameter sl-Condition1-A-2-r17 is not set to 'Disabled'.

When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s):

-	resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria:

-	the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option1List-r17, where k=pi.

-	the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option2List-r17, where k=pi.

-	resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined.

### 8.1.4B	UE procedure for determining a resource conflict

A UE configured with the higher layer parameter sl-InterUE-CoordinationScheme2-r17 enabling transmission of a resource conflict indication considers that a resource conflict occurs on a first reserved resource r1 indicated by a first received SCI format if at least one of the following conditions is satisfied:

-	the first reserved resource r1 overlaps with a second reserved resource r2 indicated by a second received SCI format, and

-	if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 1 enabled],

-	if the UE is a destination UE of a TB to be transmitted in r1, the RSRP measurement performed for the second received SCI format RSRP2 is higher than Thprio2,prio1 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively.

-	if the UE is a destination UE of a TB to be transmitted in r2, the RSRP measurement performed for the first received SCI format RSRP1 is higher than Thprio1,prio2 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively.

-	if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 4 enabled] and the UE supports [Option 4],

-	if the UE is a destination UE of a TB to be transmitted in r1, RSRP2-RSRP1 is higher than a (pre)configured RSRP threshold.

-	if the UE is a destination UE of a TB to be transmitted in r2, RSRP1-RSRP2 is higher than a (pre)configured RSRP threshold.

-	where the RSRP1, RSRP2 measurements are performed according to clause 8.4.2.1 and the parameter Th(pi,pj) is determined [as described in step 3 of clause 8.1.4].

-	the first reserved resource r1 occurs in a slot in which the UE does not expect to perform SL reception due to half-duplex operation and the UE is a destination UE of a TB to be transmitted in resource r1.

### 8.1.4C	UE procedure for using a received non-preferred resource set

A UE configured with the higher layer parameter sl-InterUE-CoordinationScheme1-r17 uses a received non-preferred resource set as follows when performing resource (re-)selection:

-	the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set.

Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement.

### 8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields 'Frequency resource assignment', 'Time resource assignment' of the associated SCI format 1-A as described below.

'Time resource assignment' carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl-MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=t1

else

if t2-t1-1≤15

TRIV=30t2-t1-1+t1+31

else

TRIV=3031-t2+t1+62-t1

end if

end if

where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31.

The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where.

If sl-MaxNumPerReserve is 2 then

FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i

If sl-MaxNumPerReserve is 3 then

FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2

where

-	nsubCH,1start denotes the starting sub-channel index for the second resource

-	nsubCH,2start denotes the starting sub-channel index for the third resource

-	N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter sl-NumSubchannel

If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.

The number of slots in one set of the time and frequency resources for transmission opportunities of PSSCH is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1.

If a set of sub-channels in slot t'mSL is determined as the time and frequency resource for PSSCH transmission corresponding to the selected sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots t'm+j×Prsvp_TX'SL are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t'0SL, t'1SL, t'2SL,… is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

### 8.1.5A	UE procedure for determining slots and resource blocks indicated by a preferred or non-preferred resource set

The set of slots and resource blocks indicated by a set of preferred or non-preferred resource(s) is determined as described below.

The set of preferred or non-preferred resources r0,r1,r2,…, is indicated by a reference slot tref and M tuples (TRIVm,FRIVm,Prsvp,m), 1≤m≤M indicated by the 'resource combination(s)' field, where for each tuple TRIVm is indicated by the 9 MSBs, followed by FRIVm and Prsvp,m (if present).

The reference slot tref is indicated by the 'reference slot location' field as a combination of DFN index and slot index [5, TS 38.212], with the 10 MSBs indicating the DFN index. TRIVm and FRIVm are interpreted according to clause 8.1.5, with the following modifications:

-	the value of sl-MaxNumPerReserve is fixed to 3.

-	"slot where SCI format 1-A was received" is replaced by slot indicated as the first resource location of a TRIVm.

-	the first resource location of each TRIVm for m>1 is indicated by a slot offset tm in logical slots with respect to the reference slot tref; the first resource location of TRIV1 is at slot offset 0 with respect to the reference slot.

-	"the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received" is replaced by "each tuple"

-	the starting sub-channel nsubCH,0start of the first resource of each tuple is separately indicated.

The starting sub-channel nsubCH,0start of the first resource of each tuple is indicated by the '[lowest subchannel index for the first resource location of each TRIV]' field. The resource reservation period Prsvp,m  is encoded as in SCI format 1-A.

If the set is indicated by an SCI format 2-C, the number of tuples is M=2.

A UE forms the union of the subsets indicated by each tuple (TRIVm,FRIVm,Prsvp,m) to obtain the set r0,r1,r2,….