3GPP TSG RAN WG1 #110			                                                              R1-2206474

Toulouse, France, August 22nd – 26th, 2022

Source:	NEC

Title:	Clarification on semi-static PUCCH cell switching for PUCCH repetitions

Agenda Item:	8.3

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss clarification of UE’s behaviors when semi-static PUCCH cell switching and PUCCH repetition are simultaneously configured for UE.

- Discussion

In last RAN1 meeting, whether and how to support joint operation of semi-static PUCCH cell switching and PUCCH repetition was discussed and following agreements were achieved [1].

Agreement For semi-static PUCCH cell switch and PUCCH repetitions: - Semi-static PUCCH cell switching is applicable only to PUCCH transmissions without repetitions. - Note: UE assumes there is no PUCCH scheduling on multiple slots mapped to PCell and PUCCH-sSCell. i.e., gNB need to schedule carefully so there is no such case where a PUCCH repetition from PCell would be need to be transmitted in a slot indicated by the pattern for PUCCH transmission on PUCCH-sSCell (as for slot #X+3 in the example figure below) - Conclusion: PUCCH repetitions are only applicable on Pcell, PScell, and PUCCH Scell.

When semi-static PUCCH cell switching and PUCCH repetition are simultaneously configured for UE, the agreement for UE’s behavior was captured in current specification as following.

## 9.A PUCCH cell switching This clause is applicable when a UE is provided a PUCCH-sSCell by pucch-sSCell and the PUCCH-sSCell is activated and does not have a dormant UL/DL active BWP. A UE can be provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern. Each bit of the pattern corresponds to a slot for a reference SCS configuration provided by tdd-UL-DL-ConfigurationCommon for the PCell with a value of '0' or a value of '1' indicating, respectively, the PCell or the PUCCH-sSCell as the cell for PUCCH transmissions during the slot of the reference SCS configuration. The UE does not transmit a PUCCH in a slot on a cell if the pattern indicates a different cell for PUCCH transmission during the slot. A slot on the active UL BWP of the PUCCH-sSCell does not overlap with more than one slot on the active UL BWP of the PCell. If a slot for the active UL BWP of the PCell overlaps with more than one slot on the active BWP of the PUCCH-sSCell and the UE would transmit a PUCCH on the PUCCH-sSCell, the UE considers the first of the overlapping slots for the PUCCH transmission on the PUCCH-sSCell.

However, we think that the current description without mentioning repetition transmission in the specification is not clear enough. Because the cell determination for PUCCH transmission with repetitions when semi-static PUCCH cell switching is enabled is not captured in the current specification, there may be two different understandings of “the cell for PUCCH transmission during the slot”, in other words, the cell here refers the cell indicated for the first PUCCH repetition or the Pcell should be clarified.

Proposal 1:

- Adopt following text change for clause 9.A in TS 38.213.

## 9.A PUCCH cell switching This clause is applicable when a UE is provided a PUCCH-sSCell by pucch-sSCell and the PUCCH-sSCell is activated and does not have a dormant UL/DL active BWP. A UE can be provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern. Each bit of the pattern corresponds to a slot for a reference SCS configuration provided by tdd-UL-DL-ConfigurationCommon for the PCell with a value of '0' or a value of '1' indicating, respectively, the PCell or the PUCCH-sSCell as the cell for PUCCH transmissions during the slot of the reference SCS configuration. The UE does not transmit a PUCCH repetition in a slot on a cell if the pattern indicates a different cell from the Pcell for PUCCH transmission during the slot. A slot on the active UL BWP of the PUCCH-sSCell does not overlap with more than one slot on the active UL BWP of the PCell. If a slot for the active UL BWP of the PCell overlaps with more than one slot on the active BWP of the PUCCH-sSCell and the UE would transmit a PUCCH on the PUCCH-sSCell, the UE considers the first of the overlapping slots for the PUCCH transmission on the PUCCH-sSCell.

- Conclusion

In this contribution, we discussed one remaining issue for HARQ-ACK feedback for URLLC. Proposals are summarized as following:

Proposal 1:

- Adopt following text change for clause 9.A in TS 38.213.

## 9.A PUCCH cell switching This clause is applicable when a UE is provided a PUCCH-sSCell by pucch-sSCell and the PUCCH-sSCell is activated and does not have a dormant UL/DL active BWP. A UE can be provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern. Each bit of the pattern corresponds to a slot for a reference SCS configuration provided by tdd-UL-DL-ConfigurationCommon for the PCell with a value of '0' or a value of '1' indicating, respectively, the PCell or the PUCCH-sSCell as the cell for PUCCH transmissions during the slot of the reference SCS configuration. The UE does not transmit a PUCCH repetition in a slot on a cell if the pattern indicates a different cell from the Pcell for PUCCH transmission during the slot. A slot on the active UL BWP of the PUCCH-sSCell does not overlap with more than one slot on the active UL BWP of the PCell. If a slot for the active UL BWP of the PCell overlaps with more than one slot on the active BWP of the PUCCH-sSCell and the UE would transmit a PUCCH on the PUCCH-sSCell, the UE considers the first of the overlapping slots for the PUCCH transmission on the PUCCH-sSCell.

References

- R1-2205504, ‘Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT,’ Nokia, Nokia Shanghai Bell.

- 3GPP RAN1 NR #109-e meeting, chairman’s notes.