3GPP TSG RAN WG1 #110			                                         R1-2206475

Toulouse, France, August 22nd – 26th, 2022

Source:	NEC

Title:	Discussion on XR-specific capacity enhancements

Agenda Item:	9.11.2

Document for: 	Discussion and Decision

- Introduction

In the RAN#95-e meeting, the SI on XR Enhancements for NR was approved [1]. For XR-specific capacity improvements, the objective is listed as below:

- Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms:

  - SPS and CG enhancements;

  - Dynamic scheduling/grant enhancements.

In this contribution, we provide our views on XR-specific capacity enhancements including SPS/CG enhancements and dynamic scheduling enhancements.

- Discussion

2.1 SPS/CG enhancements

In last RAN1 meetings, following agreements related to SPS/CG enhancements for XR capacity improvement were achieved [2].

Agreement: To study whether/how to support a candidate capacity enhancement technique for XR traffic based SPS/CG transmissions, companies are encouraged to consider the following studies: - Study enhancements related to multiple PDSCHs SPS transmission occasions in a period - Study enhancements related to multiple PUSCHs CG transmission occasions in a period - Study enhancements related to dynamic adaptation of SPS/CG parameters/configurations - Study enhancements related to non-integer periodicity for SPS/CG transmissions. - Note: Other studies are not precluded, as well as the combination of the above studies. - Follow the common principle for assessment of the candidate capacity enhancement technique

Many of the XR use cases are characterized by quasi-periodic traffic with high data rate, high reliability and variable packet size. In addition, both DL and UL traffic have relatively strict packet delay budget (PDB) requirements.  Semi-persistent scheduling transmission for periodic traffic can be considered to be applied for XR traffic to reduce scheduling overhead and latency.

While legacy SPS and CG configuration is mainly for small data packet transmission, e.g., for IIoT/URLLC service. One or more SPS/CG configurations can be configured for UE on a cell by gNB to reduce the latency or satisfy different services. For a SPS/CG configuration, only one PDSCH/PUSCH transmission occasion for a TB transmission per periodicity is allowed. So the current SPS mechanism is not well suited for XR use cases with large and varying data packet size. For example, for DL video steam of XR with 45 Mbps and 120 fps, the average payload size per frame equals 375 kbits, it is hard for UE to transmit such a large payload size in one TB carried by a SPS PDSCH. Though the issue can be addressed by configuring multiple SPS configurations, it needs multiple PDCCH transmissions for separately activating the multiple SPS configurations, which is inefficient and may lead to additional power consumption of XR UE. Therefore, we think the enhancement for SPS/CG that multi-PDSCH/PUSCH occasions per periodicity with a single SPS/CG configuration can be studied in Rel-18.

On the other hand, the multi-PDSCH/PUSCH occasions mechanism is also beneficial to alleviate the jitter effect. Due to the jitter effect, the precise arrival time of a XR packet is unpredictable, however the range of potential arrival time can be estimated by gNB/UE. Therefore, if multiple SPS-PDSCH/CG-PUSCH occasions are reserved within the range of potential arrival time, the XR packet can be transmitted in the nearest occasion after the packet arrived, which is beneficial to meet the stringent PDB requirement. However, it is possible that some of the SPS-PDSCH or CG-PUSCH occasions are not actually be used, therefore, it is necessary to study how to reduce the resource waste caused by the unused SPS-PDSCH/CG-PUSCH and the associated HARQ-ACK feedback.

In traditional NR, CBG based retransmission is supported for dynamic scheduled PDSCH to improve the transmission efficiency. Considering SPS PDSCH may be used for a TB transmission with large packet size for XR traffic in Rel-18, it is beneficial to also support CBG based transmission for SPS PDSCH in Rel-18. How to support CBG based retransmission for SPS PDSCH needs to be studied.

Proposal 1:

- Study multi-PDSCH/PUSCH occasions per SPS/CG period for XR traffic with large and varying packet size and the mechanism to alleviate the jitter effect.

Proposal 2:

- Study CBG based retransmission for SPS PDSCH for XR traffic with large packet size.

Another issue needs to be addressed is the mismatch between the periodicity of SPS/CG configuration and the XR packet arrival time. In Rel-17, the periodicity of a SPS/CG configuration is an integer number of slots, the minimum value of periodicity is 1 slot.  However, typical video stream of XR has 60/90/120 frames per second (i.e., fps), which means the arrival periodicity of XR packets is 1/60, 1/90 or 1/120 second, it is not well aligned with the periodicity of legacy SPS/CG configuration. It can be solved by configuring multiple SPS/CG configurations to match the non-integer periodicity of XR packet arrival time, but it will lead to overprovision of SPS/CG occasions. To avoid resource waste of overprovision of SPS/CG occasions, enhancement for matching the periodicity of a SPS/CG configuration and the XR packet arrival time, e.g., support non-integer periodicity of SPS/CG configuration, can be studied in Rel-18.

Proposal 3:

- Study enhancement for the mismatch between the periodicity of SPS/CG configuration and the XR packet arrival time

Currently, the following periodicities in symbols are supported depending on the configured subcarrier spacing [3]:

15 kHz:	2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640}

30 kHz:	2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280}

60 kHz with normal CP	2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560}

120 kHz:	2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120}

An offset can be added for UL configured grant type 1 and type 2 as defined in TS 38.321 [3], clause 5.8.2 in order to align the configured grant with the XR packet arrival time.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =

(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + startSymbol + N × periodicity + kOffsetSymbols) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot).

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =

[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity + kOffsetSymbols] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot).

- where kOffsetSymbols=

⌈ (1000ms/fps *2^µ *numberOfSymbolsPerSlot – periodicity) * N⌉

- where N is for Nth grant

- ‘fps’ (frame per second) is the frame rate for XR

As a burst of transport block transmissions across a number of consecutive slots may often be needed to deliver a single XR packet, a duration in number of consecutive slots can be defined so that each configured grant may support multiple transport block transmissions. ‘cg-nrofSlots’ may be reused to transmit different transport blocks if PUSCH repetition type is not set.

Proposal 4:

- Specify XR specific configured grant offset parameter such as kOffsetSymbols in Search Space Set configuration.

Proposal 5:

- ‘cg-nrofSlots’ may be reused to transmit different transport blocks if PUSCH repetition type is not set.

Proposal 6:

- Specify a higher layer parameter of ‘frame per second’ for the frame rate of XR traffic.

2.2 Dynamic scheduling enhancements

In last RAN1 meetings, following agreements related to dynamic scheduling enhancements for XR capacity improvement were achieved [2].

Agreement: To study whether/how to support a candidate capacity enhancement technique for XR traffic based dynamic scheduling/grant transmissions, companies are encouraged to consider the following studies: - Study enhancements related to extending capability of single DCI scheduling multi-PDSCHs/PUSCHs for FR2-2 to FR1/FR2. - Note: whether and how to discuss enhancements may depend on the outcome of Rel-17 B52.6G UE feature discussion - Study enhancements related to HARQ-ACK and/or CBG transmissions for single DCI scheduling one or multi PDSCH(s). - Study enhancements related to allowing different configurations per PDSCH/PUSCH - Study enhancement related to scheduling request and/or BSR with the focus on L1 enhancements. - Note: Other studies are not precluded as well as the combination of the above studies. - Follow the common principle for assessment of the candidate capacity enhancement technique.

In Rel-17, multi-slot PDSCH scheduling by single DCI is supported for above 52.6GHz to reduce the PDCCH control overhead and UE power consumption, which should be also applied to XR services to improve capacity. While CBG based transmission for multi-slot PDSCH scheduling is not supported in Rel-17 since it is not necessary for operation above 52.6GHz. For XR traffic with large packet size, CBG based transmission for multi-slot PDSCH scheduled by single DCI is beneficial for spectrum efficiency improvement and UE power consumption reduction. Therefore, CBG based transmission for multi-slot PDSCH scheduling by single DCI should be supported in Rel-18 XR. However, when the maximum number of CBGs per TB configured by RRC is large, it may lead to large bit width of CBGTI (CBG transmission information) field of scheduling DCI and large HARQ-ACK payload report, which should be further studied. In addition, when UE is configured with Type-2 HARQ-ACK, how to construct HARQ-ACK codebook for multiplexing HARQ-ACK bits for CBG based multi-slot PDSCHs transmission is also need to be studied.

Considering the high reliability requirements of XR traffic, another enhancement of multi-slot PDSCH scheduling can be considered is supporting repetition of multi-slot PDSCHs scheduled by single DCI.

Proposal 7:

- Study enhancement for CBG based transmission for multi-slot PDSCHs scheduling by single DCI.

Proposal 8:

- Study enhancement for repetition transmission for multi-slot PDSCHs scheduling by single DCI.

From an E2E perspective, an Application Data Unit (ADU) may be divided into multiple IP packets and then delivered to gNB or UE. Considering the large size of XR traffic packet, multiple transmission occasions may be needed to deliver the entire ADU. Apparently, only if all the transmissions are transmitted successfully within the PDB requirement, the ADU can be declared transmitted successfully. However, in some cases, gNB or UE may be not able to transmit all the transmissions before the PDB deadline. Then, transmitting the remaining out-of-date packets may be meaningless but lead to resource waste. Therefore, it is beneficial to support packet dropping based on the PDB requirement.

Proposal 9:

- Study mechanism of packet dropping based on the PDB requirement, in order to avoid resource waste due to the out-of-date packets.

- Conclusion

In this contribution, we discussed capacity enhancements for NR XR. Proposals are summarized as following:

Proposal 1:

- Study multi-PDSCH/PUSCH occasions per SPS/CG period for XR traffic with large and varying packet size and the mechanism to alleviate the jitter effect.

Proposal 2:

- Study CBG based retransmission for SPS PDSCH for XR traffic with large packet size.

Proposal 3:

- Study enhancement for the mismatch between the periodicity of SPS/CG configuration and the XR packet arrival time.

Proposal 4:

- Specify XR specific configured grant offset parameter such as kOffsetSymbols in Search Space Set configuration.

Proposal 5:

- ‘cg-nrofSlots’ may be reused to transmit different transport blocks if PUSCH repetition type is not set.

Proposal 6:

- Specify a higher layer parameter of ‘frame per second’ for the frame rate of XR traffic.

Proposal 7:

- Study enhancement for CBG based transmission for multi-slot PDSCHs scheduling by single DCI.

Proposal 8:

- Study enhancement for repetition transmission for multi-slot PDSCHs scheduling by single DCI.

Proposal 9:

- Study mechanism of packet dropping based on the PDB requirement, in order to avoid resource waste due to the out-of-date packets.

References

- RP-200285, ‘Revised SID: Study on XR Enhancements for NR,’ Nokia, Nokia Shanghai Bell.

[2]   3GPP RAN1 NR #109-e meeting, chairman’s notes.

[3]   3GPP TS 38.321, MAC Protocol Specification (Release 16)