3GPP TSG RAN WG1 #110			R1-2206485

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.1.1.2

Source:	Google

Title:	Discussion on two TAs for multi-DCI

Document for: 	Discussion

- Introduction

In this contribution, we discuss issues of two TA operation and provide our views in the following paragraphs. In Rel-18 MIMO WID [1], 2TA operation has been approved to study and, if justified, specify. In previous meeting(s), RAN1 has agreed to support 2TA operation, including intra- and inter-cell MTRP framework. RAN1 has also agreed 2TA operation is supported for both FR1 and FR2. Agreements from last RAN1 meeting are shown in the followings. However, there are a bunch of issues left to be technically debated and resolved. We have listed some issues and provided our opinions below.

RAN1 #109(e) Agreement Enhancement on two TAs for UL multi-DCI for multi-TRP operation is supported in Rel-18. Note 1: whether (1) the network signals two TACs or (2) the network signals one TAC and the UE deriving the second TA can be further studied. Note 2: evaluations can be considered on as-needed basis. Agreement For multi-DCI based multi-TRP operation, down-select one of the two alternatives: - Alt 1: configure two TAGs within a serving cell - Alt 2: consider two TAs within one TAG within a serving cell Agreement Support two TA enhancement for both intra-cell and inter-cell multi-DCI multi-TRP scenarios in Rel-18. Agreement Enhancements on two TAs for UL multi-DCI for multi-TRP operation are applicable to both FR1 and FR2. Agreement For multi-DCI multi-TRP operation with two TAs, study the following alternatives: - Alt 1: two reference timings are considered - Alt 2: one reference timing is considered Note: reference timing above is the timing of the DL reception Agreement For multi-DCI multi-TRP operation with two TAs, study the following alternatives further in Rel-18: - Alt 1: one n-TimingAdvanceOffset value per serving cell - Alt 2: two n-TimingAdvanceOffset value per serving cell Conclusion For multi-DCI multi-TRP operation with two TAs, the decision on the maximum uplink timing difference is left up to RAN4. - Send an LS to RAN4 asking them the maximum uplink timing difference RAN1 can assume between the two TAs for multi-DCI multi-TRP operation. The LS is endorsed in R1-2205593. Agreement Two TA enhancement for uplink multi-DCI based multi-TRP operation are applicable to at least: - TDM based multi-DCI uplink transmission - simultaneous multi-DCI uplink transmission (if simultaneous uplink multi-DCI uplink transmission is supported in Agenda 9.1.4.1) - Note: Whether two TA enhancement is applicable to other schemes is a separate discussion, which is not in the scope of AI 9.1.1.2.

- Discussion

In RAN1 #109(e), RAN1 was discussing whether to have one or two TAGs in responding to support of 2TA operation. In legacy, network may configure at most four TAGs, each of them includes a Time Alignment Timer (TAT) and is associated with a multiple of serving cells. The time alignment timer corresponds to a TA value, and controls whether the UL synchronization is aligned. In other words, one TAG corresponds to a time alignment timer and a TA value for a multiple of serving cells.

In 2TA operation, at least, UE needs to maintain two TA regardless of how to obtain the second TA value, which is a separate issue. Following the logic of configuring TAG in RAN2, we think it is more reasonable to configure two TAG for two TA values separately. In this way, RAN1’s needed functionality can be achieved without increasing too much RAN2 SPEC effort. Hence, we have the following proposal.

Proposal 1: For multi-DCI based multi-TRP operation, two TAGs corresponding to two TA values separately are configured for a serving cell.

Another one related issue is the number of time alignment timer for 2TA operation in a serving cell. In responding to two TA values, some companies were discussing whether to use one time alignment timer or two alignment timer. As mentioned in above paragraph, we support configuring two TAG for 2TA operation. Following the same logic, we also support having two respective time alignment timers as the following proposal.

Proposal 2: For multi-DCI based multi-TRP operation, two time alignment timers corresponding to two TA values separately are configured for a serving cell.

Before we go further about design details of 2TA operation, we should firstly make it clearly which beam indication framework where the 2TA operation is built on. The reason is that solutions of some open issues may be associated with beam indication (e.g., TA value association). Since the WID only said that the 2TA operation is based on M-TRP M-DCI mode, it should be sensible to design 2TA operation which works well for both R17 unified TCI framework and R15/16 UL beam indication via spatial relation. Therefore, the following is proposed.

Proposal 3: 2TA operation is designed in a way applicable for both R17 unified TCI framework and R15/16 UL beam indication via spatial relation.

Under 2TA operation framework, UE is supposed to maintain two TA values at least for a serving cell at the same time. One critical issue is how UE understands which TA value to apply when transmitting an UL channel or RS. In last RAN1 meeting, three main alternatives were put on the table as shown in the followings:

- Alt 1: association with TA value is based on CORESETPoolIndex;

- Alt 2: association with TA value is based on Joint/UL TCI or spatial relation;

- Alt 3: association with TA value is based on grouping of UL channel or RS.

Regarding Alt 1, it can be suitable for UL channel/RS scheduled dynamically. That is, which TA value to apply for the scheduled UL channel/RS depends on the CORESETPoolIndex configured for the scheduling CORESET. Before that, network may need to configure or indicate the association between TA value and CORESETPoolIndex firstly. However, preconfigured UL channel or RS is not configured with CORESETPoolIndex in legacy. For example, configured UL grant, PUCCH reporting periodic CSI or periodic SRS. Therefore, network is supposed to configure CORESETPoolIndex to those preconfigured UL channel or RS. Nonetheless, Alt 1 may have downside on some M-TRP schemes like UL repetitions and STxMP, in which more than one PUSCHs are scheduled by a CORESET as agreed to be supported 2TA operation in last RAN1 meeting.

On the other hand, Alt 2 may provide a more flexible framework. It can effortlessly tackle out TA association for UL repetitions and STxMP. Which TA value to apply for a PUSCH transmission occasion can be determined based on its indicated UL beam. The effort to achieve the goal is how to associate TA value with UL beam. Here, we should consider both unified TCI and spatial relation. When considering spatial relation, both PUCCH spatial relation and SRS spatial relation should be taken cared.

On Alt 3, it seems the grouping is natural to PUCCH and SRS. However, the drawback would be how to group at least dynamically scheduled PUSCH. For configured UL grant, the grouping may also need additional signals or rule to make that happen.

In overall, based on above analysis, we suggesting adopting Alt 2. So, we propose the following proposal.

Proposal 4: For association of TA values with UL channels/RSs, a TA value is associated or included in a Rel-17 joint/UL TCI or a Rel-15/16 spatial relation.

In the beginning of configuring M-TRP M-DCI mode and enabling 2TA feature, a RA procedure should be triggered to acquire the additional TA value. The RA procedure can be a PDCCH order. In addition, when a time alignment timer is expired, the associated TA value is outdated, where the associated TA value may be for the first TRP or the second TRP. In these cases, a per-TRP RA procedure should be introduced. We should also consider both CBRA and CFRA case, since under some cases, counting result of time alignment timers between network and UE may not be the same, e.g., MAC-CE for TA command is missed.

Proposal 5: Support introducing a per-TRP RA procedure for acquiring TA value, where the per-TRP RA procedure supports both CBRA and CFRA procedure.

- Conclusion

According to the above discussion(s), we have the following proposal(s).

Proposal 1: For multi-DCI based multi-TRP operation, two TAGs corresponding to two TA values separately are configured for a serving cell.

Proposal 2: For multi-DCI based multi-TRP operation, two time alignment timers corresponding to two TA values separately are configured for a serving cell.

Proposal 3: 2TA operation is designed in a way applicable for both R17 unified TCI framework and R15/16 UL beam indication via spatial relation.

Proposal 4: For association of TA values with UL channels/RSs, a TA value is associated or included in a Rel-17 joint/UL TCI or a Rel-15/16 spatial relation.

Proposal 5: Support introducing a per-TRP RA procedure for acquiring TA value, where the per-TRP RA procedure supports both CBRA and CFRA procedure.

# Reference

- RP-213598 New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator)

- Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0 (Online meeting, 9th – 20th May 2022)