3GPP TSG-RAN WG1 Meeting #110	R1-2206489

# Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PRS RSTD and PRS RSRPP reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | How a UE determines the paths associated with RSTD and RSRPP is left up to UE implementation. At the moment if the UE is configured to report multiple paths for RSTD and for RSRPP measurements there is nothing to say the UE can’t select different paths for the different measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Clause 5.1.6.5, add some UE behavior when UE is configured with both PRS RSTD and PRS RSRPP measurement reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behvaior when reporting PRS RSTD and PRS RSRPP measurements. LMF can’t guarantee it is not making incorrect assumptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5, 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.6.5	PRS reception procedure

<omitted text>

The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID.

If the UE is configured to report both PRS RSTD and PRS RSRPP measurements, the same detected paths for both measurements should be used in the reporting. The UE may indicate via higher layer parameter [xxxx] if the same detected paths are not used for the PRS RSTD and PRS RSRPP measurements.

<omitted text>