3GPP TSG RAN WG1#110	           R1-2206503

Toulouse, France, 22nd – 26th August 2022

Agenda item:	9.12.2

Source: 	Lenovo

Title: 	On NTN NW verified UE location

Document for:	Discussion

# Introduction

The Rel-18 NR NTN SID was agreed upon during the RAN#94-e [1] meeting and revised during the RAN#95 meeting, where one of the objectives included the study of solutions for the network verified UE location in NTN networks as indicated by the following:

Network verified UE Location: Pending on the conclusion of the RAN SI FS_NR_NTN_netw_verif_UE_loc study item, study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2,RAN1,RAN3]. RAN is expected to determine by RAN#98 whether the study has identified any need for Network verified UE location specification support in Rel-18.

Further to the above, RAN completed a use case and requirement study [TR 38.882, 2] where the key recommendation of the study includes the following:

TR 38.882 Recommendation (Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR (Release 18): In this study, we have identified the need to define a network-based solution which aims at verifying the reported UE location information. The verification should be performed independently from the location information reported by UE. The UE location information for the study is considered verified if the reported UE location is consistent with the network based assessment to within 5-10 km (similar to terrestrial network macro cell size), enabling country discrimination and selection of an appropriate core network in order to support all the regulatory services (i.e. emergency call, lawful intercept, public warning, charging/billing). The solution should not impact significantly the latency of the targeted services nor infringe privacy requirements that apply to the UE location. The study in [RAN2,RAN1,RAN3], which will study and evaluate solutions for the network to verify UE reported location information, shall consider the following aspects: - The scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority. - Multiple satellite (or HAPS) in view by the UE may be considered if time allows - Assume that the UE is attached to a network (so that its context has been set up in the network) for the purpose of positioning - Different solutions or positioning methods for NGSO, GSO or HAPS are not precluded - When considering solutions based on positioning methods, existing 3GPP defined RAT dependent positioning methods shall be considered as baseline. Other methods are not precluded. - Solutions using existing NG-RAN architecture and procedures shall be considered

This contribution provides a discussion into the different aspects for consideration in relation to network verified UE location in NTN deployments.

# Network verified UE Location Accuracy Requirements

Reliable UE location is an important functionality for NTN and its applicable services including Public Warning System, LI, Emergency Services, Charging and Tariff Notifications, which had already been studied and documented in TR 23.737.  Furthermore, it has been established that reliance on the UE reported GNSS location information is insufficient due to the reliability of such reported location information, which may be prone to tampering and/or spoofing.

Due to the inherently large cell sizes in NTN, the existing terrestrial mechanisms to obtain UE location information via Cell ID information is not sufficient. A terrestrial cell size may lie in the range of between 5-10 km and it is expected that the network verification accuracy requirement may have to at least meet this requirement. One of the requirements for operators for non-terrestrial networks, is that the network must reliably know the location information of a UE attached to the network in order to select the appropriate core network functions and successfully register the UE. Once the appropriate core network has been selected for a UE, it is possible to support some services subject to national regulations or other operational constraints. The conclusions of TR 38.882 have identified the need to define a network-based solution which aims at verifying the reported UE location information. Moreover, it specifies that the verification should be performed independently from the location information reported by UE. The criterion for verification is set as follows:

The UE location information for the study is considered verified if the reported UE location is consistent with the network based assessment to within 5-10 km (similar to terrestrial network macro cell size), enabling country discrimination and selection of an appropriate core network in order to support all the regulatory services (i.e. emergency call, lawful intercept, public warning, charging/billing).

However, emergency services may require stringent accuracy requirements as stipulated by regulatory agencies such as FCC, e.g., for E911 calls a 50 m horizontal accuracy or provide a dispatchable location for 70 percent of all wireless 911 calls is specified and the European Commission, e.g., for E112 calls, a horizontal position error of maximum 5 m in open sky conditions and maximum of 25 m in urban canyon conditions with a confidence level of 95 %.

Proposal 1: RAN1 to confirm that the network verification accuracy requirement is at least in the range between 5-10 km for NTN. FFS whether additional requirements need to be defined for other services, e.g., emergency services.

# Overview of TN NR Positioning Methods

## RAT-dependent Positioning Techniques

There are various RAT dependent positioning schemes discussed in NR TN network, such as DL TDOA, UL TDOA, DLAOD, ULAOA, Multi-RTT, ECID. They are based on timing difference, angle, RTT or Cell ID to determine the UE position at gNB/LMF side or at UE side.

- DL TDOA: based scheme is a RAT-dependent positioning scheme based on DL signal transmitted from multiple TRPs. The UE measures the PRS reception timing difference (RSTD) among multiple TRPs with respect to a reference TRP and report it to gNB/LMF. In order to enable such a positioning technique, tight synchronization is required between TRPs/gNBs. The positioning result is determined using the reference signal timing difference. with respect to at least 3 anchor nodes, i.e. 3 TRPs, i.e. 2 pairs of RSTD measurements (including 1 reference TRP), the UE position can be determined.

- UL-TDOA: based positioning technique is similar to DL TDOA. The difference is that UE will transmit uplink SRS for positioning to multiple TRPs simultaneously, and the signal reception timing difference is measured by the TRPs (also referred to as UL-RTOA measurements). The uplink SRS transmission for multiple TRPs is configured by gNB and activated/deactivated by the LMF.

- DL-AOD: based positioning technique is an angle-based scheme. The gNB shares antenna information with the LMF and based on DL PRS beam transmission, the UE measures the PRS RSRP for each transmission beam. The UE reports the corresponding RSRP to the LMF, where the LMF maps the RSRP and beam antenna information to determine the Angle of Departure, which is then used to determine UE’s range. Using multiple DL-AoDs, the LMF/UE may use triangulation to determine the UE’s position estimate.

- UL-AOA: based scheme is also an angle-based scheme. The difference form DL-AoD is that it is based on an UL SRS for positioning transmission from the UE side while the gNB/TRP measures the UL-AoA (including azimuth and zenith angles). Similarly multiple AoAs can be used to determine the UE’s position estimate.

- Multi-RTT based scheme is a time-based solution. the solution is to utilize RTT between gNB and UE to calculate the propagation delay between gNB and UE, and then to determine the UE position. There will be a DL signal transmission from gNB to UE, and upon reception of the DL signal, there will be a UL signal transmission from UE to gNB. DL transmission timing (UE Rx-Tx time difference measurement) and UL reception timing (gNB Rx-Tx time difference measurement) at gNB side will be recorded. Timing difference between DL signal reception and UL signal transmission can be configured by gNB or determined by UE and reported to the LMF. Based on the information, RTT between gNB and UE can be determined, and then the propagation delay between gNB and UE can be determined. Based on at least 3 TPRs at gNB side, the UE position can be determined.

- NR ECID: based scheme is to determine the rough/coarse UE position by the LMF using UE reported SSB/CSI-RS RSRP measurements and corresponding cell ID. The cell id with largest RSRP can be determined to be the identified cell id, and UE position is determined to be within the cell coverage of the corresponding cell.

There are some enhanced schemes such as measurement and reporting on a per branch or per path level to further increase the positioning accuracy in NR R17, mainly for IIoT scenarios. In existing GNSS system, several positioning methods are adopted. Multi-RTT is adopted for GEO satellite for simplicity with coarse positioning accuracy. DL TDOA and UL TDOA based scheme are adopted for LEO with more satellites to further reduce the positioning latency and increase positioning accuracy. Furthermore, carrier phase based measurement/reporting is introduced to GNSS to improve the positioning accuracy. However, adaptability/feasibility of these techniques in 3GPP NTN systems need to be further evaluated and studied.

Proposal 2: RAN1 should further study the feasibility and applicability of the current RAT-dependent positioning techniques in the context of NTN.

## RS type, Measurement for Positioning Techniques

Tables 1 and 2 shows a summarized mapping among RS type, positioning measurement and positioning technique already specified for TN networks.

Table 1: UE Measurements to enable RAT-dependent positioning techniques

| DL/UL Reference Signals | UE Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| DL PRS | DL RSTD | DL-TDOA |
| DL PRS | DL PRS RSRP | DL-TDOA, DL-AoD, Multi-RTT |
| DL PRS and SRS for positioning | UE Rx-Tx time difference | Multi-RTT |
| SSB / CSI-RS for RRM | SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM), SS-RSRPB (for RRM) | NR E-CID |

Table 2: gNB Measurements to enable RAT-dependent positioning techniques

| DL/UL Reference Signals | gNB Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| SRS for positioning | UL RTOA | UL-TDOA |
| SRS for positioning | UL SRS-RSRP | UL-TDOA, UL-AoA, Multi-RTT |
| SRS for positioning and DL PRS | gNB Rx-Tx time difference | Multi-RTT |
| SRS for positioning, | AoA and ZoA | UL-AoA, Multi-RTT |

## Positioning Reference Signal Configurations

Currently for DL-based positioning techniques (e.g., DL-TDOA, DL-AOD), DL PRS has been introduced for positioning while in the case of UL-based positioning techniques (e.g., UL-TDOA, UL-AoA), existing SRS has been enhanced with a new purpose of “positioning”.

DL-PRS is configured on a hierarchical level in terms of resource sets and resources. Furthermore, configurable bandwidths of up to 100 MHz (in FR1) and 400 MHz (FR2) may be configured, which has a direct dependency on the positioning accuracy performance of both timing-based and angle-based schemes. DL-PRS is also designed to optimize hearability from serving as well as neighbouring TRPs/gNBs, with features such as staggered comb pattern design, power boosting, muting to reduce interference, repetitions to increase reliability and positioning accuracy

In the context of NTN, further study is needed on whether the existing PRS or SRS for positioning may be enhanced to compensate for extended propagation delays as well as mobile anchors (e.g., LEO satellites).

Proposal 3: RAN1 to further study enhancements (if needed) to both PRS/SRS for positioning configuration design for NTN RAT-dependent positioning techniques.

## RAT-independent Positioning Techniques

According to TR 38.882 [2], Relying only on the GNSS based location information reported by the UE is not considered reliable by SA3-LI [3]. The UE reported location information (for example determined with its GNSS receiver), could be erroneous due to intentional (e.g. maliciously tampering by user or by 3rd party) or unintentional (e.g. interference) causes, hence it cannot be considered trusted by network operators. The TR 38.882 further recommends that:

“It is expected that solutions combining both UE reported GNSS information and network based information for verification of UE location can improve the reliability of core network selection in non-terrestrial networks.”

Proposal 4: RAN1 to clarify if hybrid positioning methods (RAT dependent and RAT independent) are under the scope of study.

## Network verifying UE reported position for NTN

It is recommended that to use existing RAT dependent positioning methods for network to verify UE reported position in NTN. The RAT dependent positioning methods include at least: DL TDOA, UL TDOA, DL AOD, UL AOA, Multi-RTT and ECID based scheme. However, there are some differences between TN network and NTN network. Some NTN specific characteristics should be taken into account when designing the method for network to verify UE reported position in NTN.

### Transparent payload for NTN

In an NTN network, the gNB and gateway is located on the ground, while the satellite may be at different altitudes depending on the type of satellite. Signal is generated by gNB or UE, and forwarded by satellite, and then the signal is received at UE or gNB, respectively. Additionally, the satellite is constantly moving on an orbit. These are different features when compared to that of a TN network. In a TN network, a stationary gNB/TRP generates/receive the signal to/from a UE without relaying or forwarding operation. To determine the UE position in an NTN network, using DL/UL-TDOA based solution, the propagation delay difference between satellite(s)(multiple satellites or single satellite at multiple time instances) and UE needs further study. Due to satellite movement, the satellite position and the corresponding time instance should be determined based on the signal transmission/reception time instance at gNB side and the satellite orbit. So that the propagation delay difference between gNB and satellite can be determined and then the propagation delay difference between satellite and UE.

Furthermore, DL/UL-TDOA schemes may require multi-satellite coverage to make use of at least 3 anchors.  The satellite movement presents an additional challenge to this technique. As the satellite is moving, fine timing control is required. Additionally, tight synchronization is also required between multiple satellites. For multi-RTT based solution, the satellite position for the DL signal forwarding and the UL signal forwarding may be different due to satellite moving, so it is difficult to calculate the propagation delay between satellite and UE. As a result, Multi-RTT based scheme may need further study.

Proposal 5: RAN1 to further study DL-TDoA/UL-TDoA and Multi-RTT timing-based positioning techniques and associated adaptations for NTN to verify UE reported location

Proposal 6: For NTN, UE position is determined based on the propagation delay differences between satellite(s) and UE.

Proposal 7: For NTN, satellite positions for different time instances are useful to determine the propagation delay difference between satellite and UE.

Proposal 8: Further study application of Multi-RTT based solution to different scenarios including GEO, LEO, HAPS.

### Large coverage range for NTN

Compared with TN network, a NTN cell/beam will cover a large geographical area. Some of the positioning schemes may not applicable for NTN network. In TN network, the DL and UL angle based scheme utilize the measurements for each beam to determine an angle range to determine the UE position. the range of a beam is relatively small especially considering communication on mm wave. However, in NTN network, the angle range/geographical range of a beam is quite large. Considering the positioning accuracy of network verifying UE reported position is comparable to a TN network coverage, angle based scheme may be challenging for an NTN scenario for network to verify the UE reported location. Similarly, the NR ECID needs to be adapted to an NTN scenario taking into account large cell sizes.

Proposal 9: RAN1 to further study DL/UL angle-based and NR-ECID positioning techniques and associated adaptations for NTN to verify the UE reported location.

### Verifying based on single satellite at different time instances

In TN network, for each RAT dependent positioning methods, at least 3 TRPs are used to determine the UE position. The at least 3 TRPs are designed to transmit or receive signal simultaneously (at least within CP range). So inter-TRP synchronization should be guaranteed. With timely measurement and reporting, positioning latency can be guaranteed. In NTN network, due to satellite moving, it is difficult to see at least 3 TRPs simultaneously. And it is recommended that the scenario by single satellite at multiple time instances has higher priority than the scenario with multiple satellites.

For the scenario with single satellite and multiple time instances, the satellite positions will be different for different time instances, so the difference of propagation delay between satellite and gNB should be firstly calculated, then the propagation delay difference between satellite and UE can be determined. Additionally, there may be some other issues for single satellite and multiple time instances, such as the SSB transmission timing may change, or the common TA indicated by network and UE-specific TA calculated by UE itself may also change for different time instances. Schemes for network to verify UE reported location should consider the characteristics of single satellite and multiple time instances.

Proposal 10: Characteristics for single satellite and multiple time instances should be taken into account when designing schemes for the network to verify UE reported location.

## Network Entity performing UE network verification

According to TS 23.273, there are mechanisms in the core network to verify the UE’s location based on a network-induced location request. These involve coordination between the AMF and LMF, depending on which entity performs the actual verification. The LMF is then responsible for triggering the appropriate RAT-dependent procedures to independently verify the UE’s position. However, the decision on which network entity performs the UE location verification requires further input from RAN2 and SA2 WGs. This is largely dependent on aspects related to the LCS and NTN architectures.

Proposal 11: The network entity performing the UE location verification may be up to RAN2 and SA2 decision depending on the type of location service request.

# Conclusion

The corresponding proposals are summarized as follows:

Proposal 1: RAN1 to confirm that the network verification accuracy requirement is at least in the range between 5-10 km for NTN. FFS whether additional requirements need to be defined for other services, e.g., emergency services.

Proposal 2: RAN1 should further study the feasibility and applicability of the current RAT-dependent positioning techniques in the context of NTN.

Proposal 3: RAN1 to further study enhancements (if needed) to both PRS/SRS for positioning configuration design for NTN RAT-dependent positioning techniques.

Proposal 4: RAN1 to clarify if hybrid positioning methods (RAT dependent and RAT independent) are under the scope of study.

Proposal 5: RAN1 to further study DL-TDoA/UL-TDoA and Multi-RTT timing-based positioning techniques and associated adaptations for NTN to verify UE reported location

Proposal 6: For an NTN, UE position is determined based on the propagation delay differences between satellite(s) and UE.

Proposal 7: For NTN, satellite positions for different time instances are useful to determine the propagation delay difference between satellite and UE.

Proposal 8: Further study application of Multi-RTT based solution to different scenarios including GEO, LEO, HAPS.

Proposal 9: RAN1 to further study DL/UL angle-based and NR-ECID positioning techniques and associated adaptations for NTN to verify the UE reported location.

Proposal 10: Characteristics for single satellite and multiple time instances should be taken into account when designing schemes for the network to verify UE reported location.

Proposal 11: The network entity performing the UE location verification may be up to RAN2 and SA2 decision depending on the type of location service request.

# References

- RP-221819, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, RAN#96, June. 2021.

- 3GPP TR 38.882, “Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR”, V18.0.0, June 2022.

- S3-214349, “Reply LS on NTN specific User Consent”, SA3#105-e, Nov. 2021.