3GPP TSG RAN WG1 #110																		       R1-2206529

Toulouse, France, August 22nd – 26th, 2022

Source: 	Intel Corporation

Title:	Discussion on reply LS for SDT common search space

Agenda item:	5

Document for:	Discussion and Decision

# Introduction

RAN1 received a reply LS from RAN2, which includes the following questions [1]:

To RAN1 ACTIONS: - RAN2 respectfully asks RAN1 to take the above information into account for their specification work and provide feedback, if any, on the above agreements - RAN2 respectfully asks RAN1 to confirm whether the configuration of SDT common search space as a choice between an existing search space or a new search space as noted above is aligned with RAN1 understanding.

In the contribution, we present our views on SDT common search space.

# Discussion on response to RAN2 LS

The following information was included in the RAN2 reply LS [1]:

For SDT common search space For SDT common search space RAN2 discussed signalling of an existing search space ID. RAN2 has implemented the signalling using a choice structure between configuration of a SearchSpace and SearchSpaceId. Thus network can either configure an existing search space (using searchSpaceId) or configure a new search space (using searchSpace). RAN2 requests RAN1 to kindly confirm whether this is aligned with RAN1 understanding.

As defined in the [2], a UE can be provided a CSS set by sdt-SearchSpace to monitor during CG-SDT and RA-SDT operation. Further, for RA-SDT, if sdt-SearchSpace is not configured, type-1 PDCCH CSS can be reused. In our view, from RAN1 perspective, configuration of sdt-SearchSpace as a choice between an existing search space or a new search space can be supported.

Proposal 1

- From RAN1 perspective, configuration of sdt-SearchSpace as a choice between an existing search space or a new search space can be supported.

# Conclusions

In this contribution, we presented our views on SDT common search space. Further, we summarize the proposals as follows:

Proposal 1

- From RAN1 perspective, configuration of sdt-SearchSpace as a choice between an existing search space or a new search space can be supported.

# References

- R1-2205736, “Reply LS on the L1 related agreements for SDT”, August 2022

- TS38.213, V17.2.0, “NR: Physical layer procedures for control”