3GPP TSG RAN WG1 #110			     R1-2206540

Toulouse, France, August 22nd – August 26th, 2022

Source:	Intel Corporation

Title:	Discussion on Applicability of the Short Control Signalling Exemption

Agenda item:	8.2

Document for:	Discussion and Decision

# Introduction

In this contribution, in the context of channel access to extend NR up to 71 GHz and based on the agreements and conclusions made during the previous RAN1 meetings along this WI, we discuss the applicability of the short control signaling exemption allowed by the ETSI BRAN in EN 302 567 v2.2.0 [1] for frequency ranges from 52.6 GHz up to 71 GHz, where the change requests are provided in [3].

# Details Related to the Short Signaling Exemption

Short signal exemption (SSE) is a mechanism to send relatively short signaling transmissions (compared to regular data transmissions) without sensing the channel for presence of other signals. For frequency ranges from 52.6 GHz up to 71 GHz, SSE is described in the most recent draft of EN 302 567 v2.2.0 [1] with application to short management and control frames. The total duration of the short control signaling transmissions is required to be constrained to less than 10 msec within 100 msec observation period per device.

In this context, for UL transmissions RAN1 has agreed to apply this exemption to msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS:

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc Conclusion In Rel.17, there is no consensus to apply contention exemption short control signalling to UL transmissions other than msg1 and msgA.

However, while this exemption is supported for the aforementioned signals and can be applied in CEPT/EU regions, this may not be used in other regions even when LBT may be used as an adaptivity method. As an example, while in Japan ARIB [2] mandates the use of LBT, this hasn’t defined the concept of SSE. Therefore, this relaxation cannot be applied in the related regions. Since it may not be possible for a UE to implicitly know by implementation whether it may be deployed in a region where the SSE is allowed or not, it may be necessary to introduce a one-bit RRC parameter to allow the network to indicate to a UE whether the SSE should be applied or not.

Proposal 1: The gNB indicates through a 1 bits RRC parameter in SIB1 whether the short signal exemption (SSE) should be applied or not.

- Endorse the draft CR for TS37.213 in R1-2206541.

The following provides text proposal changes required to enable short control signal LBT exemption via RRC configuration.

TS37.213

### 4.4.5 Exempted transmissions from sensing In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, if the higher layer parameter ShortControlSignalling-r17 is configured a gNB/UE may transmit the following transmission(s) without sensing the channel: - Transmission(s) of the discovery burst by the gNB; - Transmission(s) of the first message in a random access procedure by the UE. When the gNB/UE transmits the above transmission(s) without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB/UE shall not occupy the corresponding channel more than 10ms over any 100ms interval. <Unchanged parts omitted>

# Conclusions

In this contribution, we derived the following proposal:

Proposal 1: The gNB indicates through a 1 bits RRC parameter in SIB1 whether the short signal exemption (SSE) should be applied or not.

- Endorse the draft CR for TS37.213 in R1-2206541.

# References

- Draft ETSI EN 302 567 V2.2.0, “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum,” Dec. 2020.

- ARIB STD-T117, “Low Power Data Communication System / 60 GHz-band Wireless LAN for Very High Throughput Data Communications”, September 2016.

- R1-2206541, “[draft CR] Correction on short control signaling LBT exemption applicability”, Intel Corporation, August 2022.