3GPP TSG RAN WG1 #110			     R1-2206563

Toulouse, France, August 22nd – August 26th, 2022

Source:	Intel Corporation

Title:	Discussion on LS on IUC with Non-Preferred Resource Set

Agenda item:	8.11

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss on the potential reply to LS to the LS from RAN2 [1] regarding the UE’s behavior in case of inter-UE coordination scenario with non-preferred resource set.

# Discussion

In [1] RAN2 has provided an LS to RAN1 where RAN2 is inquiring several questions regarding UE’s behavior in case of inter-UE coordination scenario with non-preferred resource set, and in the following we have provided our view to each of them:

Q1:	Is the scenario described above a valid scenario or not?

A1: 	The scenario is valid, but this could be considered a very corner case. In fact, this implies that a UE has the capability of receiving information in the resource pool but is configured to perform random resource selection in the resource pool for the transmission at hand. Therefore, in our opinion this should only be the case for high priority traffic with strict latency requirements.

Observation 1:

- The scenario of inter-UE coordination scenario with non-preferred resource set is a valid scenario, but could be considered as a corner case.

Q2:	If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not?

A2: 	It is our understanding that the scenario described above may only occur when the UE B received the non-preferred set for another transmission in the same resource pool while UE B is configured by higher layers to use random resource selection for the current transmission. In this case, the non-preferred resource set of UE A does not need to be considered for random resource selection.

Proposal 1: The non-preferred resource set of UE A does not need to be considered for random resource selection in the case of inter-UE coordination scenario with non-preferred resource set.

Q3:	If the answer to Q2 is yes, then, in RAN1’s view, which specification (PHY or MAC) should capture the resource exclusion behavior?

A3: 	While we believe that the case of non-preferred resource set of UE A does not need to be considered, if RAN1 may agree otherwise we think that as random resource selection is defined in the MAC specification, the exclusion behavior should be defined in the MAC specification.

# Conclusions

In this contribution, we derived the following proposal:

Observation 1:

- The scenario of inter-UE coordination scenario with non-preferred resource set is a valid one but could be considered as a corner case.

Proposal 1: The non-preferred resource set of UE A does not need to be considered for random resource selection in the case of inter-UE coordination scenario with non-preferred resource set.

# References

- R1-2205728, “LS on IUC with non-preferred resource set” RAN2