3GPP TSG RAN WG1 #110		     R1-2206740

Toulouse, France, August 22nd – 26th, 2022

Source:	vivo

Title:	Discussion on HP PUCCH with HARQ-ACK and SR colliding with LP HARQ-ACK

Agenda Item:	8.3

Document for:	Discussion and Decision

- Introduction

In RAN1#109 e-meeting, HP PUCCH with HARQ-ACK and SR colliding with LP HARQ-ACK was discussed and no agreements were achieved [1]:

In this contribution, we share our view on HP PUCCH with HARQ-ACK and SR colliding with LP HARQ-ACK.

- PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlapping with a PUCCH carrying LP HARQ-ACK

The issue on HP SR and HP HARQ-ACK with PF0/1 overlapping with LP HARQ-ACK has been discussed in several meetings. In the last RAN1 meeting, the following was proposed.

Proposal: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK, for HP SR with F1 and HP HARQ-ACK with F0/F1, and HP SR with F0 and HP HARQ-ACK with F0 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs , K>=1 - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped when colliding with a PUCCH with HP HARQ-ACK and positive SR. UE behaviour is the same as a PUCCH with LP HARQ-ACK colliding with a PUCCH with HP HARQ-ACK without SR. - Option 2: - [If the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List,] 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. - [If HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped.]

It is a typical case for URLLC service using PUCCH format 0/1 carrying SR and HARQ-ACK. If option 0 is applied, LP HARQ-ACK would be dropped and LP service performance would be degraded especially for LP HARQ-ACK with the larger payload size.

For option 0, the potential ambiguity may occur between gNB and UE. For example, when 1 bit HP HARQ-ACK and HP SR with PUCCH format 0 overlaps with 1 bit LP HARQ-ACK and all SRs is negative, UE would treat the case as 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK. The 2 bits HARQ-ACKs (HP and LP) will use CS 0/3/6/9. If a SR is positive and LP HARQ-ACK is dropped, 1bit HP HARQ-ACK and 1bit HP SR would be transmitted based on HP PUCCH format 0 with CS 3/9. The ambiguity occurs for CS 3/9.

According to existing agreement when a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK, LP HARQ-ACK is multiplexed with HP UCI. It is reasonable to support multiplexing for PUCCH format 0/1 case from spec integrity perspective. Otherwise, the multiplexing LP HARQ-ACK would be only allowed for partial cases.

Since Option 2 can guarantee the transmission of HP SR, it is slightly preferred.

Proposal 1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK, for HP SR with F1 and HP HARQ-ACK with F0/F1, and HP SR with F0 and HP HARQ-ACK with F0 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, K>=1

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - Option 2:

    - If the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is

    - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - If HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped.

- Conclusion

In this contribution, we discuss HP PUCCH with HARQ-ACK and SR colliding with LP HARQ-ACK, and the following proposal is made.

Proposal 1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK, for HP SR with F1 and HP HARQ-ACK with F0/F1, and HP SR with F0 and HP HARQ-ACK with F0 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, K>=1

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - Option 2:

    - If the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is

    - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - If HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped.

References

- Chairman notes RAN1#109 e-meeting.