3GPP TSG RAN WG1 #110		    R1-2206741

Toulouse, France, August 22nd – 26th, 2022

Source:	vivo

Title:	Discussion on multiplexing for HP SPS HARQ-ACK

Agenda Item:	8.3

Document for:	Discussion and Decision

- Introduction

In RAN1#109 e-meeting, multiplexing for HP SPS HARQ-ACK was discussed. However, no agreements were achieved [1].

In this contribution, we share our view on multiplexing for HP SPS HARQ-ACK.

- Intra-UE multiplexing for UCI enhancements

  - Multiplexing for HP SPS HARQ-ACK

LP HARQ-ACK multiplexing with HP SPS HARQ-ACK provided by n1PUCCH-AN was discussed in the last meeting. In NR Rel-16, the support of multiple SPS configurations and related HARQ-ACK enhancements for SPS configurations are optionally based on UE capability, i.e. FG12-2. Supporting multiplexing UCI with different priorities and support of multiple SPS configurations is obvious independent UE features.

The main issue for this case is that the total HARQ-ACK payload with both HP and LP HARQ-ACK may not be accommodated by the capacity of the HP PUCCH resource determined by n1PUCCH-AN which can only carry up to 2 bits. A straightforward way is to drop LP HARQ-ACK under such collision.

For LP DG PDSCH cases, it is discussed whether gNB scheduling should avoid the case considering PUCCH with LP HARQ-ACK for DG PDSCH overlapping with HP PUCCH for SPS PDSCH only would be always dropped. Considering different numerologies case, for URLLC service with a stringent latency requirement, a SPS configuration with a higher numerology and 1 slot periodicity can be configured. For LP DG PDSCH with a lower numerology configuration, the PUCCH overlapping with HP SPS HARQ-ACK may frequently occur. We slightly prefer a unified solution to handle both PUCCH for LP SPS PDSCH and LP DG PDSCH overlapping with HP SPS HARQ-ACK. The following proposal is suggested.

Proposal: For resolving collision of two overlapping PUCCHs with different priorities in R17, if a HP PUCCH with SPS PDSCH HARQ-ACK only overlaps with a LP PUCCH with HARQ-ACK and sps-PUCCH-AN-List is not provided in the second PUCCH-config, the LP PUCCH is dropped.

- Conclusion

In this contribution, we discuss multiplexing for HP SPS HARQ-ACK, and the following proposal is made.

Proposal: For resolving collision of two overlapping PUCCHs with different priorities in R17, if a HP PUCCH with SPS PDSCH HARQ-ACK only overlaps with a LP PUCCH with HARQ-ACK and sps-PUCCH-AN-List is not provided in the second PUCCH-config, the LP PUCCH is dropped.

References

- Chairman notes RAN1#109 e-meeting.