3GPP TSG RAN WG1 #110	        		   R1-2206785

# Toulouse, France, August 22th – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | Draft | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on HARQ-ACK feedback for beam indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Ambiguity in the gNB, whether or not the UE received the “transmission configuration indication” to update the quasi-co-location/spatial filters based on the indicated TCI state. The TCI state conveyed by the “transmission configuration indication” is applied after HARQ-ACK is received. This ensures beam alignment between the gNB and the UE. The HARQ-ACK can be part of a HARQ-ACK codebook. The HARQ-ACK can be one of the following: - A positive HARQ-ACK if the PDCCH is received and the corresponding PDSCH is successfully decoded. - A negative HARQ-ACK if the PDCCH is received but the corresponding PDSCH is not successfully decoded. - A DTX (no HARQ-ACK) if the PDCCH is not received (i.e., missed). If the PDCCH is received, the “transmission configuration indication” field would have also been received, and hence it seems natural to update the TCI state based on a positive or negative HARQ-ACK feedback. However, in some scenarios for HARQ-ACK codebook, the gNB can’t distinguish a negative HARQ-ACK feedback from a DTX. In those cases, it would see natural to update the TCI state based on a positive HARQ-ACK feedback only. One example, when a gNB can distinguish a negative HARQ-ACK feedback from DTX is when the HARQ-ACK feedback provides feedback for one DL transmission only. In case of DTX, there is no PUCCH transmission, in case of positive or negative HARQ-ACK different signals are transmitted that can be distinguished by the gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the gNB can distibguish negative HARQ-ACK feedback and DTX, the gNB applies the TCI state after receiveing a corresponding HARQ-ACK feedback with postive HARQ-ACK feedback or negative HARQ-ACK feedback. If the gNB can’t distibguish negative HARQ-ACK feedback and DTX, the gNB applies the TCI state after receiveing a corresponding HARQ-ACK feedback with postive HARQ-ACK feedback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the gNB, whether or not the UE received the “transmission configuration indication” to update the quasi-co-location/spatial filters based on the indicated TCI state. This can lead to misalignment between the gNB beam and the UE beam. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

## 5.1.5	Antenna ports quasi co-location

## *** Unchanged text is omitted ***

When the UE would transmit the last symbol of a PUCCH with positive or negative HARQ-ACK information or a PUSCH with positive or negative HARQ-ACK information when DTX and negative HARQ-ACK are distinct, or a PUCCH with positive HARQ-ACK information or a PUSCH with positive HARQ-ACK information, when DTX and negative HARQ-ACK are not distinct, corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduling by the DCI carrying the TCI State indication, and if the indicated TCI State is different from the previously indicated one, the indicated DLorJointTCIState or UL-TCIstate should be applied starting from the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH or the PUSCH. The first slot and the BeamAppTime_r17 symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication.