3GPP TSG RAN WG1 #110	R1-2206789

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	8.2

Source:	Samsung

Title:	Discussion for cell-defining SSB indication using non-cell-defining SSB in FR2-2

Document for:	Discussion and Decision

# Introduction

This contribution explains the technical issue associated with the draft CR R1-2206790, regarding the issue of cell-defining SSB indication using non-cell-defining SSB in FR2-2.

# CD-SSB indication in FR2-2

In Rel-15, a functionality of indicating cell-defining SS/PBCH block (e.g. SS/PBCH block with associated RMSI) was supported, by using a non-cell-defining SS/PBCH block on the sync raster. For FR2, the indication range is -256 to 256 GSCN values, subject to the number of codepoints available from kSSB, as described in Table 13-17 of TS 38.213 (quoted below for information). Such range for indication is sufficient for Rel-15 since the maximum number of GSCNs per band is restricted to 230 for FR2 (refer to Table 1 for information).

Table 13-17: Mapping between the combination of kSSB and controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1 to NGSCNOffset for FR2

| kSSB | 16×controlResourceSetZero +searchSpaceZero | NGSCNOffset |
| --- | --- | --- |
| 12 | 0, 1, …, 255 | 1, 2, …, 256 |
| 13 | 0, 1, …, 255 | -1, -2, …, -256 |
| 14 | 0, 1, …, 255 | Reserved, Reserved, …, Reserved |

However, in FR2-2, based on RAN4 agreements on the synchronization raster design for both licensed (e.g. band n264) and unlicensed bands (e.g. band n263), the range of GSCN values has exceed 256 for FR2-2 band (refer to Table 1 for information), so enhancement to the Rel-15 method on the indication of CD-SSB should be considered, and we propose to add a step size on top of the indication on the GSCN offset value, such that the indication range can be enlarged.

Table 1 Synchronization raster entries per band for FR2.

| NR operating band | SS Block SCS | Range of GSCN (First – <Step size> – Last) | Number of GSCNs |
| --- | --- | --- | --- |
| n257 | 120 kHz | 22388 – <1> – 22558 | 171 |
|  | 240 kHz | 22390 – <2> – 22556 | 167 |
| n258 | 120 kHz | 22257 – <1> – 22443 | 187 |
|  | 240 kHz | 22258 – <2> – 22442 | 185 |
| n259 | 120 kHz | 23140 – <1> – 23369 | 230 |
|  | 240 kHz | 23142 – <2> – 23368 | 227 |
| n260 | 120 kHz | 22995 – <1> – 23166 | 172 |
|  | 240 kHz | 22996 – <2> – 23164 | 169 |
| n261 | 120 kHz | 22446 – <1> – 22492 | 47 |
|  | 240 kHz | 22446 – <2> – 22490 | 45 |
| n263 | 120 kHz | 24156 + 6 * N – 3 * floor((N+5)/18), N=0:137 | 802 |
|  | 480 kHz | 24162 + 24 * N – 12 * floor((N+4)/18), N=0:33 | 769 |
|  | 960 kHz | 24162 – <6> – 24954 | 793 |
| n264 | 120 kHz | 24674 <3> 24959 | 286 |
|  | 480 kHz | 24677 <12> 24953 | 277 |
|  | 960 kHz | 24680 <6> 24950 | 271 |

Proposal: For CD-SSB indication using NCD-SSB on synchronization raster, support the indicated GSCN of the indicated CD-SSB as NGSCNReference+NGSCNStep⋅NGSCNOffset, wherein NGSCNStep is the step size.

- Adopt the draft CR R1-2206790.

A same text proposal as the draft CR is repeated in the Appendix for information.

# Conclusion

The proposal made in this contribution is summarized below:

Proposal: For CD-SSB indication using NCD-SSB on synchronization raster, support the indicated GSCN of the indicated CD-SSB as NGSCNReference+NGSCNStep⋅NGSCNOffset, wherein NGSCNStep is the step size.

- Adopt the draft CR R1-2206790.

# Appendix

================================ Start of TP for TS 38.213 ==================================

13 	UE procedure for monitoring Type0-PDCCH CSS sets

=============================== Unchanged Text Omitted ===================================

If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, the UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set as NGSCNReference+NGSCNSize⋅NGSCNOffset. NGSCNReference is the GSCN of the first SS/PBCH block, NGSCNSize=1 for FR1 and FR2-1, and NGSCNSize=3 and 6 for 120 kHz and 480 kHz, respectively, for FR2-2, and NGSCNOffset is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If the UE detects the second SS/PBCH block and the second SS/PBCH block does not provide a CORESET for Type0-PDCCH CSS set, as described in clause 4.1, the UE may ignore the information related to GSCN of SS/PBCH block locations for performing cell search.

=============================== Unchanged Text Omitted ===================================

================================ End of TP for TS 38.213 ==================================