3GPP TSG RAN WG1 #110	    			                      R1-2206835

Toulouse, France, August 22nd – August 26th, 2022

Agenda item:	9.5.2.3

Source:	Samsung

Title:	Discussion on Positioning for RedCap UEs

Document for:	Discussion and Decision

# Introduction

In the SID [1], the following objective is included regarding the support of positioning for RedCap UEs

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In the last RAN1 meeting, some simulation and design aspects have been agreed for redcap Positioning, pls refer to the annex for the details. This contribution continue to discusse aspects related to the support of positioning for RedCap UEs.

# Positioning accuracy requirement

Release-17 has specified support for RedCap UEs with reduced bandwidth support (i.e., 20MHz/100MHz RF BW for FR 1/FR2) and reduced complexity including reduced number of receive chains (i.e., reduced number of antennas at UE). Such UEs could support NR positioning functionality but there could be some positioning performance variation due to the change of UE capability. As indicated by [1], the performance requirements have not been specified for the positioning related measurements performed by RedCap UEs, and no evaluation was performed to see how the reduced capabilities of RedCap UEs might impact eventual position accuracy. Thus one important work for this study item is to check whether there is a problem to support Redcap UE to do RAT-dependent positioning with current specification.

During last meeting, there were some discussions on setting the target KPI for Redcap UE positioning as comparable to Rel17 KPI. However, as the RedCap UE is defined as not a high-end device, it seems not reasonable to set its required positioning accuracy as high as the that for rel-17 normal UE. Thus, we prefer the requirement defined for normal commercial use cases in Rel16 could be used as the requirement for Redcap UE.

-	Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

-	Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

Considering some improvement on the new UEs, one alternative is to raise the requirement for 90% instead of 80%.

Proposal 1: using positioning accuracy requirement for commercial use cases defined in R16 as :

-	Horizontal positioning error < 3m for 90% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 90% of UEs in outdoor deployments scenarios

# Impact of reduced capability

As major simulation assumptions are concluded in last meeting. The simulation based on DL-TDOA is presented below to check the performance under the 20Mhz. The simulation settings are following the agreement parameters as shown in following table.

| Parameter | InF-SH, FR1 | UMi, FR1 |
| --- | --- | --- |
| Carrier frequency | 3.5Ghz | 3.5Ghz |
| Subcarrier spacing | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100Mhz vs 20MHz | 100Mhz vs 20MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) | PRS | PRS |
| Number of sites | 18 | 19*3 |
| Number of symbols used per occasion | 6 symbols | 6 symbols |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series |
| Network synchronization assumptions | ideal | ideal |
| UE/gNB Tx/Rx Calibration Error | ideal | Ideal |
| UE type | Convex UE | All UE |

- For InF-SH,

Fig.1 – the Positioning performance under InF-SH

| BW(Hz) | 50% | 80% | 90% |
| --- | --- | --- | --- |
| 100M | 0.23 | 0.37 | 0.59 |
| 20M | 0.92 | 1.46 | 1.71 |

- For UMi,

Fig.2 – the number of LoS path per UE under UMi.

Fig.3 – the Positioning performance under UMi.

| BW(Hz) | 50% | 80% | 90% |
| --- | --- | --- | --- |
| 100M | 2.8 | 5.9 | 8.5 |
| 20M | 5.8 | 8.9 | 11.9 |

As shown in above figures, we can see:

- For both cases, the positioning error under 20Mhz is larger than that under 100Mhz, which means the positioning performance is degraded when reducing the BW to 20MHz;

- For InF-SH case, the Redcap UE with 20Mhz could achieve <2m accuracy @90%

- For UMi case, the Redcap UE with 20Mhz could achieve <12m accuracy@90%, this could be mainly because the quite few LoS path in the UMi case, e.g., 90% of the UE cannot have <3 LoS link as shown in Fig2;

Observation 1: the RedCap UE with 20MHz has degraded positioning accuracy compared to the case with 100MHz at least for DL-TDOA method;

Observation 2: even with performance degradation, the RedCap UE with 20MHz could still reach the proposed target for indoor case and close to the target for outdoor case.

Potential enhancement consideration

During last meeting, some consideration on using the FH like manner has been proposed. However, for the redcap UE, it cannot jump over the different BW as operating in small sub-BW within a larger BWP, and without switching time gap. As shown in following figure case (a), the different PRS FH part (e.g., FH 1, 2, 3, 4) should be consecutive in time domain. This could be beneficial for combine the different PRS in different OFDM symbols assuming the same/similar channel response within a limited time. Nonetheless, for the case (b) in which the FH part is operating based BWP, e.g., different FH part switching needs to consider the TimeGap for BWP switching. In existing NR, the TimeGap could be as large as in slot level, this is totally not desirable for positioning purpose using the different FH part.

Fig 4 – Illustration of FH based method

Proposal 2: for considering the FH-like adaption to reduced bandwidth, the impact of the TimeGap between different FH part in different BWP should be studied.

# Conclusion

The proposals made in this contribution are summarized below:

Observation 1: the RedCap UE with 20MHz has degraded positioning accuracy compared to the case with 100MHz at least for DL-TDOA method;

Observation 2: even with performance degradation, the RedCap UE with 20MHz could still reach the proposed target for indoor case and close to the target for outdoor case.

Proposal 1: using positioning accuracy requirement for commercial use cases defined in R16 as :

-	Horizontal positioning error < 3m for 90% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 90% of UEs in outdoor deployments scenarios

Proposal 2: for considering the FH-like adaption to reduced bandwidth, the impact of the TimeGap between different FH part in different BWP should be studied.

# Reference

[1] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel

# Annex 1 – RAN1#109e agreement

Agreement

For evaluation of RedCap UE positioning performances, all RAT based positioning methods can be considered. Sources should detail the chosen method(s) when presenting performance evaluations.

Agreement

For evaluation of positioning performance of redcap UEs, adopt the general parameters are detailed in the table below

- TBD parameters are discussed separately

Table 6-1: Common scenario parameters applicable for all scenarios for Redcap UEs evaluations

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz, 700MHz (optional) Note 1 | 28GHz Note 1 |
| Bandwidth, MHz | TBD | TBD |
| Subcarrier spacing, kHz | 30KHz, 15KHz (for 700MHz carriers) | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |  |

Agreement

For the evaluation of RedCap positioning, the following bandwidth can be evaluated:

- FR1: 20MHz baseline, 5MHz optional

- FR2: 100MHz

Agreement

Adopt the following table for the UE model parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| UE model parameters |  |  |
| UE antenna configuration | Panel model 1 – Note 1 dH = 0.5λ, for 1Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) for 2Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) | - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline) - (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration. |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| Number of UE branches | Baseline: 1Rx 1Tx Optional: 2Rx 1 Tx | TBD |
| Note 1: According to 3GPP TR 38.802 |  |  |

R1-2205526	Feature Lead Summary#1 for [109-e-R18-Pos-08] Positioning for RedCap Ues	Moderator (Ericsson)

Agreement

The following scenarios are evaluated for positioning performance of Redcap

- Baseline: (Case 1): Umi street canyon, as described in Table 6.1-1-4 of 38.855

- Optional outdoor:

  - (Case 2): Uma, as described in Table 6.1-1-6 of 38.855

  - (Case 3): Rma (FFS details of the scenario)

- Baseline: (Case 4): InF-SH as described in Table 6.1-1 of 38.857

- Optional indoor: (Case 5) Indoor Open Office, as described in Table 6.1-1-3 of 38.855

- Optional indoor: (Case 6) InF-DH as described in Table 6.1-1 of 38.857

Agreement

The FR2 UE antenna configuration is as follow:

- (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline)

- (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration.

Agreement

The evaluation methodology for RedCap UEs positioning performance uses DL PRS and/or UL SRS for positioning.

- The methodology does not define any baseline reference signal configuration. Sources should detail the chosen configuration of reference signal(s) when presenting performance evaluations.

Agreement

For evaluation of positioning performance of redcap UEs in 700MHz band, the gNB antenna model is:

- gNB antenna configuration from TR38.830, (M,N,P,Mg,Ng) = (4,2,2,1,1), (dH, dV) = (0.5, 0.8)λ

Agreement

Use 2Rx and 1Tx for baseline number of UE branches in FR2 in the UE antenna configuration table for RedCap UEs evaluation.

FFS: optional configurations for number of UE branches in FR2.