3GPP TSG RAN WG1 #110				                            R1-2206871

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.1.4.1

Source:	LG Electronics

Title:	UL precoding indication for multi-panel transmission

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss UL precoding indication for multi-panel transmission based on S-DCI mTRP PUSCH framework and based on M-DCI mTRP PUSCH framework, separately. Specifically, we discuss how to reuse S-DCI/M-DCI based mTRP PUSCH framework to schedule STxMP PUSCH, the limitation of the legacy framework, and potential candidates of STxMP transmission scheme.

Discussion

STxMP based on S-DCI mTRP PUSCH framework

In Rel-17, S-DCI based mTRP PUSCH repetition is introduced. In order to support this feature, many scheduling fields and parameters such as SRI, TPMI, TPC, PTRS, SRS resource set, and PUSCH power control parameters are extended in TRP specific manner and SRS resource set indicator field is also introduced. Even though STxMP and mTRP PUSCH repetition are different transmission schemes, the precoder indication mechanism designed for mTRP PUSCH can be a starting point for STxMP precoder indication because two features have similarity from signaling perspective. Therefore, these fields and parameters for mTRP PUSCH repetition can be reused or slightly modified to support STxMP without starting from the scratch.

Observation 1: Many scheduling fields and parameters to support Rel-17 mTRP PUSCH repetition such as 2 SRI fields, 2 TPMI fields, 2 TPC fields, 2 PTRS fields, 2 SRS resource sets, 2 PUSCH power control parameters, and SRS resource set indicator field can be reused or slightly modified to support STxMP.

Two TPMI fields are already supported in a single DCI so each TPMI field can be used to indicate RI/PMI for each panel. In Rel-17, in order to minimize TPMI field size increase, second TPMI field is designed without rank information, which is reasonable given that same rank is applied for the PUSCH repetition. However, when it comes to STxMP, this restriction is not needed and it may reduce the performance of STxMP transmission. So, we can consider to use two TPMI fields without dependency each other unlike Rel-17.

Proposal 1: Reuse 2 TPMI field without same rank restriction for STxMP transmission

SRS resource set indicator field also can be reused for STxMP transmission. In case of 00/01, single panel is used for UL transmission and panel selection can be done with 00 or 01. In case of 10/11, STxMP transmission is applied and those two codepoints can be used for different purpose. For example, it can be used for dynamic switching STxMP schemes for eMBB and for URLLC. If repetition number is indicated as N, STxMP transmission is repeated N times for N transmission occasions. As a result, PUSCH reliability can be further enhanced.

Proposal 2: Reuse SRS resource set indicator field for panel selection transmission (00/01) and STxMP transmission (10/11).

Proposal 3: Support to configure STxMP and time domain repetition at the same time for further PUSCH reliability enhancement.

Legacy DMRS field can be reused for SDM STxMP but DMRS ports should be grouped for different panels. Specifically, total rank is determined by the sum of RI for 1st SRI/TPMI (e.g. RI1) and RI for 2nd SRI/TPMI (e.g. RI2) and DMRS ports are indicated based on the total rank. Then, DMRS ports for each panel can be grouped in a following simple way. First RI1 DMRS ports among indicated DMRS ports are allocated for 1st SRI/TPMI and rest of them are for 2nd SRI/TPMI.

Proposal 4: Reuse a single DMRS port indication field and group indicated ports for each panel for SDM STxMP.

STxMP is simultaneous transmission across multi-panel by definition but there are multiple transmission schemes that can be considered with STxMP. Depending on whether transmission layer from multi-panel is the same or different, two different STxMP transmission schemes can be considered; SFN STxMP and SDM STxMP, respectively.

In SFN STxMP case, the same layers are transmitted from multi-panel. It can be considered as PUSCH repetition but, unlike Rel-17 using different TO, two repetitions are transmitted at the same time but from different panel with potentially different Tx beams. Since each PUSCH repetition is transmitted toward different TRP, macro diversity gain can be achieved even in blockage channel environment and latency can be more reduced compared to conventional mTRP PUSCH repetition. In addition, it may also be considered to indicate a co-phasing parameter for coherent SFN STxMP transmission to improve combining performance further.

On the other hand, in SDM STxMP case, different layers of PUSCH are transmitted from multi-panel. It can be considered as an uplink version of S-DCI based DL NCJT. For example, the first layer of rank 2 PUSCH is transmitted from panel 0 to TRP 0 and the second layer is transmitted from panel 1 to TRP 1 at the same time. Given that total transmission rank increases by using two different UL channels, throughput gain can be expected.

In our view, the usage of SFN STxMP and SDM STxMP are different and both are very useful.

Proposal 5: Support SFN STxMP for URLLC and for cell edge UE performance enhancement and SDM STxMP for eMBB as S-DCI based STxMP transmission schemes.

In the last meeting, Frequency and time overlapping types are shortly discussed. The overlapping types are determined depending on STxMP schemes as follows:

- SDM STxMP and SFN STxMP: fully overlapped in time and frequency

- FDM STxMP: fully overlapped in time and non-overlapped in frequency

Since those STxMP schemes guarantee fully overlapped PUSCH in time, symbol level power control is not needed. However, different power control for different panel should be supported due to different pathloss and the total transmission power from two panels can be over Pc_max. Therefore, further study is needed on how to reduce per panel transmission power to satisfy maximum total power constraints.

Proposal 6: Study how to reduce per panel transmission power to satisfy maximum total power constraints for time domain fully overlapped PUSCH.

STxMP based on M-DCI mTRP PUSCH framework

In Rel-16, M-DCI based mTRP PUSCH transmission is introduced. Specifically, each TRP independently schedules its own PUSCH in different symbol with DCI associated with its CORESET pool index. Since gNB should avoid time domain overlapping between different TRP’s PUSCH/PUCCH, different TRP’s UL channels are transmitted in TDM manner. Even if multiple TRPs use separate DCI, the same UL scheduling parameters are shared such as SRS resource set, PUSCH power control parameters and codebook configuration without TRP specific extension. As a result, it has a limitation on TRP specific scheduling flexibility.

For STxMP transmission, M-DCI mTRP PUSCH framework also can be reused or slightly modified. For example, one DCI associated with CORESET pool index 0 can be used to schedule PUSCH transmission for panel 0 and another DCI associated with CORESET pool index 1 can be used to schedule PUSCH transmission for panel 1.

Observation 2: M-DCI mTRP PUSCH framework, in which DCI can be separated based on associated CORESET pool index, can be reused or slightly modified to support STxMP.

However, in order to support simultaneous transmission, unlike Rel-16 M-DCI mTRP PUSCH, scheduling restriction to avoid time domain overlapping should be removed. In this way, two PUSCH transmitted from different panels can be time domain overlapped or not. One issue related to the time domain overlapping is that, if the two PUSCH are partial overlapped in time, symbol level PUSCH power control may be needed, which increases UE complexity.

For example, if symbol 0 and 1 are allocated for PUSCH 0 from panel 0 and symbol 1 and 2 are allocated for PUSCH 1 from panel 1, PUSCH in symbol 0 and 2 can use maximum transmission power because single PUSCH is transmitted in those symbol but each PUSCH in symbol 1 can use the half of maximum transmission power because two PUSCHs are transmitted in the same symbol. In order to avoid symbol level power control in this example, the same transmission power can be allocated for non-overlapped symbols 0 as PUSCH 0 in symbol 1 but maximum power in symbol 0 is unnecessary limited by half, resulting in UL performance degradation. Meanwhile, in case of fully overlapped PUSCHs, there is no such issue.

Observation 3: If STxMP PUSCHs are partially overlapped in time domain, symbol level PUSCH power control is needed, which increases UE complexity.

Proposal 7: In order to avoid symbol level PUSCH power control, consider scheduling restriction for time domain fully overlapped PUSCHs.

Even though different TPMI can be indicated for different panel by using separate DCI, different codebook configuration, such as max rank, codebook subset restriction, full power mode and so on, cannot be applied for different panel according to Rel-16 M-DCI based mTRP PUSCH framework since CORESET pools share the same PUSCH-Config IE. However, in practice, two panels can be implemented with different antenna configuration, antenna coherency characteristic and PA combination. Taking this panel specific implementation into account, separate codebook configuration should be supported for multi-panel.

Proposal 8: Support separate codebook configuration for different Tx panels.

Regarding STxMP transmission scheme based on M-DCI mTRP PUSCH framework, each panel transmits separate PUSCH containing different TB, unlike S-DCI based STxMP which transmits the same TB as we described in Section 2.1. Given that M-DCI based mTRP transmission is introduced mainly for non-ideal backhaul network, in which dynamic coordination between TRPs are not available, different TB transmission for multi-panel seems a natural candidate STxMP transmission scheme under M-DCI framework.

Proposal 9: Consider DTxMP (Different TB across Multi-Panel) for eMBB as a potential candidate of M-DCI based STxMP transmission scheme.

Evaluation results for SFN STxMP

With evaluation assumption in Table 2, Figure 1 shows RSRP difference between the strongest and the second strongest UE panels and between the strongest and the third strongest UE panels, respectively, when UE has 3 panels. From the figure, RSRP difference between the strongest and the second strongest UE panels is less than 5dB in urban macro and 9dB in indoor hotspot scenarios, respectively, for half of UEs. It means that, for majority UEs, the UL channel quality from the second strongest UE panel is not significantly degraded compared to the strongest UE panel so that using STxMP based on two strong panels can provide a potential gain.

Figure 1. RSRP difference between multi-panel in dense urban macro and indoor hotspot.

Observation 4: RSRP difference between the strongest and the second strongest UE panels are less than 5dB in urban macro and 9dB in indoor hotspot scenarios, respectively, for half of UEs

Table 1. System level throughput results in dense urban macro scenario.

| Scenario | Scheme | Cell edge UE throughput [Mbps] | Average UE throughput [Mbps] |
| --- | --- | --- | --- |
| Urban macro | 1-panel selection (baseline) |  |  |
|  | SFN STxMP with max UE power option 1 |  |  |
|  | SFN STxMP with max UE power option 2 |  |  |

[To be updated with revision]

Table 2. Evaluation assumptions

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, BW: 80 MHz, |
| Scenarios | 1. Dense urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per cell), 100% outdoor 2. Indoor (TR 38.901/802) |
| UE speed | 3 km/hr for all UEs |
| Maximum UE Tx Power | - Option 1: Max TRP of 23 dBm and max EIRP 43 dBm of two panels - Option 2: Max TRP of 23 dBm and max EIRP 43 dBm per panel Note 1: Companies to state additional details on their simulation assumptions, if any. Note 2: In Option 2, the max TRP of two panels might exceed the limit of PC2 in one band based on existing RAN4 power class definitions (which is currently per band). Companies to provide details whether Option 2 results in exceeding such limit in actual simulations. Note 3: In Option 1, companies to explain how max EIRP across two panels transmitting beams in different directions is determined. Note 4: In case that Option 2 is used and max TRP of two panels exceeds the limit of PC2, companies to provide the excess value of the TRP of STxMP transmission over the TRP of the single panel transmission baseline. |
| BS receiver Noise Figure | 7 dB |
| BS Antenna Configuration | For dense urban: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ For Indoor: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | According to TR 38.802 |
| UE antenna configuration | Panel structure: 1x4x2 or (M, N, P) = (1, 4, 2), dH = 0.5 λ. Number of panels: 3 (left, right, front) |
| UE Antenna radiation pattern | According to TR 38.802, Front-to-back ratio = 40 dB |
| UE dropping | Random |
| Traffic Model | Option 1: FTP model 1 with packet size 0.5Mbytes |
| UE Antenna height | 1.5 m |
| UL MIMO Mode, rank | UL SU-MIMO Up to rank 2 for STxMP with 2 panels. |
| Baseline scheme | Single panel transmission with panel selection |

Conclusion

In this contribution, we discuss we discuss UL precoding indication for multi-panel transmission based on S-DCI mTRP PUSCH framework and based on M-DCI mTRP PUSCH framework, separately, and propose the followings:

STxMP based on S-DCI mTRP PUSCH framework:

Observation 1: Many scheduling fields and parameters to support Rel-17 mTRP PUSCH repetition such as 2 SRI fields, 2 TPMI fields, 2 TPC fields, 2 PTRS fields, 2 SRS resource sets, 2 PUSCH power control parameters, and SRS resource set indicator field can be reused or slightly modified to support STxMP.

Proposal 1: Reuse 2 TPMI field without same rank restriction for STxMP transmission

Proposal 2: Reuse SRS resource set indicator field for panel selection transmission (00/01) and STxMP transmission (10/11).

Proposal 3: Support to configure STxMP and time domain repetition at the same time for further PUSCH reliability enhancement.

Proposal 4: Reuse a single DMRS port indication field and group indicated ports for each panel for SDM STxMP.

Proposal 5: Support SFN STxMP for URLLC and for cell edge UE performance enhancement and SDM STxMP for eMBB as S-DCI based STxMP transmission schemes.

Proposal 6: Study how to reduce per panel transmission power to satisfy maximum total power constraints for time domain fully overlapped PUSCH.

STxMP based on M-DCI mTRP PUSCH framework:

Observation 2: M-DCI mTRP PUSCH framework, in which DCI can be separated based on associated CORESET pool index, can be reused or slightly modified to support STxMP.

Observation 3: If STxMP PUSCHs are partially overlapped in time domain, symbol level PUSCH power control is needed, which increases UE complexity.

Proposal 7: In order to avoid symbol level PUSCH power control, consider scheduling restriction for time domain fully overlapped PUSCHs.

Proposal 8: Support separate codebook configuration for different Tx panels.

Proposal 9: Consider DTxMP (Different TB across Multi-Panel) for eMBB as a potential candidate of M-DCI based STxMP transmission scheme.

Observations from evaluation for SFN STxMP:

[To be updated with revision]