3GPP TSG RAN WG1 #110							       R1-2206902

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.2.2.1

Title:	Discussion on evaluation on AI/ML for CSI feedback enhancement

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 SID for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface was approved [1], the objective of this study item is to study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. In this SID, one specific use case for AI/ML is CSI feedback enhancement.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this contribution, we present our views on the evaluation methodology and KPI for AI/ML based CSI feedback enhancement and some initial evaluation results are also provided here.

- Evaluation methodology and preliminary results

AI based CSI feedback enhancement is aimed at CSI reporting overhead reduction and accuracy improvement. In our contribution [2], we have discussed the sub use case for CSI feedback enhancement. In this section, we show our specific evaluation methodology, KPI and preliminary results for the sub use case of AI based CSI compression and feedback.

As shown in Fig.1, for the use case of AI based CSI enhancement, UE obtain the joint eigenvectors form the channel matrix H, and then compress the eigenvectors to a bitstream via encoder and quantization. After receiving the compressed bitstream, gNB will recover the feedback bits to the eigenvectors through dequantization and decoder.

Fig.1 AI based CSI feedback framework

  - KPI

2.1.1 Metrics for Performance Evaluation of AI-based CSI Feedback

For the evaluation of AI based CSI enhancement, we use the square of generalized cosine similarity (SGCS) as the criterion to evaluate the difference between recovered channel and the original channel.

The SGCS is defined as:

wherein, the  is the original eigenvector of the n-th suband, the  is the recovered eigenvector of the n-th suband (n=1, …, N).

Besides, in RAN1#109-e meeting [3], for rank > 1 cases, the potential KPIs for evaluation metric were discussed:

Agreement For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI - For GCS/SGCS, - FFS: how to calculate GCS/SGCS for rank>1 - FFS: whether GCS or SGCS is adopted - FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap. Agreement For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:  Method 1: Average over all layers o Note: is the eigenvector of the target CSI at resource unit i and K is the rank. is the output vector of the output CSI of resource unit i. is the total number of resource units. denotes the average operation over multiple samples.  Method 2: Weighted average over all layers o Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)  Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)  Other methods are not precluded  FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

In Method 1 and Method 2, the idea is generally to average the GCSs/SGCSs over all layers to one single GCS/SGCS. We think the similarity between output CSI and target CSI may not be shown clearly after these kinds of process, hence, Method 3 is our preference. The SGCS is separately calculated for each layer and the comparison is performed per layer.

Proposal 1: Adopt SGCS as the intermediate KPI as part of the ‘Evaluation Metric’.

Proposal 2: For rank > 1 cases, SGCS is separately calculated for each layer.

2.2 Data set

Statistical channels used in link-level simulation or system level simulation can be used to construct the data set for the evaluation of AI based CSI feedback. In this contribution, the UMA channel and CDL-C-30 channel in TR 38.901 are both used to generate channel data. The eigenvectors of K subbands are used for model training, model validation and model testing.

The detail evaluation assumptions are shown in Table 1 and Table 2:

Table 1.  Simulation parameters for UMA channel

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) 19 cells, 3 sectors for each cell |
| Frequency Range |  | 2.1GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (4,4,2,1,1,4,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Rank 1 |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| Channel estimation |  | ideal channel estimation |
| dataset |  | 150 drops, 1140UE/drop, 171k samples |

Table 2.  Simulation parameters for CDL-C-30 channel

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| RB number | 48 |
| Subband number | 12 |
| BS antenna number | 32 |
| UE antenna number | 4 |
| Channel Model | CDL-C |
| UE speed | 3km/h |
| Delay spread | 30ns |
| Rank | 1 |
| UE number | 1000 |
| Slot number | 100 |

2.3 Preliminary results

The EVCsiNet model based on [4] and a transformer model are used in AI model training. Besides, uniform quantization is used in the evaluation. The AI models apply SGCS as its loss function. And we apply separate AI models for each number of feedback bits.

2.3.1 AI model performance on CDL-C channel and UMA channel

The square of generalized cosine similarity (SGCS) between recovered eigenvectors and original eigenvectors under different number of feedback bits in UMA scenario and CDL-C channel are shown below. Besides, the SGCS for Type-I and enhanced Type-II codebook under approximate feedback bits are also given here for comparison.

Table 3. The SGCS of different CSI approaches in CDL-C channel

| Approach | Feedback bits | Performance |
| --- | --- | --- |
| Type-I | 32 | 0.626 |
| EVCsiNet | 32 | 0.913 |
| Transformer | 32 | 0.900 |
| e-Type-II, L=2 | 49 | 0.840 |
| EVCsiNet | 48 | 0.941 |
| Transformer | 48 | 0.942 |
| e-Type-II, L=4 | 128 | 0.941 |
| EVCsiNet | 120 | 0.977 |
| Transformer | 120 | 0.980 |

Table 4. The SGCS of different CSI approaches in UMA channel

| Approach | Feedback bits | Performance |
| --- | --- | --- |
| EVCsiNet | 32 | 0.781 |
| Transformer | 32 | 0.814 |
| EVCsiNet | 48 | 0.810 |
| Transformer | 48 | 0.843 |
| EVCsiNet | 120 | 0.867 |
| Transformer | 120 | 0.900 |

According the evaluation results above, it can be observed that:

- With the same or similar number of feedback bits, AI based approach could obtain 4%~40% performance gain over traditional codebook in the square of generalized cosine similarity;

- With similar performance in the square of generalized cosine similarity, AI based approach could reduce 30%~60% feedback bits.

Observation 1: Compared with traditional codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same or similar number of feedback bits.

Observation 2: Compared with traditional codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits when achieving the same CSI or higher accuracy.

2.3.2 AI model complexity and memory storage

In RAN1#109-e meeting [3], for the model complexity and memory storage evaluation/ comparison, we have the following agreements:

Agreement For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies. Agreement For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both. - FFS: the format of the AI/ML parameters

The complexity of the AI models considered in this contribution is listed in Table 5 from the perspective of float point operations (FLOPs) and trainable parameters:

Table 5. FLOPS and trainable parameters of AI models

| Number of Feedback bits | FLOPs () |  | Trainable Parameters () |  |
| --- | --- | --- | --- | --- |
|  | EVCsiNet | Transformer | EVCsiNet | Transformer |
| 32 bits | 9.35 | 42.85 | 4.67 | 21.42 |
| 48 bits | 9.38 | 42.87 | 4.68 | 21.43 |
| 120 bits | 9.51 | 43.00 | 4.75 | 21.50 |

2.3.2 Generalization

In RAN1#109-e meeting [3], for the verification of generalization, we have the following agreement:

Agreement For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including: - The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios - The configuration(s)/ scenario(s) for testing/inference - Other details are not precluded

The generalization capability is to verify whether a model trained under a certain assumption can be applied well under different assumptions. The different assumptions may be different scenarios (e.g, Uma, Umi, InH), or different configurations (e.g., different bandwidth, different number of antenna ports). Different scenarios will not affect the size of the input/output data for training and testing/inference, while the dimension of input/output data in different configurations may be different.

In real communication system, UE might experience different scenarios and configurations due to UE’s mobility and gNB’s scheduling. It is unrealistic to train and store the models for each configuration/ scenario. Therefore, it is important to improve the AI/ML model generalization capability to adapt to different configurations/ scenarios.

2.3.2.1 Generalization over different scenarios

CSI is highly correlated to the varying physical radio environment (e.g. the distribution of multipath, SINR, intra- or inter-cell movement of UEs), which can be quite diversified for different scenarios, e.g., Uma/UMi/ Indoor, LOS/ NLOS and etc. As a consequence, the model of auto-encoder based CSI feedback trained in one scenario may suffer performance degradation in another scenario.

Usually, training the scenario specific AI model for each scenario can be considered as a method to achieve the upper bound performance of AI based approach. However, the UE complexity of handling and storing multiple AI models is also very challenging. There might be two approaches to solve this problem. One method is to utilize a mixed dataset from different typical scenarios to train one model and apply this model to perform inference in different scenario. The other way is to use transfer learning method. For example, a baseline auto-encoder model can be well trained with abundant samples under a certain scenario. To apply the baseline auto-encoder to new scenario, the baseline auto-encoder can be fine-tuned with a small number of data samples under the new scenario.

Proposal 3: The generalization capability of AI model across different scenarios could be evaluated.

Proposal 4: The solution to improve the generalization capability of AI model across different scenarios could be further studied.

In this section, for generalization across different scenario, we construct a synthetic dataset with samples from CDL-C-30 and CDL-C-300. A mixed dataset of 50K samples from CDL-C-30 and 50K samples from CDL-C-300 is generated as the training dataset. We additionally adopt the AI model trained with the samples only from CDL-C-30 for performance comparison. Both two models are used to perform inference over samples from CDL-C-30. The SGCS using different AI algorithms is shown below:

Table 6. SGCS of AI model trained with mixed dataset using EVCsiNet

| Feedback bits | Performance |  |
| --- | --- | --- |
|  | CDL-C-30 dataset | Mix dataset |
| 32 | 0.913 | 0.809 |
| 48 | 0.934 | 0.847 |
| 120 | 0.968 | 0.907 |

Table 7. SGCS of AI model trained with mixed dataset using Transformer

| Feedback bits | Performance |  |
| --- | --- | --- |
|  | CDL-C-30 dataset | Mix dataset |
| 32 | 0.919 | 0.882 |
| 48 | 0.948 | 0.923 |
| 120 | 0.977 | 0.965 |

Observation 3: The AI model trained with mixed dataset across various scenarios might have some performance loss comparing with dedicated model.

2.3.2.2 Generalization over different configurations

Considering verifying the generalization capability over different configurations, in this section, we evaluate the generalization of applying AI model to different number of subbands and different number of feedback bits.

For generalization across different number of subbands, we train AI model with the samples composed of the eigenvectors of 12 subbands and apply this AI model to test the samples composed of the eigenvectors of 8 subbands.

According the evaluation result, when applying the model trained by the data set composed of 12 subbands to compression for 8 subbands, the SGCS performance will be degraded.

Table 8. The SGCS for different numbers of subbands using Transformer

|  | Performance |  |  |
| --- | --- | --- | --- |
|  | 32 bits | 48 bits | 120 bits |
| Baseline (12 subbands) | 0.919 | 0.948 | 0.977 |
| 8 subbands | 0.891 | 0.885 | 0.928 |

Table 9. The SGCS for different numbers of subbands using EVCsiNet

|  | Performance |  |  |
| --- | --- | --- | --- |
|  | 32 bits | 48 bits | 120 bits |
| Baseline (12 subbands) | 0.913 | 0.934 | 0.968 |
| 8 subbands | 0.894 | 0.891 | 0.875 |

Observation 4: The AI model trained under one number of subbands might have some performance loss when performing interference on CSI compression of a different number of subbands.

In NR systems, different number of feedback bits can be configured for CSI reporting. For generalization across different number of feedback bits, we train a common encoder which can be applied to different numbers of feedback bits to avoid the management of many models at the UE side.

For each number of feedback bits, one common encoder and the specific decoder are used. We choose 3 different number of feedback bits for performance evaluation and the performance of the dedicated model for each number of feedback bits is given as baseline.

Baseline: dedicated models of which the number of feedback bits are 32, 48 and 120 bits.

Case 1: one joint encoder and 2 separate decoders of which the number of feedback bits are 32 and 48 bits.

Case 2: one joint encoder and 3 separate decoders of which the number of feedback bits are 32, 48 and 120 bits.

The SGCS performance for each case is shown below:

Table 10. The SGCS for different number of feedback bits using Transformer

|  | Performance |  |  |
| --- | --- | --- | --- |
|  | 32 bits | 48 bits | 120 bits |
| Baseline | 0.91900 | 0.94799 | 0.97709 |
| Case 1: 32+48 | 0.91690 | 0.94249 | \ |
| Case 2: 32+48+120 | 0.91043 | 0.94170 | 0.97227 |

Table 11. The SGCS for different number of feedback bits using EVCsiNet

|  | Performance |  |  |
| --- | --- | --- | --- |
|  | 32 bits | 48 bits | 120 bits |
| Baseline | 0.91288 | 0.93366 | 0.96822 |
| Case 1: 32+48 | 0.91228 | 0.93019 | \ |
| Case 2: 32+48+120 | 0.91270 | 0.93290 | 0.96335 |

According the evaluation result, the unified model of one common encoder and multiple specific decoders has similar performance on SGCS as dedicated model under various number of feedback bits.

Observation 5:  The unified AI model of one common encoder and multiple specific decoders performs well across different number of feedback bits.

- Conclusion

In this contribution, we share our views on the evaluation methodology and KPI for AI based CSI compression and the preliminary evaluation results are also provided. The observations and proposals are summarized below:

Observation 1: Compared with traditional codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same or similar number of feedback bits.

Observation 2: Compared with traditional codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits when achieving the same CSI or higher accuracy.

Observation 3: The AI model trained with mixed dataset across various scenarios might have some performance loss comparing with dedicated model.

Observation 4: The AI model trained under one number of subbands might have some performance loss when performing interference on CSI compression of a different number of subbands.

Observation 5:  The unified AI model of one common encoder and multiple specific decoders performs well across different number of feedback bits.

Proposal 1: Adopt SGCS as the intermediate KPI as part of the ‘Evaluation Metric’.

Proposal 2: For rank > 1 cases, SGCS is separately calculated for each layer.

Proposal 3: The generalization capability of AI model across different scenarios could be evaluated.

Proposal 4: The solution to improve the generalization capability of AI model across different scenarios could be further studied.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R1-2206903, Discussion on other aspects on AI/ML for CSI feedback enhancement

- 3GPP RAN1#109-e, RAN1 Chairman’s Notes, May 2022.

- W. Liu, W. Tian, H. Xiao, S. Jin, X. Liu and J. Shen, "EVCsiNet: Eigenvector-based CSI Feedback under 3GPP Link-Level Channels," IEEE Wireless Communications Letters, doi: 10.1109/LWC.2021.3112747.