3GPP TSG RAN WG1 Meeting #110	R1-2206936

Toulouse, France, August 22nd – 26th, 2022

Source:	Sharp

Title:	Remaining issues on NR sidelink enhancement

Agenda Item:	8.11

Document for:	Discussion and Decision

Introduction

In this document, we share our views on a few aspects relating to remaining issues of resource allocation for power saving and inter-UE coordination, with corresponding TPs.

Discussion of remaining issues on resource allocation for power saving

Pre-emption check and re-evaluation in partial sensing

Indicating “qth” reservation period for pre-emption and re-evaluation in partial sensing

In Rel-16 NR V2X, MAC layers may request PHY to determine a subset of resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure by providing a set of resources (r0,r1,r2,…) and (r0',r1',r2',…) subject to re-evaluation and pre-emption respectively. In Rel-17 NR partial sensing, for pre-emption check/re-evaluation when both PBPS and CPS are performed if Prsvp_TX≠0, the initialization of candidate resource set is based on the “qth” reservation period. The corresponding specs text are quoted as follows,

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot t'yiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to t'y+q×Prsvp_TX'SL, where t'ySL is a slot index of Y candidate slots used in the initial resource (re)selection. - t'yiSL is the first candidate slot after slot n+T3. - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. - By default, M is 31 unless (pre-)configured with another value. by sl-CPS-WindowPeriodic.

It is noted that when MAC layers provide resources subject to re-evaluation/pre-emption check, the parameter of “qth” reservation period is not indicated to PHY.

A corresponding TP is shown as follows.

TP1

Reason for change:; PHY is unaware of the parameter of “qth” reservation period for the resources provided by MAC layers for pre-emption check or re-evaluation.. Summary of change:; Clarify that the set of resources (r0,r1,r2,…) and the set of resources (r0',r1',r2',…) are in the qth reservation period ((q=0,1,2,…, Cresel-1)).. Consequences if not approved:; Unable to determine remaining Y candidate slots for pre-emption check/re-evaluation.. ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission: - the resource pool from which the resources are to be reported; - L1 priority, prioTX; - the remaining packet delay budget; - the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH; - optionally, the resource reservation interval, Prsvp_TX, in units of msec. - if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption. If Prsvp_TX≠0, the set of resources (r0,r1,r2,…) and the set of resources (r0',r1',r2',…) are in the qth reservation period (q=0,1,2,…, Cresel-1). - it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP. - Optionally, the indication of resource selection mechanism(s), as allowedResourceSelectionConfig, which may comprise of full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof. <Unchanged text omitted>

Proposal 1: Adopt TP1 as above, i.e. when MAC layers provide resources for pre-emption check or re-evaluation, it indicates the order “qth” reservation period, i.e. the resources are within the qth reservation period to PHY.

Monitoring slots for pre-emption check

Note that the resource(s) subject to pre-emption check has been indicated by a prior SCI, e.g. an SCI in the previous reservation period. E.g. for a resource in slot ty''SL subject to pre-emption, an SCI in slot ty'-P'rsvp_TX'SL may have already indicated the resource, which means the UE is unable to monitor slot ty'-P'rsvp_TX'SL due to half-duplex constraint. According to current specs, for slot ty''SL, the UE shall monitor a set of occasions ty'-k×P'reserve'SLwhile the set ty'-k×P'reserve'SL may include the slot ty'-P'rsvp_TX'SL (i.e.k=1), since Prsvp_TX can be included in the set of Preserve.

Therefore, it is straightforward to preclude the slot ty'-P'rsvp_TX'SL which indicates the resource subject to pre-emption check from the periodical sensing occasions ty'-k×P'reserve'SL, if any.

A corresponding TP is provided as follows.

TP2

Reason for change:; For pre-emption check in partial sensing, due to half-duplex constraint, the UE is unable to monitor the slot included in ty'-k×P'reserve'SL if the UE transmit a prior SCI in this slot to indicate the resource subject to pre-emption check in ty''SL.. Summary of change:; Clarify that the UE performs PBPS for the remaining Y candidate slots according to ty'-k×P'reserve'SL except for the slot(s) of a prior SCI which indicates the resource in slot ty''SL subject to pre-emption, where ty''SL is a slot belonging to the remaining Y candidate slots. Consequences if not approved:; Unable to monitor the corresponding periodical sensing occasion for pre-emption check in periodic-based partial sensing.. ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged text omitted> When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot t'yiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to t'y+q×Prsvp_TX'SL, where t'ySL is a slot index of Y candidate slots used in the initial resource (re)selection. - t'yiSL is the first candidate slot after slot n+T3. - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL except for the slot(s) of a prior SCI transmitted by the UE which indicates the resource in slot ty''SL subject to pre-emption checking, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. - By default, M is 31 unless (pre-)configured with another value. by sl-CPS-WindowPeriodic. <Unchanged text omitted>

Proposal 2: Adopt TP2 as above, i.e. for a resource subject to pre-emption in slot ty''SL which is among the remaining Y candidate slots, the UE monitors slots of periodic sensing occasions ty'-k×Preserve'SL except for the slot of a prior SCI which indicates the resource, if any.

Miscellaneous corrections

Clarification on candidate slots selection

When periodic reservation for another TB (sl-MultiReserveResource) is enabled for a resource pool, if the UE is configured by higher layer to perform partial sensing, the UE shall perform at least PBPS. For periodic transmission (Prsvp_TX≠0), the UE shall select Y candidate slots and for aperiodic transmission (Prsvp_TX=0), the UE shall select Y’ candidate slots, respectively. In current specs, i.e. Step 1) of clause 8.1.4, it may result in the case that the UE selects both Y and Y’ candidate slots in terms of aperiodic transmission and it clearly does not make sense. Therefore, the following TP is provided.

TP3

Reason for change:; The UE shall select both Y and Y’ candidate slots for aperiodic transmission (i.e. Prsvp_TX=0) when periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool according to current specs.. Summary of change:; Clarify that the UE selects Y candidate slots if Prsvp_TX≠0 when performing PBPS.. Consequences if not approved:; Unclear determination of candidate slots in terms of aperiodic transmission when periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool.. ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged text omitted> The following steps are used: 1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource if Prsvp_TX≠0, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). - Y is selected by UE where Y≥Ymin. - Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. The total number of candidate single-slot resources is denoted by Mtotal. <Unchanged text omitted>

Proposal 3: Adopt TP3 as above.

Determination of Preserve

The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList. It is noted that the list of sl-ResourceReservePeriodList in Rel-16 NR V2X always include the periodicity of ‘0ms’.

sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. The value ms0 is always configured.

When Preserve=0, the periodical sensing occasions ty-k×PreserveSL corresponds to the slot tySL, which is actually a candidate slot and not for monitoring. Therefore, in determination of Preserve, the values of Preserve correspond to all the non-zero periodicities from sl-ResourceReservePeriodList.

A corresponding TP is provided as follows.

TP4

Reason for change:; For PBPS, the UE shall monitor slots ty-k×P'reserve'SL. It is specified that if periodicSensingOccasionReservePeriodList is not configured, Preserve correspond to all periodicity from sl-ResourceReservePeriodList and the list always includes ‘0ms’. If Preserve=0, the UE shall monitor ty'SL which is a candidate slot and not for monitoring.. Summary of change:; Clarify that Preserve correspond to all the non-zero periodicities from the list of sl –ResourceReservePeriodList.. Consequences if not approved:; The resources in the candidate slots cannot be for SL transmission due to half-duplex constraint.. ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged text omitted> 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all the non-zero periodicity from sl-ResourceReservePeriodList. <Unchanged text omitted>

Proposal 4: Adopt TP4 as above.

Clarification on ty'SL in PBPS for aperiodic transmission

For aperiodic transmission in a resource pool enabling periodic reservation, the UE selects Y’ candidate slots within the RSW. If the candidate slots with corresponding PBPS sensing results are less than the (pre-)configured Ymin’, it is up to UE implementation to include other candidate slots with no PBPS sensing results. In current specs, in terms of determining the periodical sensing occasions for the above case, it is specified that ty'SL is a slot of the selected candidate slots which implies ty'SL also includes the other candidate slots which is selected up to implementation when Y’< Ymin’.

1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). - Y is selected by UE where Y≥Ymin. - Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. The total number of candidate single-slot resources is denoted by Mtotal. 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

As discussed above, the other candidate slots are slots w/o PBPS results. Therefore, further clarification on ty'SL is needed.

A corresponding TP is provided as follows.

TP5

Reason for change:; For aperiodic transmission in a resource pool enabling periodical reservation, if the Y’ candidate slots selected by the UE is less than Ymin’, it is up to UE implementation to include other candidate slots. The other candidate slots selected by the UE have no corresponding PBPS results. In determination of the sensing occasions for PBPS, ty'SL is a slot of all the selected candidate slots which include the other candidate slots as in the above case.. Summary of change:; Clarify that ty'SL is a slot of the selected candidate slots except for the other candidate slots included when Y’<Ymin’.. Consequences if not approved:; The periodical sensing occasions include slots the UE does not monitor.. ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged text omitted> The following steps are used: 1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). - Y is selected by UE where Y≥Ymin. - Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. The total number of candidate single-slot resources is denoted by Mtotal. 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots except for the other candidate slots when Y' is smaller than Ymin' and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.. <Unchanged text omitted>

Proposal 5: Adopt TP5 as above.

Discussion of remaining issues on inter-UE coordination

Determination of preferred resource set

In RAN1#109-e, the relation between RSW of IUC transmission and RSW of preferred/non-preferred resource set was agreed as a compromised way, i.e. X1, X2, X3 are determined by UE-A’s implementation under some certain constraints and whether to make the time gap from the IUC transmission to preferred/non-preferred resource in the IUC message larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL) is also up to implementation. In our understanding, the agreement was reached to cover both camps (Option 1 and Option 2 in the FL summary) for this issue, i.e. UE-A may implement to ensure the minimum time gap of (Tproc,0SL+Tproc,1SL+Tproc,2SL) from the IUC transmission to preferred/non-preferred resource as Option 1 implies. For this case, the RSW of IUC transmission comes before the indicated RSW of preferred resource set in time domain.

Observation 1: There can be UE-A’s implementation when the time gap from IUC transmission to the preferred/non-preferred resource larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL).

When UE-A triggers resource selection for IUC transmission at slot n’, the content of IUC message, i.e. the determined preferred resource set, is already determined at slot n, which naturally means n’>n. If the case as discussed above happens (i.e. the time gap from IUC transmission to the preferred/non-preferred resource is larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL)), the time gap from slot n to the indicated RSW of the preferred resource set would be much larger than the corresponding T1(0≤T1≤Tproc,1SL) in Rel-16 NR V2X, as shown in Fig 1,

Fig 1. Relation between RSW of IUC transmission and IUC content

In Rel-16 NR V2X, UE determines the number Q of hypothetical received SCI within RSW with the assumption that time gap from the triggering slot n to the start of RSW is not larger than Tproc,1SL, as shown in the following,

1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). The total number of candidate single-slot resources is denoted by Mtotal. […] […] 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX; c) the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

As discussed above, when the corresponding time gap grows much larger than T1, the determination of Q(q) should be changed, since all the hypothetical received SCI(s) within the time gap are actually not within RSW, as shown in Fig 2.

Fig 2. Step 6) for determination of preferred resource set

Specifically, in Fig 2, for the case of Prsvp_RX< Tscal(W) and n'-m≤Prsvp_RX', the determined Q=TscalPrsvp_RX=2 while clearly the slots of tm+Prsvp_RX''SL and tm+2×Prsvp_RX''SL are even not overlapped with the slots within the RSW, which means the exclusion procedures in Step 6) are not actually performed. To address the issue, one straightforward way is to skip the periodical occasions between the triggering slot n and the start of the RSW, i.e. the parameter q=1, 2, …, Q is changed to q=1+ starting time of RSW-triggering slot nPrsvp_RX, 2+starting time of RSW-triggering slot nPrsvp_RX,…, Q+starting time of RSW-triggering slot nPrsvp_RX directly, which is illustrated in Fig 3 as follows. To be specific, Q=3,4 and q starts from 3, accordingly tm+3×Prsvp_RX''SL and tm+4×Prsvp_RX''SL are both within the indicated RSW.

Fig 3. Determination of Q(q) for preferred resource set in IUC

Proposal 6: For determination of preferred resource set at UE-A, when the time gap from IUC transmission to the preferred/non-preferred resource is larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL), Step 6) in clause 8.1.4 of TS38.214 is reused with the following change:

- q=1+ starting time of RSW-triggering slot nPrsvp_RX, 2+starting time of RSW-triggering slot nPrsvp_RX,…, Q+starting time of RSW-triggering slot nPrsvp_RX.

Resource pool for transmission of IUC information

The following were agreed in RAN1#107-e meeting,

Agreement: - For inter-UE coordination information triggered by an explicit request in Scheme 1, - UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B Agreement: - For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1, - UE-A transmitting in a resource pool provides inter-UE coordination information associated with the same resource pool

And the following was captured in the latest version (17.2.0) of TS 38.214, which indicates that the resource pool for transmission of IUC information was supposed to be determine in higher layers.

### 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission When this procedure is triggered, the following parameters are provided by the higher layer: - the resource pool from which the preferred or non-preferred resources are to be determined; - […]

On the other hand, in our understanding of the latest version (17.1.0) of TS 38.321, the above RAN1 agreements have not been considered. Therefore, we propose to send an LS to ask RAN2 to implement the above RAN1 agreements.

Proposal 7: Send an LS to ask RAN2 to implement the RAN1 agreements on determination of resource pool for transmission of IUC information.

Corrections on RRC parameter name and value misalignment between TS38.213 and TS38.331

In subclause 16.3.0 of TS38.213, misalignments of RRC parameter name and value are corrected according to TS38.331.

TP6

Reason for change:; Misalignment of RRC parameter name and value between TS38.213 and TS38.331.. Summary of change:; - Change ‘typeAUEScheme2’ to ‘sl-TypeUE-A’. - Change ‘indicationUEBScheme2’ to ‘sl-IndicationUE-B’. - Change ‘optionForCondition2A1Scheme2’ to ‘sl-OptionForCondition2-A-1’. - Change ‘RSRP-ThresPerPriorities’ to ‘value 0’. - Change ‘RSRP-ThresWithRsrpMeasurement’ to ‘value 1’. - Change ‘PSFCHOccasionScheme2’ to ‘sl-PSFCH-Occasion’. - Change ‘followSCI’ to ‘value 0’. - Change ‘followReservedResource’ to ‘value 1’.. Consequences if not approved:; Misaligned RRC parameter name and value between TS38.213 and TS38.331.. ### 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged text omitted> A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - determines to transmit to the second UE the PSFCH with the conflict information. A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, sl-TypeUE-A is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if sl-IndicationUE-B = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 The first UE can be provided conditions by sl-OptionForCondition2-A-1 to determine conflict of reserved resources in a resource pool - if sl-OptionForCondition2-A-1 = 'value 0', the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if sl-OptionForCondition2-A-1 = 'value 1', the first UE can be provided a value Delta_Th by deltaRSRPThresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion - If sl-PSFCH-Occasion = 'value 0', the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If sl-PSFCH-Occasion = 'value 1', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. <Unchanged text omitted>

Proposal 8: Adopt TP6 as above.

Conclusion

In this contribution, we discuss a few aspects relating to remaining issues of sidelink enhancement and provide corresponding TPs to current specs, and make the following observation and proposals:

Proposal 1: Adopt TP1 as above, i.e. when MAC layers provide resources for pre-emption check or re-evaluation, it indicates the order “qth” reservation period, i.e. the resources are within the qth reservation period to PHY.

Proposal 2: Adopt TP2 as above, i.e. for a resource subject to pre-emption in slot ty''SL which is among the remaining Y candidate slots, the UE monitors slots of periodic sensing occasions ty'-k×Preserve'SL except for the slot of a prior SCI which indicates the resource, if any.

Proposal 3: Adopt TP3 as above.

Proposal 4: Adopt TP4 as above.

Proposal 5: Adopt TP5 as above.

Observation 1: There can be UE-A’s implementation when the time gap from IUC transmission to the preferred/non-preferred resource larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL).

Proposal 6: For determination of preferred resource set at UE-A, when the time gap from IUC transmission to the preferred/non-preferred resource is larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL), Step 6) in clause 8.1.4 of TS38.214 is reused with the following change:

- q=1+ starting time of RSW-triggering slot nPrsvp_RX, 2+starting time of RSW-triggering slot nPrsvp_RX,…, Q+starting time of RSW-triggering slot nPrsvp_RX.

Proposal 7: Send an LS to ask RAN2 to implement the RAN1 agreements on determination of resource pool for transmission of IUC information.

Proposal 8: Adopt TP6 as above.