3GPP TSG RAN WG1 #110	R1-2206953

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.2.2.1

Source: 	ETRI

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion

Introduction

This contribution presents ETRI’s views on the evaluation of AI/ML for CSI feedback enhancement use case for the AI/ML for NR Air Interface study [1].

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: o CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] o Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] o Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] …

Discussion

Initial evaluation results on CSI compression sub use case

In this section, we provide initial evaluation results on CSI compression sub use case based on LLS (link level simulations).

Evaluation assumption

For evaluation of AI/ML based CSI compression sub use case, the wireless channel data generated using the LLS for AI/ML based CSI compression sub use case. The parameters used for the LLS are provided in Table 1.

Table 1. Parameters for the LLS

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| BWP | 48 RBs |
| Subcarrier spacing | 15 kHz |
| Subband/PRG size | 4 RBs |
| Number of transmit antennas (NTX) | 32 |
| Number of receive antennas (NRX) | 4 |
| Number of layers | 1 |
| Delay profile | CDL-C |
| Delay spread | 30ns, 300ns |
| Channel estimation | Ideal |

AI model for CSI compression

For AI/ML based CSI compression sub use case, AI model at UE (Encoder) gets a wireless channel information as the input and generates compressed feedback information as the output and AI model at gNB (Decoder) gets compressed feedback information as the input and generates the original wireless channel information as the output.

Autoencoder architecture is deployed for the evaluation of CSI compression. The architecture of Autoencoder is in Figure 1.

Figure 1. Architecture of the Autoencoder

We set subband eigenvectors as input of Encoder and output of Decoder in the evaluation. Number of latent variables are M and and Q bits are used to represent each latent variable, i.e. size of CSI feedback information is MQ bits.

The Encoder and Decoder in Autoencoder architecture are deployed using neural networks and design choices of the neural networks can be made with considerations of performance and complexity. In [3], CsiNet is presented for Autoencoder structure for compression and reconstruction of the wireless channel information in time delay and angular spread domain. We use neural network structure of encoder and decoder in [3] and have made several modifications for the alignment with input, latent and output formats. Figure 2 shows the modified CsiNet model for AI/ML based CSI compression.

Figure 2. Modified CsiNet model for AI/ML based CSI compression

In order to train the AI Model, training samples are collected by the LLS. The parameters of AI Model and training are in Table 2.

Table 2. Parameters of AI Models

| Parameter | Value |
| --- | --- |
| Total number of samples | 5e4 |
| Portion of validation samples | 0.1 |
| Batch size | 256 |
| Total number of epoches | 512 |
| Learning algorithm | Adam |
| Learning rate | 0.001 |

Evaluation results

To see potential benefits of using AI/ML for CSI compression sub use case, we use an intermediate performance metric of GCS (generalized cosine similarity) as

GCS=E1Ni=1Nwi'Hwiwi'wi

where wi and wi' are original and reconstructed eigenvector(s) of subband i and E[⋅] denotes averaging over multiple samples. We use Rel-16 enhanced type 2 codebook as a reference and evaluates the performance of using the AI model for CSI compression compared to the reference scheme.

The evaluation results of the AI Model compared to eTypeII codebook in terms of GCS with CDL-C channel model with 30ns delay spread is in Table 3.

Table 3. GCS of eTypeII and AI Models (CDL-C, 30ns)

| Case | GCS |
| --- | --- |
| eTypeII, 49 bits | 0.826 |
| AI Model, 48 bits (M=24, Q=2) | 0.895 |
| eTypeII, 87 bits | 0.905 |
| AI Model, 86 bits (M=43, Q=2) | 0.937 |

The result with CDL-C channel model with 300ns delay spread is in Table 4.

Table 4. GCS of eTypeII and AI Models (CDL-C, 300ns)

| Case | GCS |
| --- | --- |
| eTypeII, 49 bits | 0.717 |
| AI Model, 48 bits (M=24, Q=2) | 0.834 |
| eTypeII, 87 bits | 0.796 |
| AI Model, 86 bits (M=43, Q=2) | 0.883 |

Observation 1: With an Autoencoder using a previously developed neural network structure, CsiNet, there are significant improvements in terms of GCS compared to the baseline (eTypeII) in CSI compression sub use case.

Conclusion

In this contribution, ETRI’s views on evaluation on AI/ML for CSI feedback enhancement were shown and the following observation were made:

Observation 1: With an Autoencoder using a previously developed neural network structure, CsiNet, there are significant improvements in terms of GCS compared to the baseline (eTypeII) in CSI compression sub use case.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- RAN#95-e Meeting Report

- C.-K. Wen, W.-T. Shih, and S. Jin, “Deep learning for massive MIMO CSI feedback,” IEEE Wireless Communications Letters, 2018.

- 3GPP TS 38.214 V17.0.0.