3GPP TSG RAN WG1 #110         	                                                  R1-2207002
Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.6.1

Source: 	MediaTek Inc.

Title: 	Potential solutions to further reduce UE complexity

Document for:	Discussion and decision

# Introduction

A new study item on further NR UE complexity reduction was approved with the following objectives [1]:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In RAN1#109-e meeting [2], the following are further agreed:

Agreement For cost reduction estimation, the detailed cost breakdown for the Rel-15 reference NR devices (as provided in Table 6.1-1 in TR 38.875) is reused.

Agreement For comparison with a Rel-17 baseline when evaluating the potential Rel-18 UE complexity reduction features, - The Rel-17 RedCap UE supports 20 MHz, 1 Rx, 1 layer, DL 64QAM, UL 64QAM, FDD or TDD. - In addition, optional results for the following comparisons can also be reported: - Results for HD-FDD UEs - Results for UEs with 2 Rx - In all comparisons, the UEs being compared have the same number of antenna branches, the same number of layers, the same maximum supported modulation order, and the same duplex mode (among HD-FDD, FD-FDD, and TDD).

Agreement For each potential Rel-18 further UE complexity reduction feature, at least the following aspects will be studied: - UE complexity reduction - Performance impacts [details FFS] - Network deployment and coexistence impacts [details FFS] - Specification impacts

In this contribution, the following complexity reduction options and their performance and coexistence impacts will be investigated:

- Section 2: Reduced maximum bandwidth to 5MHz in FR1

- Section 3: Reduced peak data rate

- Section 4: Combined complexity reduction options

Finally, recommendations for potential work item will be provided in Section 5.

# Reduced maximum bandwidth to 5MHz in FR1

## Characteristics of bandwidth reduction options

For options of bandwidth reduction, BW1/BW2/BW3 are agreed [2]:

Agreement - The following options for further UE bandwidth reduction can be studied: - Option BW1: Both RF and BB bandwidths are 5 MHz for UL and DL. - Option BW3: 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - In addition, optional results for the following option can also be reported: - Option BW2: 5 MHz BB bandwidth for all signals and channels with 20 MHz RF bandwidth for UL and DL. - At least the following cases are studied: - The resource allocation spans a bandwidth of maximum 5 MHz. - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - It is FFS whether to study other cases. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only.

Agreement - For Options BW1, - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Larger number of RBs that fit within 5 MHz can optionally be studied. - For Options BW2, - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Larger number of RBs that fit within 5 MHz can optionally be studied. - For Options BW3, - For 15 kHz SCS, 25 contiguous RBs are assumed to fit within the 5 MHz. - For 30 kHz SCS, 11 contiguous RBs are assumed to fit within the 5 MHz. - Larger number of RBs that fit within 5 MHz can optionally be studied. - Relevant assumptions (e.g., regarding potential scheduling restrictions) should be reported.

Based on definition, Table 1 shows which modem components can benefit from bandwidth reduction. The following observations can be obtained:

Observation 1: RF components can achieve no complexity reduction with bandwidth reduction.

- No complexity difference between BW1 and BW2

- Narrower bandwidth can even require larger capacitor size and slightly increase the complexity

Observation 2: Bandwidth reduction cannot achieve complexity reduction in the following modem components:

- DL control processing & decoder: Same maximum Poler codeword size and no critical timing relaxation

- Synchronization/cell search block: Same SSB processing

- MIMO specific processing blocks: Same MIMO and CSI functionality as reference Rel-17 RedCap UE

Observation 3: To accommodate 20MHz BW for non-PDSCH/PUSCH channels, BW3 cannot achieve complexity reduction in ADC/DAC and FFT/IFFT for BB

Table 1: Which modem components can achieve complexity reduction with bandwidth reduction

| Modem Components | Rel-15 Ref | Rel-17 RedCap (TDD 1RX) | BW1/BW2 | BW3 |
| --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 25.00% | ## No complexity difference below 20 MHz BW | ## No complexity difference below 20 MHz BW |
| ## RF: Filters | ## 15% | ## 3.75% |  |  |
| ## RF: Transceiver (including LNAs, mixer, and local oscillator) | ## 55% | ## 27.50% |  |  |
| ## RF: Duplexer / Switch | ## 5% | ## 5.00% |  |  |
| # RF: Total | # 100% | # 61.3% |  |  |
| ## BB: ADC / DAC | ## 9% | ## 1.8% |  |  |
| ## BB: FFT/IFFT | ## 4% | ## 0.8% |  |  |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.0% |  |  |
| ## BB: Receiver processing block | ## 29% | ## 8.7% |  |  |
| ## BB: LDPC decoding | ## 9% | ## 1.6% |  |  |
| ## BB: HARQ buffer | ## 12% | ## 2.2% |  |  |
| ## BB: DL control processing & decoder | ## 4% | ## 4.0% |  |  |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.5% |  |  |
| ## BB: UL processing block | ## 5% | ## 4.0% |  |  |
| ## BB: MIMO specific processing blocks | ## 9% | ## 4.5% |  |  |
| # BB: Total | # 100% | # 34.1% |  |  |
| RF+BB: Total (with RF:BB cost split 40:60) | 100% | 44.9% |  |  |

## Analysis of UE complexity reduction

With the complexity reduction characteristics in Table 1, Table 2 further shows the estimated total complexity reduction for difference configurations. It is noticed that the reference is based on the corresponding Rel-17 RedCap UE configurations (instead of those of Rel-15 eMBB UE). Additionally, the following observations are also provided:

Observation 4: Only limited complexity reduction, up to ~10%, can be achieved w.r.t. Rel-17 RedCap UE by bandwidth reduction options.

- For each component in BB, fundamental area overhead (due to at least control registers and Place-and-Route) cause saturation effect in further complexity reduction w.r.t. Rel-17 RedCap modem.

Table 2: Complexity reduction summary for bandwidth reduction options w.r.t Rel-17 RedCap UE

| Option | TDD with 1Rx | FDD with 1Rx | HD-FDD with 1Rx | TDD with 2Rx | FDD with 2Rx | HD-FDD with 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| BW1/BW2 | 8.2% | 8.6% | 9.6% | 8.8% | 9.5% | 10.3% |
| BW3 | 6.9% | 7.1% | 7.9% | 7.2% | 7.8% | 8.5% |

## Analysis of performance and coexistence impacts

For checking critical performance and addressing coexistence impact, PBCH/PDCCH CSS/SIB1 performances are compared since these channels are commonly shared by Rel-15/17 UEs and 5MHz-RedCap UEs. These three channels are also mandatory for coverage evaluations:

Agreement - Coverage for the following channels is evaluated for “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels” - SIB1 - PBCH - PDCCH CSS - [Msg4] - Following channels can be optionally evaluated - …

In Table 3, there show the performance comparisons among the bandwidth reduction options and Rel-17 reference RedCap for FDD (15 kHz SCS) case. For detailed performance curves, please refer to Appendix A of [3]. The following observations are provided:

Observation 5: For FDD (15 kHz SCS), there is around 3-4 dB performance loss for PDCCH CSS and/or SIB1 if there is RE truncation due to larger RB number than UE reception bandwidth.

Observation 6: For FDD (15 kHz SCS), reducing RB number of PDCCH CSS or SIB1 to be 24 RB also causes performance loss of 3-4 dB due to increased code rate.

Table 3: Required SNR (dB) for common channels with BW1/BW2/BW3 in FDD (15 kHz SCS) case

| FDD (15 kHz SCS) | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |
| R17 RedCap | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |
| BW1/BW2 (25 RB) | -4.8 | -1.5 | -1.6 | 2.2 | 2.3 |
| BW3 (5M PDSCH/PUSCH; 25 RB) | -4.8 | -4.5 | -1.6 | 2.2 | 2.3 |

In Table 4, there show the performance comparisons among the bandwidth reduction options and Rel-17 reference RedCap for TDD case with 30 kHz SCS. For detailed performance curves, please refer to Appendix A of [3]. There can observe significant performance loss due to RE truncation:

Observation 7: For TDD (30 kHz SCS), there is significant performance loss due to RE truncation. SIB1 is the worst cahnnel, and there is >10 dB (>6 dB) loss due to RE truncation from 48 RBs (24 RBs) to 5MHz bandwidth.

Observation 8: For TDD (30 kHz SCS), reducing RB number of PDCCH CSS or SIB1 to 24 RBs (for less RE truncation) cannot achieve better performance due to increased code rate.

Observation 9: Support 12 RBs for 5MHz reception in 30 kHz SCS can bring ~1 dB benefit w.r.t. 11-RB setting

Table 4: Required SNR (dB) for common channels with BW1/BW2/BW3 in TDD (30 kHz SCS) case

| TDD (30 kHz SCS) | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |
| R17 RedCap | -7.8 | -5.3 | -2.1 | -2 | 2 |
| BW1/BW2 (12 RB) | -3.2 | 3.2 | 3.8 | 8.4 | 8.9 |
| BW1/BW2 (11 RB) | -3.2 | 3.9 | 4.5 | 9.5 | 10.3 |
| BW3 (5M PDSCH/PUSCH; 12 RB) | -7.8 | -5.3 | -2.1 | 8.4 | 8.9 |
| BW3 (5M PDSCH/PUSCH; 11 RB) | -7.8 | -5.3 | -2.1 | 9.5 | 10.3 |

By the above performance results, we can assume 48-RB frequency span should be utilized for the common channels in order to minimize the performance impact to legacy UEs. Consequently, the following observation can be obtained:

Observation 10: SIB1 is the bottleneck common channel for bandwidth reduction options. While SIB1 performance can be compensated by combing multiple repetitions, the combined performance is bounded by PDCCH CSS performance since UE cannot decode PDSCH without the scheduling DCI information.

Observation 11: Options BW1 and BW2 prevent 5MHz UE from compensating SIB1 performance due to the inferior PDCCH CSS performance caused by RE truncation.

Proposal 1: Considering option BW1/BW2/BW3 for FDD (15 kHz SCS) and TDD (30 kHz SCS), BW3 with 12-RB reception bandwidth is relatively recommended because of achieving complexity reduction while being able to compensate SIB1 performance loss, assuming UE is allowed to combine sufficient SIB1 repetitions.

- Note: There may still require long delay to combine more than 16 SIB1 repetitions for compensating ~12 dB performance loss w.r.t. Rel-17 RedCap UE.

Observation 12: Regarding RF retuning solution to enable “smart” combing of SIB1, such operation requires UE to buffer PDSCH larger than 5MHz BW before decoding the combined SIB1, which effectively increases BW3 complexity to be the same as PR3.

- Note: Combing with RF retuning is different from HARQ combining since a truncated SIB1 is not any valid RV of SIB1 LDPC codeword.

Proposal 2: Combing with RF retuning is not considered for bandwidth reduction options.

# Reduced peak data rate

## Characteristics of peak data rate reduction options

In RAN1#109-e meeting [2], there also agree three options for peak data rate reduction, PR1/PR2/PR3:

Agreement - The following options for further UE peak rate reduction can be studied: - Option PR1: Relaxation of the constraint (vLayersj⋅Qmj⋅fj≥4) for peak data rate reduction. - Option PR2: Restriction of maximum TBS for PDSCH and PUSCH. - Option PR3: Restriction of maximum number of PRBs for PDSCH and PUSCH. - At least the following cases are studied: - The studied peak rate reduction applies to both UE-specific (unicast) and common (broadcast) channels. - The resource allocation spans a bandwidth of maximum 20 MHz (maximum UE channel bandwidth). - The same option is used for UL and DL. - The same option is used for idle/inactive and connected mode. - It is FFS whether to study other cases. - Note: As part of study of above options, it is not precluded to indicate that an observation is relevant for UL only or DL only.

Agreement - The restricted number of PRBs in Option PR3 is a hardcoded limit.

Agreement - For Option PR1, - The relaxed constraint is 1 (instead of 4). - Other values for the relaxed constraint that meet the 10-Mbps peak rate target can optionally be studied. - The parameters (, , ) [38.306] can be as in Rel-17 RedCap. - For Option PR2, - For 15 kHz SCS, the maximum TBS is 10000 bits per TB and per slot. - For 30 kHz SCS, the maximum TBS is 5000 bits per TB and per slot. - For Option PR3, - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 11. - Other number of RBs that meet the 10-Mbps peak rate target can optionally be studied. - Note: It is not precluded to report results also for other values. - Relevant assumptions (e.g., regarding potential limitations of the TBS sum in case of more than one simultaneous TB) should be reported.

Based on definition, Table 5 shows which modem components can benefits from bandwidth reduction. The following observations can be obtained:

Observation 13: PR1/PR2 can only achieve complexity reduction in the following modem BB components:

- LDPC decoding: Reduced peak data rate

- HARQ buffer: Reduced maximum TBS

Observation 14: PR3 can additionally achieve complexity reduction in the following modem BB components:

- Receiver processing block: RB number limit reduces (inner) receiver complexity

- UL processing block: RB number limit reduces total UL processing complexity

Observation 15: Compared with BW3, PR3 is assumed to buffer PDSCH of 20 MHz BW since, before DCI is decoded, UE has no knowledge of PDSCH RB allocation.

Table 5: Which modem components can achieve complexity reduction with peak-data-rate reduction

| Modem Components | Rel-15 Ref | Rel-17 RedCap (TDD 1RX) | PR1/PR2 | PR3 |
| --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 25.00% | ## No complexity reduction | ## No complexity reduction |
| ## RF: Filters | ## 15% | ## 3.75% |  |  |
| ## RF: Transceiver (including LNAs, mixer, and local oscillator) | ## 55% | ## 27.50% |  |  |
| ## RF: Duplexer / Switch | ## 5% | ## 5.00% |  |  |
| # RF: Total | # 100% | # 61.3% |  |  |
| ## BB: ADC / DAC | ## 9% | ## 1.8% |  |  |
| ## BB: FFT/IFFT | ## 4% | ## 0.8% |  |  |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.0% |  |  |
| ## BB: Receiver processing block | ## 29% | ## 8.7% |  |  |
| ## BB: LDPC decoding | ## 9% | ## 1.6% |  |  |
| ## BB: HARQ buffer | ## 12% | ## 2.2% |  |  |
| ## BB: DL control processing & decoder | ## 4% | ## 4.0% |  |  |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.5% |  |  |
| ## BB: UL processing block | ## 5% | ## 4.0% |  |  |
| ## BB: MIMO specific processing blocks | ## 9% | ## 4.5% |  |  |
| # BB: Total | # 100% | # 34.1% |  |  |
| RF+BB: Total (with RF:BB cost split 40:60) | 100% | 44.9% |  |  |

## Analysis of UE complexity reduction

With the complexity reduction characteristics in Table 5, Table 6 further shows the estimated total complexity reduction for difference configurations. It is noticed that the reference is based on the corresponding Rel-17 RedCap UE configurations (instead of those of Rel-15 eMBB UE). Additionally, the following observations are provided:

Observation 16: Only limited complexity reduction, up to 7%, can be achieved w.r.t. Rel-17 RedCap UE by peak-data-rate reduction options.

- PR1/PR2 can even achieve <4% complexity reduction

Table 6: Complexity reduction summary for peak-data-rate reduction options w.r.t Rel-17 RedCap UE

| Option | TDD with 1Rx | FDD with 1Rx | HD-FDD with 1Rx | TDD with 2Rx | FDD with 2Rx | HD-FDD with 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| PR1: | 2.4% | 3.2% | 3.5% | 2.7% | 3.6% | 3.9% |
| PR2: | 2.4% | 3.2% | 3.5% | 2.7% | 3.6% | 3.9% |
| PR3: | 5.8% | 6.0% | 6.6% | 6.2% | 6.5% | 7.0% |

## Analysis of performance and coexistence impacts

While there is less complexity reduction, peak-data-rate reduction options can achieve better performance. In Table 7 and Table 8, there show the performance comparisons among the peak-data-rate reduction options and Rel-17 reference RedCap for FDD (15 kHz SCS) and TDD (30 kHz SCS) cases, respectively. For detailed performance curves, please refer to Appendix A of [3]. The following can then be obtained:

Observation 17: Peak-data-rate reduction options can achieve the same performance as Rel-17 RedCap UE

Observation 18: For PR3, support processing 12 RBs in 30 kHz SCS can bring ~1 dB benefit w.r.t. 11-RB limit

Observation 19: PR3 can buffer whole SIB1. But the reduced complexity in “receiver processing block” requires more slots to accomplish SIB1 processing. The same SIB1 performance can be achieved if SIB1 processing time of more than one slot can be allowed.

- Note: RF retuning based combining is not required for PR3

Proposal 3: Jointly considering complexity reduction and performance impact over peak-data-rate reduction options, PR3 is relatively recommended for the better complexity reduction while being able to achieve the same common channel performance as Rel-17 RedCap, assuming PDSCH processing time of more than one slot is allowed for SIB1.

- Note: Up to 2 (4) slots will be sufficient for FDD (TDD) cases

Proposal 4: SIB1 processing time of more than one slot is allowed for PR3

- FFS: Extension to other PDSCH channels

Table 7: Required SNR (dB) for common channels with PR1/PR2/PR3 in FDD (15 kHz SCS) case

| FDD (15 kHz SCS) | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |
| R17 RedCap | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |
| PR1/PR2 (Data rate limit) | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |
| PR3 (25-RB limit on PDSCH/PUSCH) | -4.8 | -4.5 | -1.6 | 2.2 or -1.6* | 2.3 |

* PDSCH processing time of more than one slot is allowed

Table 8: Required SNR (dB) for common channels with PR1/PR2/PR3 in TDD (30 kHz SCS) case

| TDD (30 kHz SCS) | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |
| R17 RedCap | -7.8 | -5.3 | -2.1 | -2 | 2 |
| PR1/PR2 (Data rate limit) | -7.8 | -5.3 | -2.1 | -2 | 2 |
| PR3 (12-RB limit on PDSCH/PUSCH) | -7.8 | -5.3 | -2.1 | 8.4 or -2* | 8.9 or 2* |
| PR3 (11-RB limit on PDSCH/PUSCH) | -7.8 | -5.3 | -2.1 | 9.5 or -2* | 10.3 or 2* |

* PDSCH processing time of more than one slot is allowed

# Combined of complexity reduction options

## Characteristics of processing time relaxation options and combined complexity reduction options

In addition to bandwidth reduction and peak-data-rate reduction options, there also agree processing time relaxation options, PT1 and PT2, for combined complexity reduction:

Agreement - The following options for relaxed UE processing timeline will be studied: - Option PT1: Relaxation of UE processing time for PDSCH/PUSCH in terms of N1 and N2 - Option PT2: Relaxation of UE processing time for CSI in terms of Z and Z’ - UE complexity reduction estimates for relaxed UE processing timeline are only reported for combinations with UE bandwidth reduction or UE peak rate reduction.

Agreement - In Option PT1, the relaxation factor for N1 and N2 is 2. - In Option PT2, the relaxation factor for Z and Z’ is 2. - The combination of Options PT1 and PT2 is also studied.

Agreement - UE complexity reduction is studied for the following combinations: - Reference case (Rel-17 RedCap UE) - BW1 + PT1 + PT2 - BW3 + PT1 + PT2 - PR1 + PT1 + PT2 - PR3 + PT1 + PT2 - In addition, optional results for the following combinations can also be reported: - BW1 + PT1 - BW3 + PT1 - PR1 + PT1 - PR3 + PT1 - BW2 + PT1 + PT2 - PR2 + PT1 + PT2

Based on definition, Table 9 shows which modem components can benefits from processing time relaxation. The following observations can be checked:

Observation 20: PT1 is useful to additionally reduce the complexity for the following modem BB components by allowing longer processing time:

- Receiver processing block

- LDPC decoding

- DL control processing & decoder

- UL processing block

Observation 21: PT1 + PT2 can additionally achieve complexity reduction in MIMO specific processing blocks by allowing longer CSI related processing time.

Observation 22: PT1 (and PT2) can additionally contributed complexity reduction in “DL control processing & decoder” (and “MIMO specific processing blocks”), which cannot be achieved with BW3 or PR3.

Table 9: Which modem components can achieve additional complexity reduction with processing time relaxation

| Modem Components | Rel-15 Ref | Rel-17 RedCap (TDD 1RX) | PT1 | PT1+PT2 |
| --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 25.00% | ## No complexity reduction | ## No complexity reduction |
| ## RF: Filters | ## 15% | ## 3.75% |  |  |
| ## RF: Transceiver (including LNAs, mixer, and local oscillator) | ## 55% | ## 27.50% |  |  |
| ## RF: Duplexer / Switch | ## 5% | ## 5.00% |  |  |
| # RF: Total | # 100% | # 61.3% |  |  |
| ## BB: ADC / DAC | ## 9% | ## 1.8% |  |  |
| ## BB: FFT/IFFT | ## 4% | ## 0.8% |  |  |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.0% |  |  |
| ## BB: Receiver processing block | ## 29% | ## 8.7% |  |  |
| ## BB: LDPC decoding | ## 9% | ## 1.6% |  |  |
| ## BB: HARQ buffer | ## 12% | ## 2.2% |  |  |
| ## BB: DL control processing & decoder | ## 4% | ## 4.0% |  |  |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.5% |  |  |
| ## BB: UL processing block | ## 5% | ## 4.0% |  |  |
| ## BB: MIMO specific processing blocks | ## 9% | ## 4.5% |  |  |
| # BB: Total | # 100% | # 34.1% |  |  |
| RF+BB: Total (with RF:BB cost split 40:60) | 100% | 44.9% |  |  |

## Analysis of UE complexity reduction

With the complexity reduction characteristics in Table 9, Table 10 further shows the estimated total complexity reduction for difference configurations.

Observation 23: With PT1 + PT2, the combined complexity reduction with BW3 can be improved to 11%, which is 1% better than BW1/BW2-only that induces significant performance loss w.r.t. Rel-17 RedCap UE.

Observation 24: With PT1 + PT2, the combined complexity reduction with BW3 can be improved to ~10%, which is comparable to BW1/BW2-only that induces significant performance loss w.r.t. Rel-17 RedCap UE.

Table 10: Complexity reduction summary for combined complexity reduction options w.r.t Rel-17 RedCap UE

| Combined Options | TDD with 1Rx | FDD with 1Rx | HD-FDD with 1Rx | TDD with 2Rx | FDD with 2Rx | HD-FDD with 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| BW1+PT1 | 10.5% | 10.5% | 11.6% | 10.7% | 11.2% | 12.2% |
| BW1+PT1+PT2 | 10.9% | 11.1% | 12.4% | 11.3% | 12.0% | 13.0% |
| BW2+PT1+PT2 | 10.9% | 11.1% | 12.4% | 11.3% | 12.0% | 13.0% |
| BW3+PT1 | 8.9% | 8.8% | 9.8% | 9.1% | 9.5% | 10.3% |
| BW3+PT1+PT2 | 9.6% | 9.5% | 10.5% | 9.8% | 10.2% | 11.0% |
| PR1+PT1 | 4.9% | 5.1% | 5.7% | 5.0% | 5.5% | 6.0% |
| PR1+PT1+PT2 | 5.3% | 5.8% | 6.5% | 5.7% | 6.3% | 6.9% |
| PR2+PT1+PT2 | 5.3% | 5.8% | 6.5% | 5.7% | 6.3% | 6.9% |
| PR3+PT1 | 8.0% | 7.8% | 8.7% | 8.1% | 8.2% | 8.9% |
| PR3+PT1+PT2 | 8.5% | 8.3% | 9.2% | 8.6% | 9.0% | 9.7% |

It is noticed that, while the agreed relaxation factor for N1 and N2 is 2, we see the necessary relaxation on N1 and N2 can be limited to [7] symbols while still achieving the complexity reduction benefit. Since extended N1 and N2 can be regarded as allowing longer PDSCH and PUSCH processing for RedCap UE, the following proposal is thus suggested:

Proposal 5: To achieve the benefit be processing time relaxation while minimizing network scheduling impact, the following two options can be considered for further complexity reduction on RedCap UEs:

- Alt1: N1 and N2 are extended by [7] symbols

- Alt2: PDSCH/PUSCH processing time of more than one slots is allowed

Figure 1: Illustration of complexity reduction by allowing PDSCH processing time of more than one slot

# Recommendations for potential work item

In Table 11 and Table 12, there summarize the trade-off in performance and complexity reduction for the considered complexity reduction options for TDD (30 kHz SCS) and FDD (15 kHz SCS), respectively. The following are suggested:

Conclusion 1: There can achieve up to ~12% complexity reduction w.r.t. Rel-17 RedCap UE based on the options of bandwidth reduction, peak-data-rate reduction or combinations with processing time relaxation.

- Subject to no performance loss w.r.t. Rel-17 RedCap UE, only up to ~8% complexity reduction

Conclusion 2: Targeting minimum impact to legacy UEs and minimum performance loss w.r.t. Rel-17 RedCap UE, the following combined complexity reduction options can be considered for potential specification in Rel-18:

- Peak-data-rate reduction option PR3 and processing time relaxation option PT1 + PT2:

  - PDSCH/PUSCH #RB up to 12 RBs (25 RBs) for TDD/30 kHz SCS (FDD/15 kHz SCS)

  - PDSCH processing time of more than one slot is allowed

    - FFS: Whether to extend N1 and N2 by [7] symbols

  - CSI processing time of more slot(s) is allowed

Table 11: Summary for performance and complexity reduction trade-off for RedCap UE in TDD (30 kHz SCS)

| Performance: Required SNR (dB) |  |  |  |  |  |  | Complexity reduction w.r.t Rel-17 ReCap (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option | PBCH | PDCCH CSS |  | SIB1 |  |  | 1RX |  |  | 2RX |  |  |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |  | w/o PT1/2 | + PT1 | + PT1 + PT2 | w/o PT1/2 | + PT1 | + PT1 + PT2 |
| Rel-17 RedCap | -7.8 | -5.3 | -2.1 | -2 | 2 |  | 0% (Ref) |  |  | 0% (Ref) |  |  |
| BW1/BW2 | -3.2 | 3.2 | 3.8 | 8.4 | 8.9 |  | 8.2% | 10.5% | 10.9% | 8.8% | 10.7% | 11.3% |
| BW3 | -7.8 | -5.3 | -2.1 | 8.4 | 8.9 |  | 6.9% | 8.9% | 9.6% | 7.2% | 9.1% | 9.8% |
| PR1/PR2 | -7.8 | -5.3 | -2.1 | -2 | 2 |  | 2.4% | 4.9% | 5.3% | 2.7% | 5.0% | 5.7% |
| PR3 | -7.8 | -5.3 | -2.1 | -2* | 2* |  | 5.8% | 8.0% | 8.5% | 6.2% | 8.1% | 8.6% |

* PDSCH processing time of more than one slot is allowed

Table 12: Summary for performance and complexity reduction trade-off for RedCap UE in FDD (15 kHz SCS)

| Performance: Required SNR (dB) |  |  |  |  |  |  | Complexity reduction w.r.t Rel-17 ReCap (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option | PBCH | PDCCH CSS |  | SIB1 |  |  | 1RX |  |  | 2RX |  |  |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |  | w/o PT1/2 | + PT1 | + PT1 + PT2 | w/o PT1/2 | + PT1 | + PT1 + PT2 |
| Rel-17 RedCap | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |  | 0% (Ref) |  |  | 0% (Ref) |  |  |
| BW1/BW2 | -4.8 | -1.5 | -1.6 | 2.2 | 2.3 |  | 8.6% | 10.5% | 11.1% | 9.5% | 11.2% | 12.0% |
| BW3 | -4.8 | -4.5 | -1.6 | 2.2 | 2.3 |  | 7.1% | 8.8% | 9.5% | 7.8% | 9.5% | 10.2% |
| PR1/PR2 | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |  | 3.2% | 5.1% | 5.8% | 3.6% | 5.5% | 6.3% |
| PR3 | -4.8 | -4.5 | -1.6 | -1.6* | 2.3 |  | 6.0% | 7.8% | 8.3% | 6.5% | 8.2% | 9.0% |

* PDSCH processing time of more than one slot is allowed

# Conclusions

In this contribution, we have made the following observations and proposals:

Observation 1: RF components can achieve no complexity reduction with bandwidth reduction.

- No complexity difference between BW1 and BW2

- Narrower bandwidth can even require larger capacitor size and slightly increase the complexity

Observation 2: Bandwidth reduction cannot achieve complexity reduction in the following modem components:

- DL control processing & decoder: Same maximum Poler codeword size and no critical timing relaxation

- Synchronization/cell search block: Same SSB processing

- MIMO specific processing blocks: Same MIMO and CSI functionality as reference Rel-17 RedCap UE

Observation 3: To accommodate 20MHz BW for non-PDSCH/PUSCH channels, BW3 cannot achieve complexity reduction in ADC/DAC and FFT/IFFT for BB

Table 1: Which modem components can achieve complexity reduction with bandwidth reduction

| Modem Components | Rel-15 Ref | Rel-17 RedCap (TDD 1RX) | BW1/BW2 | BW3 |
| --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 25.00% | ## No complexity difference below 20 MHz BW | ## No complexity difference below 20 MHz BW |
| ## RF: Filters | ## 15% | ## 3.75% |  |  |
| ## RF: Transceiver (including LNAs, mixer, and local oscillator) | ## 55% | ## 27.50% |  |  |
| ## RF: Duplexer / Switch | ## 5% | ## 5.00% |  |  |
| # RF: Total | # 100% | # 61.3% |  |  |
| ## BB: ADC / DAC | ## 9% | ## 1.8% |  |  |
| ## BB: FFT/IFFT | ## 4% | ## 0.8% |  |  |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.0% |  |  |
| ## BB: Receiver processing block | ## 29% | ## 8.7% |  |  |
| ## BB: LDPC decoding | ## 9% | ## 1.6% |  |  |
| ## BB: HARQ buffer | ## 12% | ## 2.2% |  |  |
| ## BB: DL control processing & decoder | ## 4% | ## 4.0% |  |  |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.5% |  |  |
| ## BB: UL processing block | ## 5% | ## 4.0% |  |  |
| ## BB: MIMO specific processing blocks | ## 9% | ## 4.5% |  |  |
| # BB: Total | # 100% | # 34.1% |  |  |
| RF+BB: Total (with RF:BB cost split 40:60) | 100% | 44.9% |  |  |

Observation 4: Only limited complexity reduction, up to ~10%, can be achieved w.r.t. Rel-17 RedCap UE by bandwidth reduction options.

- For each component in BB, fundamental area overhead (due to at least control registers and Place-and-Route) cause saturation effect in further complexity reduction w.r.t. Rel-17 RedCap modem.

Table 2: Complexity reduction summary for bandwidth reduction options w.r.t Rel-17 RedCap UE

| Option | TDD with 1Rx | FDD with 1Rx | HD-FDD with 1Rx | TDD with 2Rx | FDD with 2Rx | HD-FDD with 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| BW1/BW2 | 8.2% | 8.6% | 9.6% | 8.8% | 9.5% | 10.3% |
| BW3 | 6.9% | 7.1% | 7.9% | 7.2% | 7.8% | 8.5% |

Observation 5: For FDD (15 kHz SCS), there is around 3-4 dB performance loss for PDCCH CSS and/or SIB1 if there is RE truncation due to larger RB number than UE reception bandwidth.

Observation 6: For FDD (15 kHz SCS), reducing RB number of PDCCH CSS or SIB1 to be 24 RB also causes performance loss of 3-4 dB due to increased code rate.

Table 3: Required SNR (dB) for common channels with BW1/BW2/BW3 in FDD (15 kHz SCS) case

| FDD (15 kHz SCS) | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |
| R17 RedCap | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |
| BW1/BW2 (25 RB) | -4.8 | -1.5 | -1.6 | 2.2 | 2.3 |
| BW3 (5M PDSCH/PUSCH; 25 RB) | -4.8 | -4.5 | -1.6 | 2.2 | 2.3 |

Observation 7: For TDD (30 kHz SCS), there is significant performance loss due to RE truncation. SIB1 is the worst cahnnel, and there is >10 dB (>6 dB) loss due to RE truncation from 48 RBs (24 RBs) to 5MHz bandwidth.

Observation 8: For TDD (30 kHz SCS), reducing RB number of PDCCH CSS or SIB1 to 24 RBs (for less RE truncation) cannot achieve better performance due to increased code rate.

Observation 9: Support 12 RBs for 5MHz reception in 30 kHz SCS can bring ~1 dB benefit w.r.t. 11-RB setting

Table 4: Required SNR (dB) for common channels with BW1/BW2/BW3 in TDD (30 kHz SCS) case

| TDD (30 kHz SCS) | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |
| R17 RedCap | -7.8 | -5.3 | -2.1 | -2 | 2 |
| BW1/BW2 (12 RB) | -3.2 | 3.2 | 3.8 | 8.4 | 8.9 |
| BW1/BW2 (11 RB) | -3.2 | 3.9 | 4.5 | 9.5 | 10.3 |
| BW3 (5M PDSCH/PUSCH; 12 RB) | -7.8 | -5.3 | -2.1 | 8.4 | 8.9 |
| BW3 (5M PDSCH/PUSCH; 11 RB) | -7.8 | -5.3 | -2.1 | 9.5 | 10.3 |

Observation 10: SIB1 is the bottleneck common channel for bandwidth reduction options. While SIB1 performance can be compensated by combing multiple repetitions, the combined performance is bounded by PDCCH CSS performance since UE cannot decode PDSCH without the scheduling DCI information.

Observation 11: Options BW1 and BW2 prevent 5MHz UE from compensating SIB1 performance due to the inferior PDCCH CSS performance caused by RE truncation.

Proposal 1: Considering option BW1/BW2/BW3 for FDD (15 kHz SCS) and TDD (30 kHz SCS), BW3 with 12-RB reception bandwidth is relatively recommended because of achieving complexity reduction while being able to compensate SIB1 performance loss, assuming UE is allowed to combine sufficient SIB1 repetitions.

- Note: There may still require long delay to combine more than 16 SIB1 repetitions for compensating ~12 dB performance loss w.r.t. Rel-17 RedCap UE.

Observation 12: Regarding RF retuning solution to enable “smart” combing of SIB1, such operation requires UE to buffer PDSCH larger than 5MHz BW before decoding the combined SIB1, which effectively increases BW3 complexity to be the same as PR3.

- Note: Combing with RF retuning is different from HARQ combining since a truncated SIB1 is not any valid RV of SIB1 LDPC codeword.

Proposal 2: Combing with RF retuning is not considered for bandwidth reduction options.

Observation 13: PR1/PR2 can only achieve complexity reduction in the following modem BB components:

- LDPC decoding: Reduced peak data rate

- HARQ buffer: Reduced maximum TBS

Observation 14: PR3 can additionally achieve complexity reduction in the following modem BB components:

- Receiver processing block: RB number limit reduces (inner) receiver complexity

- UL processing block: RB number limit reduces total UL processing complexity

Observation 15: Compared with BW3, PR3 is assumed to buffer PDSCH of 20 MHz BW since, before DCI is decoded, UE has no knowledge of PDSCH RB allocation.

Table 5: Which modem components can achieve complexity reduction with peak-data-rate reduction

| Modem Components | Rel-15 Ref | Rel-17 RedCap (TDD 1RX) | PR1/PR2 | PR3 |
| --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 25.00% | ## No complexity reduction | ## No complexity reduction |
| ## RF: Filters | ## 15% | ## 3.75% |  |  |
| ## RF: Transceiver (including LNAs, mixer, and local oscillator) | ## 55% | ## 27.50% |  |  |
| ## RF: Duplexer / Switch | ## 5% | ## 5.00% |  |  |
| # RF: Total | # 100% | # 61.3% |  |  |
| ## BB: ADC / DAC | ## 9% | ## 1.8% |  |  |
| ## BB: FFT/IFFT | ## 4% | ## 0.8% |  |  |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.0% |  |  |
| ## BB: Receiver processing block | ## 29% | ## 8.7% |  |  |
| ## BB: LDPC decoding | ## 9% | ## 1.6% |  |  |
| ## BB: HARQ buffer | ## 12% | ## 2.2% |  |  |
| ## BB: DL control processing & decoder | ## 4% | ## 4.0% |  |  |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.5% |  |  |
| ## BB: UL processing block | ## 5% | ## 4.0% |  |  |
| ## BB: MIMO specific processing blocks | ## 9% | ## 4.5% |  |  |
| # BB: Total | # 100% | # 34.1% |  |  |
| RF+BB: Total (with RF:BB cost split 40:60) | 100% | 44.9% |  |  |

Observation 16: Only limited complexity reduction, up to 7%, can be achieved w.r.t. Rel-17 RedCap UE by peak-data-rate reduction options.

- PR1/PR2 can even achieve <4% complexity reduction

Table 6: Complexity reduction summary for peak-data-rate reduction options w.r.t Rel-17 RedCap UE

| Option | TDD with 1Rx | FDD with 1Rx | HD-FDD with 1Rx | TDD with 2Rx | FDD with 2Rx | HD-FDD with 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| PR1: | 2.4% | 3.2% | 3.5% | 2.7% | 3.6% | 3.9% |
| PR2: | 2.4% | 3.2% | 3.5% | 2.7% | 3.6% | 3.9% |
| PR3: | 5.8% | 6.0% | 6.6% | 6.2% | 6.5% | 7.0% |

Observation 17: Peak-data-rate reduction options can achieve the same performance as Rel-17 RedCap UE

Observation 18: For PR3, support processing 12 RBs in 30 kHz SCS can bring ~1 dB benefit w.r.t. 11-RB limit

Observation 19: PR3 can buffer whole SIB1. But the reduced complexity in “receiver processing block” requires more slots to accomplish SIB1 processing. The same SIB1 performance can be achieved if SIB1 processing time of more than one slot can be allowed.

- Note: RF retuning based combining is not required for PR3

Proposal 3: Jointly considering complexity reduction and performance impact over peak-data-rate reduction options, PR3 is relatively recommended for the better complexity reduction while being able to achieve the same common channel performance as Rel-17 RedCap, assuming PDSCH processing time of more than one slot is allowed for SIB1.

- Note: Up to 2 (4) slots will be sufficient for FDD (TDD) cases

Proposal 4: SIB1 processing time of more than one slot is allowed for PR3

- FFS: Extension to other PDSCH channels

Table 7: Required SNR (dB) for common channels with PR1/PR2/PR3 in FDD (15 kHz SCS) case

| FDD (15 kHz SCS) | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |
| R17 RedCap | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |
| PR1/PR2 (Data rate limit) | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |
| PR3 (25-RB limit on PDSCH/PUSCH) | -4.8 | -4.5 | -1.6 | 2.2 or -1.6* | 2.3 |

* PDSCH processing time of more than one slot is allowed

Table 8: Required SNR (dB) for common channels with PR1/PR2/PR3 in TDD (30 kHz SCS) case

| TDD (30 kHz SCS) | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |
| R17 RedCap | -7.8 | -5.3 | -2.1 | -2 | 2 |
| PR1/PR2 (Data rate limit) | -7.8 | -5.3 | -2.1 | -2 | 2 |
| PR3 (12-RB limit on PDSCH/PUSCH) | -7.8 | -5.3 | -2.1 | 8.4 or -2* | 8.9 or 2* |
| PR3 (11-RB limit on PDSCH/PUSCH) | -7.8 | -5.3 | -2.1 | 9.5 or -2* | 10.3 or 2* |

* PDSCH processing time of more than one slot is allowed

Observation 20: PT1 is useful to additionally reduce the complexity for the following modem BB components by allowing longer processing time:

- Receiver processing block

- LDPC decoding

- DL control processing & decoder

- UL processing block

Observation 21: PT1 + PT2 can additionally achieve complexity reduction in MIMO specific processing blocks by allowing longer CSI related processing time.

Observation 22: PT1 (and PT2) can additionally contributed complexity reduction in “DL control processing & decoder” (and “MIMO specific processing blocks”), which cannot be achieved with BW3 or PR3.

Table 9: Which modem components can achieve additional complexity reduction with processing time relaxation

| Modem Components | Rel-15 Ref | Rel-17 RedCap (TDD 1RX) | PT1 | PT1+PT2 |
| --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25% | ## 25.00% | ## No complexity reduction | ## No complexity reduction |
| ## RF: Filters | ## 15% | ## 3.75% |  |  |
| ## RF: Transceiver (including LNAs, mixer, and local oscillator) | ## 55% | ## 27.50% |  |  |
| ## RF: Duplexer / Switch | ## 5% | ## 5.00% |  |  |
| # RF: Total | # 100% | # 61.3% |  |  |
| ## BB: ADC / DAC | ## 9% | ## 1.8% |  |  |
| ## BB: FFT/IFFT | ## 4% | ## 0.8% |  |  |
| ## BB: Post-FFT data buffering | ## 10% | ## 2.0% |  |  |
| ## BB: Receiver processing block | ## 29% | ## 8.7% |  |  |
| ## BB: LDPC decoding | ## 9% | ## 1.6% |  |  |
| ## BB: HARQ buffer | ## 12% | ## 2.2% |  |  |
| ## BB: DL control processing & decoder | ## 4% | ## 4.0% |  |  |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.5% |  |  |
| ## BB: UL processing block | ## 5% | ## 4.0% |  |  |
| ## BB: MIMO specific processing blocks | ## 9% | ## 4.5% |  |  |
| # BB: Total | # 100% | # 34.1% |  |  |
| RF+BB: Total (with RF:BB cost split 40:60) | 100% | 44.9% |  |  |

Observation 23: With PT1 + PT2, the combined complexity reduction with BW3 can be improved to 11%, which is 1% better than BW1/BW2-only that induces significant performance loss w.r.t. Rel-17 RedCap UE.

Observation 24: With PT1 + PT2, the combined complexity reduction with BW3 can be improved to ~10%, which is comparable to BW1/BW2-only that induces significant performance loss w.r.t. Rel-17 RedCap UE.

Table 10: Complexity reduction summary for combined complexity reduction options w.r.t Rel-17 RedCap UE

| Combined Options | TDD with 1Rx | FDD with 1Rx | HD-FDD with 1Rx | TDD with 2Rx | FDD with 2Rx | HD-FDD with 2Rx |
| --- | --- | --- | --- | --- | --- | --- |
| BW1+PT1 | 10.5% | 10.5% | 11.6% | 10.7% | 11.2% | 12.2% |
| BW1+PT1+PT2 | 10.9% | 11.1% | 12.4% | 11.3% | 12.0% | 13.0% |
| BW2+PT1+PT2 | 10.9% | 11.1% | 12.4% | 11.3% | 12.0% | 13.0% |
| BW3+PT1 | 8.9% | 8.8% | 9.8% | 9.1% | 9.5% | 10.3% |
| BW3+PT1+PT2 | 9.6% | 9.5% | 10.5% | 9.8% | 10.2% | 11.0% |
| PR1+PT1 | 4.9% | 5.1% | 5.7% | 5.0% | 5.5% | 6.0% |
| PR1+PT1+PT2 | 5.3% | 5.8% | 6.5% | 5.7% | 6.3% | 6.9% |
| PR2+PT1+PT2 | 5.3% | 5.8% | 6.5% | 5.7% | 6.3% | 6.9% |
| PR3+PT1 | 8.0% | 7.8% | 8.7% | 8.1% | 8.2% | 8.9% |
| PR3+PT1+PT2 | 8.5% | 8.3% | 9.2% | 8.6% | 9.0% | 9.7% |

Proposal 5: To achieve the benefit be processing time relaxation while minimizing network scheduling impact, the following two options can be considered for further complexity reduction on RedCap UEs:

- Alt1: N1 and N2 are extended by [7] symbols

- Alt2: PDSCH/PUSCH processing time of more than one slots is allowed

Figure 1: Illustration of complexity reduction by allowing PDSCH processing time of more than one slot

Conclusion 1: There can achieve up to ~12% complexity reduction w.r.t. Rel-17 RedCap UE based on the options of bandwidth reduction, peak-data-rate reduction or combinations with processing time relaxation.

- Subject to no performance loss w.r.t. Rel-17 RedCap UE, only up to ~8% complexity reduction

Conclusion 2: Targeting minimum impact to legacy UEs and minimum performance loss w.r.t. Rel-17 RedCap UE, the following combined complexity reduction options can be considered for potential specification in Rel-18:

- Peak-data-rate reduction option PR3 and processing time relaxation option PT1 + PT2:

  - PDSCH/PUSCH #RB up to 12 RBs (25 RBs) for TDD/30 kHz SCS (FDD/15 kHz SCS)

  - PDSCH processing time of more than one slot is allowed

    - FFS: Whether to extend N1 and N2 by [7] symbols

  - CSI processing time of more slot(s) is allowed

Table 11: Summary for performance and complexity reduction trade-off for RedCap UE in TDD (30 kHz SCS)

| Performance: Required SNR (dB) |  |  |  |  |  |  | Complexity reduction w.r.t Rel-17 ReCap (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option | PBCH | PDCCH CSS |  | SIB1 |  |  | 1RX |  |  | 2RX |  |  |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |  | w/o PT1/2 | + PT1 | + PT1 + PT2 | w/o PT1/2 | + PT1 | + PT1 + PT2 |
| Rel-17 RedCap | -7.8 | -5.3 | -2.1 | -2 | 2 |  | 0% (Ref) |  |  | 0% (Ref) |  |  |
| BW1/BW2 | -3.2 | 3.2 | 3.8 | 8.4 | 8.9 |  | 8.2% | 10.5% | 10.9% | 8.8% | 10.7% | 11.3% |
| BW3 | -7.8 | -5.3 | -2.1 | 8.4 | 8.9 |  | 6.9% | 8.9% | 9.6% | 7.2% | 9.1% | 9.8% |
| PR1/PR2 | -7.8 | -5.3 | -2.1 | -2 | 2 |  | 2.4% | 4.9% | 5.3% | 2.7% | 5.0% | 5.7% |
| PR3 | -7.8 | -5.3 | -2.1 | -2* | 2* |  | 5.8% | 8.0% | 8.5% | 6.2% | 8.1% | 8.6% |

* PDSCH processing time of more than one slot is allowed

Table 12: Summary for performance and complexity reduction trade-off for RedCap UE in FDD (15 kHz SCS)

| Performance: Required SNR (dB) |  |  |  |  |  |  | Complexity reduction w.r.t Rel-17 ReCap (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option | PBCH | PDCCH CSS |  | SIB1 |  |  | 1RX |  |  | 2RX |  |  |
|  |  | 48RB /AL16 | 24RB /AL8 | 48RB /MCS0 | 24RB /MCS3 |  | w/o PT1/2 | + PT1 | + PT1 + PT2 | w/o PT1/2 | + PT1 | + PT1 + PT2 |
| Rel-17 RedCap | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |  | 0% (Ref) |  |  | 0% (Ref) |  |  |
| BW1/BW2 | -4.8 | -1.5 | -1.6 | 2.2 | 2.3 |  | 8.6% | 10.5% | 11.1% | 9.5% | 11.2% | 12.0% |
| BW3 | -4.8 | -4.5 | -1.6 | 2.2 | 2.3 |  | 7.1% | 8.8% | 9.5% | 7.8% | 9.5% | 10.2% |
| PR1/PR2 | -4.8 | -4.5 | -1.6 | -1.6 | 2.3 |  | 3.2% | 5.1% | 5.8% | 3.6% | 5.5% | 6.3% |
| PR3 | -4.8 | -4.5 | -1.6 | -1.6* | 2.3 |  | 6.0% | 7.8% | 8.3% | 6.5% | 8.2% | 9.0% |

* PDSCH processing time of more than one slot is allowed

# References

- RP-213661, “New SID on Study on further NR RedCap UE complexity reduction,” Ericsson, 3GPP TSG RAN meeting #94e, December 2021

- Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0; online available: https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_109-e/Report/Draft_Minutes_report_RAN1%23109-e_v030.zip

- R1-2207003, “Evaluation of coverage impact for Rel-18 RedCap”, MediaTek, RAN1#110

- TR 38.875, “Study on support of reduced capability NR devices, Rel-17”

# Appendix A: Detailed complexity reduction analysis

In this section, more detailed UE cost reduction analysis is provided for different options where the total cost/reduction is calculated assuming the cost ratio of BB to RF is 6:4 in FR1 as in [3].

## A.1: Bandwidth reduction options

Table 13: TDD with 1Rx in FR1

| UE BW reduction Option | TDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE in TDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE | 20 MHz, 1 layer, 1 Rx, 64QAM, TDD | 38.7% | 65.9% | 55.1% | 0.0% |
| Option BW1 | 5 MHz for both BB and RF, 1 layer, 1 Rx, 64QAM, TDD | 38.7% | 72.2% | 58.8% | 8.2% |
| Option BW2 | 20 MHz for RF and 5 MHz for BB, 1 layer, 1 Rx, 64QAM, TDD | 38.7% | 72.2% | 58.8% | 8.2% |
| Option BW3 | 20 MHz for RF and control channels and reference signals, 5MHz for PDSCH (unicast and broadcast) and PUSCH, 1 layer, 1 Rx, 64QAM, TDD | 38.7% | 71.2% | 58.2% | 6.9% |

Table 14: FDD with 1Rx in FR1

| UE BW reduction Option | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE with FDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE | 20 MHz, 1 layer, 1 Rx, 64QAM, FDD | 16.2% | 55.6% | 39.8% | 0% |
| Option BW1 | 5 MHz for both BB and RF, 1 layer, 1 Rx, 64QAM, FDD | 16.2% | 64.2% | 45.0% | 8.6% |
| Option BW2 | 20 MHz for RF and 5 MHz for BB, 1 layer, 1 Rx, 64QAM, FDD | 16.2% | 64.2% | 45.0% | 8.6% |
| Option BW3 | 20 MHz for RF and control channels and reference signals, 5MHz for PDSCH (unicast and broadcast) and PUSCH, 1 layer, 1 Rx, 64QAM, FDD | 16.2% | 62.6% | 44.1% | 7.1% |

Table 15: HD-FDD with 1Rx in FR1

| UE BW reduction Option | HD-FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE with HD-FDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with HD-FDD | 20 MHz, 1 layer, 1 Rx, 64QAM, HD-FDD | 31.2% | 55.6% | 45.8% | 0% |
| Option BW1 | 5 MHz for both BB and RF, 1 layer, 1 Rx, 64QAM, HD-FDD | 31.2% | 64.2% | 51.0% | 9.6% |
| Option BW2 | 20 MHz for RF and 5 MHz for BB, 1 layer, 1 Rx, 64QAM, HD-FDD | 31.2% | 64.2% | 51.0% | 9.6% |
| Option BW3 | 20 MHz for RF and control channels and reference signals, 5MHz for PDSCH (unicast and broadcast) and PUSCH, 1 layer, 1 Rx, 64QAM, HD-FDD | 31.2% | 62.6% | 50.1% | 7.9% |

Table 16: TDD with 2Rx in FR1

| UE BW reduction Option | TDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE in TDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE | 20 MHz, 1 layer, 2 Rx, 64QAM, TDD | 21.2% | 55.5% | 41.8% | 0% |
| Option BW1 | 5 MHz for both BB and RF, 1 layer, 2 Rx, 64QAM, TDD | 21.2% | 64.0% | 46.9% | 8.8% |
| Option BW2 | 20 MHz for RF and 5 MHz for BB, 1 layer, 2 Rx, 64QAM, TDD | 21.2% | 64.0% | 46.9% | 8.8% |
| Option BW3 | 20 MHz for RF and control channels and reference signals, 5MHz for PDSCH (unicast and broadcast) and PUSCH, 1 layer, 2 Rx, 64QAM, TDD | 21.2% | 62.6% | 46.0% | 7.2% |

Table 17: FDD with 2Rx in FR1

| UE BW reduction Option | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE with FDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE | 20 MHz, 1 layer, 2 Rx, 64QAM, FDD | 0% | 40.4% | 24.2% | 0% |
| Option BW1 | 5 MHz for both BB and RF, 1 layer, 2 Rx, 64QAM, FDD | 0% | 52.3% | 31.4% | 9.5% |
| Option BW2 | 20 MHz for RF and 5 MHz for BB, 1 layer, 2 Rx, 64QAM, FDD | 0% | 52.3% | 31.4% | 9.5% |
| Option BW3 | 20 MHz for RF and control channels and reference signals, 5MHz for PDSCH (unicast and broadcast) and PUSCH, 1 layer, 2 Rx, 64QAM, FDD | 0% | 50.1% | 30.1% | 7.8% |

Table 18: HD-FDD with 2Rx in FR1

| UE BW reduction Option | HD-FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE with HD-FDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with HD-FDD | 20 MHz, 1 layer, 2 Rx, 64QAM, HD-FDD | 15.0% | 40.4% | 30.2% | 0% |
| Option BW1 | 5 MHz for both BB and RF, 1 layer, 2 Rx, 64QAM, HD-FDD | 15.0% | 52.3% | 37.4% | 10.3% |
| Option BW2 | 20 MHz for RF and 5 MHz for BB, 1 layer, 2 Rx, 64QAM, HD-FDD | 15.0% | 52.3% | 37.4% | 10.3% |
| Option BW3 | 20 MHz for RF and control channels and reference signals, 5MHz for PDSCH (unicast and broadcast) and PUSCH, 1 layer, 2 Rx, 64QAM, HD-FDD | 15.0% | 50.1% | 36.1% | 8.5% |

## A.2: Peak data rate reduction options

Table 19: TDD with 1Rx in FR1

| Peak data rate reduction option | TDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE in TDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE | 20 MHz, 1 layer, 1 Rx, 64QAM, TDD | 38.7% | 65.9% | 55.1% | 0.0% |
| Option PR1 |  | 38.7% | 67.9% | 56.2% | 2.4% |
| Option PR2 |  | 38.7% | 67.9% | 56.2% | 2.4% |
| Option PR3 |  | 38.7% | 70.4% | 57.7% | 5.8% |

Table 20: FDD with 1Rx in FR1

| Peak data rate reduction option | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE with FDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE | 20 MHz, 1 layer, 1 Rx, 64QAM, FDD | 16.2% | 55.6% | 39.8% | 0.0% |
| Option PR1 |  | 16.2% | 58.7% | 41.7% | 3.2% |
| Option PR2 |  | 16.2% | 58.7% | 41.7% | 3.2% |
| Option PR3 |  | 16.2% | 61.5% | 43.4% | 6.0% |

Table 21: HD-FDD with 1Rx in FR1

| Peak data rate reduction option | HD-FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE with HD-FDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with HD-FDD | 20 MHz, 1 layer, 1 Rx, 64QAM, HD-FDD | 31.2% | 55.6% | 45.8% | 0.0% |
| Option PR1 |  | 31.2% | 58.7% | 47.7% | 3.5% |
| Option PR2 |  | 31.2% | 58.7% | 47.7% | 3.5% |
| Option PR3 |  | 31.2% | 61.5% | 49.4% | 6.6% |

Table 22: TDD with 2Rx in FR1

| Peak data rate reduction option | TDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE in TDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE | 20 MHz, 1 layer, 2 Rx, 64QAM, TDD | 21.2% | 55.5% | 41.8% | 0.0% |
| Option PR1 |  | 21.2% | 58.2% | 43.4% | 2.7% |
| Option PR2 |  | 21.2% | 58.2% | 43.4% | 2.7% |
| Option PR3 |  | 21.2% | 61.4% | 45.4% | 6.2% |

Table 23: FDD with 2Rx in FR1

| Peak data rate reduction option | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE with FDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE | 20 MHz, 1 layer, 2 Rx, 64QAM, FDD | 0% | 40.4% | 24.2% | 0.0% |
| Option PR1 |  | 0% | 44.8% | 26.9% | 3.6% |
| Option PR2 |  | 0% | 44.8% | 26.9% | 3.6% |
| Option PR3 |  | 0% | 48.5% | 29.1% | 6.5% |

Table 24: HD-FDD with 2Rx in FR1

| Peak data rate reduction option | HD-FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE with HD-FDD |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with HD-FDD | 20 MHz, 1 layer, 2 Rx, 64QAM, HD-FDD | 15.0% | 40.4% | 30.2% | 0.0% |
| Option PR1 |  | 15.0% | 44.8% | 32.9% | 3.9% |
| Option PR2 |  | 15.0% | 44.8% | 32.9% | 3.9% |
| Option PR3 |  | 15.0% | 48.5% | 35.1% | 7.0% |

## A.3: Combinations of options

Table 25: TDD-1Rx, detailed cost reduction with different combination of options compared to R17 RedCap UE.

| Combination of options | TDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with TDD, 1Rx | 20 MHz, 1 layer, 1 Rx, 64QAM, TDD | 38.7% | 65.9% | 55.1% | 0.0% |
| BW1+PT1 |  | 38.7% | 73.8% | 59.8% | 10.5% |
| BW1+PT1+PT2 |  | 38.7% | 74.2% | 60.0% | 10.9% |
| BW3+PT1 |  | 38.7% | 72.7% | 59.1% | 8.9% |
| BW3+PT1+PT2 |  | 38.7% | 73.2% | 59.4% | 9.6% |
| PR1+PT1 |  | 38.7% | 69.6% | 57.3% | 4.9% |
| PR1+PT1+PT2 |  | 38.7% | 70.1% | 57.5% | 5.3% |
| PR2+PT1+PT2 |  | 38.7% | 70.1% | 57.5% | 5.3% |
| PR3+PT1 |  | 38.7% | 71.9% | 58.7% | 8.0% |
| PR3+PT1+PT2 |  | 38.7% | 72.4% | 58.9% | 8.5% |

Table 26: FDD-1Rx, detailed cost reduction with different combination of options compared to R17 RedCap UE

| Combination of options | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with FDD, 1Rx | 20 MHz, 1 layer, 1 Rx, 64QAM, FDD | 16.2% | 55.6% | 39.8% | 0.0% |
| BW1+PT1 |  | 16.2% | 66.0% | 46.1% | 10.5% |
| BW1+PT1+PT2 |  | 16.2% | 66.6% | 46.5% | 11.1% |
| BW3+PT1 |  | 16.2% | 64.4% | 45.1% | 8.8% |
| BW3+PT1+PT2 |  | 16.2% | 65.0% | 45.5% | 9.5% |
| PR1+PT1 |  | 16.2% | 60.7% | 42.9% | 5.1% |
| PR1+PT1+PT2 |  | 16.2% | 61.4% | 43.3% | 5.8% |
| PR2+PT1+PT2 |  | 16.2% | 61.4% | 43.3% | 5.8% |
| PR3+PT1 |  | 16.2% | 63.3% | 44.5% | 7.8% |
| PR3+PT1+PT2 |  | 16.2% | 63.9% | 44.8% | 8.3% |

Table 27: HD-FDD-1Rx, detailed cost reduction with different combination of options compared to R17 RedCap UE

| Combination of options | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with HD-FDD, 1Rx | 20 MHz, 1 layer, 1 Rx, 64QAM, HD-FDD | 31.2% | 55.6% | 45.8% | 0.0% |
| BW1+PT1 |  | 31.2% | 66.0% | 52.1% | 11.6% |
| BW1+PT1+PT2 |  | 31.2% | 66.6% | 52.5% | 12.4% |
| BW3+PT1 |  | 31.2% | 64.4% | 51.1% | 9.8% |
| BW3+PT1+PT2 |  | 31.2% | 65.0% | 51.5% | 10.5% |
| PR1+PT1 |  | 31.2% | 60.7% | 48.9% | 5.7% |
| PR1+PT1+PT2 |  | 31.2% | 61.4% | 49.3% | 6.5% |
| PR2+PT1+PT2 |  | 31.2% | 61.4% | 49.3% | 6.5% |
| PR3+PT1 |  | 31.2% | 63.3% | 50.5% | 8.7% |
| PR3+PT1+PT2 |  | 31.2% | 63.9% | 50.8% | 9.2% |

Table 28: TDD-2Rx, detailed cost reduction with different combination of options compared to R17 RedCap UE

| Combination of options | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with TDD, 2Rx | 20 MHz, 1 layer, 2 Rx, 64QAM, TDD | 21.2% | 55.5% | 41.8% | 0.0% |
| BW1+PT1 |  | 21.2% | 65.9% | 48.0% | 10.7% |
| BW1+PT1+PT2 |  | 21.2% | 66.5% | 48.4% | 11.3% |
| BW3+PT1 |  | 21.2% | 64.4% | 47.1% | 9.1% |
| BW3+PT1+PT2 |  | 21.2% | 65.0% | 47.5% | 9.8% |
| PR1+PT1 |  | 21.2% | 60.4% | 44.7% | 5.0% |
| PR1+PT1+PT2 |  | 21.2% | 61.0% | 45.1% | 5.7% |
| PR2+PT1+PT2 |  | 21.2% | 61.0% | 45.1% | 5.7% |
| PR3+PT1 |  | 21.2% | 63.3% | 46.5% | 8.1% |
| PR3+PT1+PT2 |  | 21.2% | 63.9% | 46.8% | 8.6% |

Table 29: FDD-2Rx, detailed cost reduction with different combination of options compared to R17 RedCap UE

| Combination of options | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with FDD, 2Rx | 20 MHz, 1 layer, 2 Rx, 64QAM, FDD | 0.0% | 40.4% | 24.2% | 0.0% |
| BW1+PT1 |  | 0.0% | 54.5% | 32.7% | 11.2% |
| BW1+PT1+PT2 |  | 0.0% | 55.4% | 33.3% | 12.0% |
| BW3+PT1 |  | 0.0% | 52.3% | 31.4% | 9.5% |
| BW3+PT1+PT2 |  | 0.0% | 53.2% | 31.9% | 10.2% |
| PR1+PT1 |  | 0.0% | 47.4% | 28.4% | 5.5% |
| PR1+PT1+PT2 |  | 0.0% | 48.3% | 29.0% | 6.3% |
| PR2+PT1+PT2 |  | 0.0% | 48.3% | 29.0% | 6.3% |
| PR3+PT1 |  | 0.0% | 50.7% | 30.4% | 8.2% |
| PR3+PT1+PT2 |  | 0.0% | 51.6% | 31.0% | 9.0% |

Table 30: HD-FDD-2Rx, detailed cost reduction with different combination of options compared to R17 RedCap UE

| Combination of options | FDD UE features in FR1 | RF reduction compared to reference Rel-15 NR UE | BB reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-15 NR UE | Total reduction compared to reference Rel-17 RedCap UE |
| --- | --- | --- | --- | --- | --- |
| Reference Rel-17 RedCap UE with FDD, 2Rx | 20 MHz, 1 layer, 2 Rx, 64QAM, HD-FDD | 15.0% | 40.4% | 30.2% | 0.0% |
| BW1+PT1 |  | 15.0% | 54.5% | 38.7% | 12.2% |
| BW1+PT1+PT2 |  | 15.0% | 55.4% | 39.3% | 13.0% |
| BW3+PT1 |  | 15.0% | 52.3% | 37.4% | 10.3% |
| BW3+PT1+PT2 |  | 15.0% | 53.2% | 37.9% | 11.0% |
| PR1+PT1 |  | 15.0% | 47.4% | 34.4% | 6.0% |
| PR1+PT1+PT2 |  | 15.0% | 48.3% | 35.0% | 6.9% |
| PR2+PT1+PT2 |  | 15.0% | 48.3% | 35.0% | 6.9% |
| PR3+PT1 |  | 15.0% | 50.7% | 36.4% | 8.9% |
| PR3+PT1+PT2 |  | 15.0% | 51.6% | 37.0% | 9.7% |