3GPP TSG RAN WG1 #110         	                                                 R1-2207003 Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.6.2

Source: 	MediaTek Inc.

Title: 	Evaluation of coverage impact for Rel-18 RedCap

Document for:	Discussion and decision

# Introduction

A new study item on further NR UE complexity reduction was approved with the following objectives 0:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

Furthermore, following agreements have been made at RAN1 #109e meeting about simulation assumptions and coverage evaluation assumptions.

Agreement - At least the option of RF+BB BW reduction to 5MHz is considered for coverage evaluation - FFS whether/which other options are also considered - FFS which DL/UL Channels of all the DL/UL channels are evaluated Agreement - Evaluation methodology and assumption in Clause 6.3 in TR 38.875 is reused for coverage evaluation of reference UE and Rel-17 RedCap UE. - Note: It is up to each company whether to reuse the LLS results Agreement - Coverage for the following channels is evaluated for “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels” - SIB1 - PBCH - PDCCH CSS - [Msg4] - Following channels can be optionally evaluated - PUSCH - PUCCH 2bits - PUCCH 11bits - PUCCH 22bits - PRACH - PDSCH - PDCCH USS - Msg2 - Msg3 - Evaluation methodology and assumption in Clause 6.3 in TR 38.875 is reused for coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels” by default, except for, UE bandwidth, cell edge data rate, and small form factor degradation - FFS which evaluation assumption should be updated for the above channels Agreement - Following evaluations are not conducted in Rel-18 RedCap SI - Latency - Throughput - Power saving gain Agreement - Coverage of Msg4 can be optionally evaluated for “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels” Agreement - For coverage evaluation of Rel-17 and Rel-18 RedCap UEs, only 1 Rx branch is assumed. - Note: it does not mean that 2Rx is precluded for Rel-18 RedCap UE Agreement - For coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, following parameters are used. Parameters; FR1 values. UE bandwidth; Rural: 5 MHz (25 PRBs, 15 kHz SCS) Urban: 5 MHz (11 PRBs or 12 PRBs (optional), 30 kHz SCS). - Note: Rural scenario at 0.7 GHz, Urban scenario at 2.6 GHz, and Urban scenario at 4 GHz (optional) are considered. Agreement - For coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, target data rates are - FR1 Rural: 250 kbps on DL and 25 kbps in UL - FR1 Urban: 500 kbps on DL and 250 kbps in UL - Note: The target data rates are the scaled value in the Rel-17 RedCap SI by a factor of 0.25 Agreement - 3dB antenna efficiency loss can be optionally assumed for coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels” Agreement (further updated as shown in red – from May 20th GTW) - For at least PDCCH USS coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, following revision are assumed - For 15KHz SCS, CORESET size is 3 symbols and 24 PRBs, AL is 8. - For 30KHz SCS, - Opt1: CORESET size is 3 symbols and 6 PRBs, AL is 2 (baseline) - Opt2: CORESET size is 3 symbols and 12 PRBs, AL is 4 (optional) FFS：Use all CCEs of the CORESET Other configurations are also not precluded Agreement - For coverage evaluation of Rel-18 RedCap UE, 1 Tx branch is assumed. Conclusion - Evaluation of PDCCH blocking probability is not conducted in Rel-18 RedCap SI Agreement - For SIB1 coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, followings are assumed - Opt1: SIB1 BW is larger than 5MHz, e.g., 48PRB - The UE can receive a part of SIB1 PDSCH at a time. Detail assumption of reception scheme (e.g., puncturing the bits transmitted outside UE BW) is reported by each company. - Opt2: SIB1 BW is within 5MHz - A TBS of 1256 bits(other size is not precluded) Note: whether interleaving mapping is assumed depends on companies’ report Agreement - For PDCCH CSS coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, following revision are assumed - Opt1: CORESET BW is larger than 5MHz - The UE can receive a part of PDCCH at a time. Detail assumption of reception scheme (e.g., puncturing the bits transmitted outside UE BW) is reported by each company. - For 15/30kHz SCS, CORESET size is 2 symbols and 48 PRBs, AL is 16. - For 30kHz SCS, CORESET size is 2 symbols and 24 PRBs, AL is 8. Other configurations are also not precluded - Opt2: CORESET BW is within 5MHz - For 15kHz SCS, CORESET size is 3 symbols and 24 PRBs, AL is 8. - For 30kHz SCS, - Opt2-1: CORESET size is 3 symbols and 6 PRBs, AL is 2. Other configurations are also not precluded - Opt2-2: CORESET size is 3 symbols and 12 PRBs, AL is 4 Agreement - The LLS results of the option of “RF+BB BW reduction to 5MHz for all DL/UL channels” can be reused for the coverage evaluation of other BW reduction options, if applicable. Agreement - For coverage evaluation in Urban scenario at 4 GHz, DL PSD 33 dBm/MHz is baseline and DL PSD 24 dBm/MHz is optional. Agreement - For Msg4 coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, a TBS of 1040 bits is assumed - a TBS smaller than 1040 bits can be optionally evaluated and reported by each company. Agreement - For PRACH coverage evaluation of “Rel-18 RedCap UE with RF+BB BW reduction to 5MHz for all DL/UL channels”, Format 0 is used for Rural scenario and Format B4 is used for Urban scenario - Format C2 can be used optionally. Agreement For Msg2 coverage evaluation of reference UE, Rel-17 RedCap UE, and Rel-18 RedCap UE, A TBS of 72 bits is assumed.

# Link-level simulation assumptions

According to Error! Reference source not found., the following deployment scenarios in FR1 are considered in Rel-18 RedCap SI.

- Rural with the carrier frequency of 0.7 GHz and DL PSD of 36 dBm/MHz

- Urban with the carrier frequency of 2.6 GHz and DL PSD of 33 dBm/MHz

- Urban with the carrier frequency of 4 GHz and DL PSD of 33 dBm/MHz and 24 dBm/MHz

In this contribution, rural with carrier frequency of 0.7 GHz and 15kHz SCS and urban with carrier frequency of 2.6 GHz and 30 kHz SCS are evaluated. The parameters used for evaluation are listed in Section 2.1 and Section 2.2.

## Generic assumptions

In this section, the general parameters used for coverage evaluations are shown in Table 1.

Table 1: General parameters for coverage evaluation simulation

| Parameters | Value |
| --- | --- |
| Scenario and frequency | Rural: 700MHz (FDD) Urban: 2.6GHz (TDD) 4GHz (TDD) |
| SCS | Rural: 15kHz Urban: 30kHz |
| Frame structure for TDD | DDDDDDDSUU (S: 6D:4G:4U) for 2.6GHz |
| Channel model | TDL-C, NLoS |
| Delay spread | 300ns |
| UE velocity | 3km/hr |
| # of gNB TX chains | Rural (15kHz SCS): 2; Urban (30kHz SCS): 4 |
| # of gNB RX chains | Rural (15kHz SCS): 2; Urban (30kHz SCS): 4 |

The other general assumptions, which are common to all physical channel, are based on UE capability. To estimate and compare the coverage impact of Rel-18 (5MHz) RedCap UE, not only Rel-15 NR UE but also Rel-17 RedCap UE are evaluated in this contribution. The general assumptions are captured in

Table 2, Table 3 and Table 4 for Rel-15 NR UE, Rel-17 RedCap UE and 5MHz RedCap UE, respectively.

Table 2: Assumptions for reference Rel-15 NR UE

| Parameters | FR1 values |
| --- | --- |
| # UE Tx chains | 1 |
| # UE Rx chains | Rural (15kHz SCS): 2; Urban (30kHz SCS): 4 |
| UE bandwidth | Rural (15kHz SCS): 20 MHz (106 PRBs) Urban (30kHz SCS): 100 MHz (273 PRBs) |

Table 3: Assumptions for reference Rel-17 RedCap UE

| Parameters | FR1 values |
| --- | --- |
| # UE Tx chains | 1 |
| # UE Rx chains | Rural (15kHz SCS): 1; Urban (30kHz SCS): 1 |
| UE bandwidth | Rural (15kHz SCS): 20 MHz (106 PRBs) Urban (30kHz SCS): 20 MHz (51 PRBs) |

Table 4: Assumptions for 5MHz RedCap UE

| Parameters | FR1 values |
| --- | --- |
| # UE Tx chains | 1 |
| # UE Rx chains | Rural (15kHz SCS): 1; Urban (30kHz SCS): 1 |
| UE bandwidth | Rural (15kHz SCS): 5 MHz (25 PRBs) Urban (30kHz SCS): 5 MHz (11 PRBs or 12 PRBs) |

## Channel-specific assumptions

The assumptions for channel specific parameters are also based on reusing the Rel-17 RedCap SI agreements [5], with the revision or addition described below.

The cell-specific parameters for PBCH are listed in Table 5. In this contribution, we evaluate the PBCH w/ and without soft combining. In the case with soft combining, the combing over up to 4 shots are utilized over the same PRBs without performing RF retuning.

Table 5: Channel-specific assumptions for PBCH

| Parameter | Rel-15 NR UE | Rel-17 RedCap UE | 5MHz RedCap UE |
| --- | --- | --- | --- |
| Target BLER | 1% | 1% | 1% |
| SSB periodicity | 20ms | 20ms | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Precoder | Precoder cycling | Precoder cycling | Precoder cycling |

From

Table 6 to Table 9 are the assumptions for PDCCH. Different payload sizes are used for CSS and USS, respectively. In CSS, the payload size is set to 40 bits to simulate DCI format 1_0; In USS, the payload size is set to 56 to estimate DCI format 1_1.

Different CORESET configurations are included. In case of smaller UE reception BW than CORESET frequency span, there will be RE truncation, as indicated in the final row of each table.

Table 6: Channel-specific assumptions for PDCCH CSS (Reference UE)

| Parameter | Rel-15 NR UE | Rel-17 RedCap UE |
| --- | --- | --- |
| Target BLER | 1% | 1% |
| Aggregation level (AL) | 16 | 16 |
| DCI payload size | 40 bits (to simulate DCI format 1_0; + 24 bits for CRC) |  |
| CORESET | 2 symbols x 48 PRBs | 2 symbols x 48 PRBs |
| CCE-to-REG mapping type | Interleaved |  |
| Precoding | Precoder cycling at CCE level; REG bundle=6 | Precoder cycling at CCE level; REG bundle=6 |
| RE Truncation | N | N |

Table 7 Channel-specific assumptions for PDCCH CSS (5MHz RedCap UE)

| Parameter | Config 1 | Config 2 | Config 3 (Urban only) | Config 4 (Urban only) |
| --- | --- | --- | --- | --- |
| Target BLER | 1% | 1% | 1% | 1% |
| Aggregation level (AL) | 16 | 8 | 4 | 2 |
| DCI payload size | 40 bits (to simulate DCI format 1_0; + 24 bits for CRC) |  |  |  |
| CORESET | 2 symbols x 48 PRBs | Rural (15 kHz SCS): 3 symbols x 24 PRBs Urban (30 kHz SCS): 2 symbols x 24 PRBs | 3 symbols x 12 PRBs | 3 symbols x 6 PRBs |
| CCE-to-REG mapping type | Interleaved |  |  |  |
| Precoding | Precoder cycling at CCE level; REG bundle=6 |  |  |  |
| RE Truncation | Y | Rural (15 kHz SCS): N Urban (30 kHz SCS): Y | N | N |

Table 8 Channel-specific assumptions for PDCCH USS (Reference UE)

| Parameter | Rel-15 NR UE | Rel-17 RedCap UE |
| --- | --- | --- |
| Target BLER | 1% | 1% |
| Aggregation level (AL) | 16 | 16 |
| DCI payload size | 56 bits (to simulate DCI format 1_1; + 24 bits for CRC) |  |
| CORESET | 2 symbols x 48 PRBs | 2 symbols x 48 PRBs |
| CCE-to-REG mapping type | Interleaved |  |
| Precoding | Precoder cycling at CCE level; REG bundle=6 | Precoder cycling at CCE level; REG bundle=6 |
| RE Truncation | N | N |

Table 9: Channel-specific assumptions for PDCCH USS (5MHz RedCap UE)

| Parameter | Config 1 (Rural only) | Config 2 (Urban only) | Config 3 (Urban only) |
| --- | --- | --- | --- |
| Target BLER | 1% | 1% | 1% |
| Aggregation level (AL) | 8 | 4 | 2 |
| DCI payload size | 56 bits (to simulate DCI format 1_1; + 24 bits for CRC) |  |  |
| CORESET | 3 symbols x 24 PRBs | 3 symbols x 12 PRBs | 3 symbols x 6 PRBs |
| CCE-to-REG mapping type | Interleaved |  |  |
| Precoding | Precoder cycling at CCE level; REG bundle=6 |  |  |
| RE Truncation | N | UE bandwidth 11PRBs: Y UE bandwidth 12PRBs: N | N |

Table 10 and Table 11 describe the assumption for SIB1, taking into account

- SIB1 RB size can be set to 48 and 24, following CORESET#0 span

- Configurations of 11 and 12 RB are additionally included for Urban (30kHz SCS)

- All MCS values are adjusted to accommodate TBS of 1256 bits

Table 10: Channel-specific assumptions for SIB1 (Reference UE)

| Parameter | Rel-15 NR UE | Rel-17 RedCap UE |
| --- | --- | --- |
| Target BLER | 10% | 10% |
| Waveform | CP-OFDM | CP-OFDM |
| HARQ configuration | No retransmission | No retransmission |
| DMRS configuration | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol |
| PDSCH duration | 12 OFDM symbols | 12 OFDM symbols |
| # of PDSCH RB | 48 | 48 |
| MCS | 0, table 1 (Table 5.1.3.1-1, TS 38.214) | 0, table 1 (Table 5.1.3.1-1, TS 38.214) |
| TBS (bits) | 1256 | 1256 |
| TB scaling factor | 1 | 1 |
| VRB-to-PRB mapping | Interleaved | Interleaved |
| Precoding granularity | 2 RB bundle | 2 RB bundle |

Table 11: Channel-specific assumptions for SIB1 (5MHz RedCap UE)

| Parameter | Config 1 | Config 2 | Config 3 (Urban only) |
| --- | --- | --- | --- |
| Target BLER | 10% | 10% | 10% |
| Waveform | CP-OFDM | CP-OFDM | CP-OFDM |
| HARQ configuration | No retransmission | No retransmission | No retransmission |
| DMRS configuration | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol |
| PDSCH duration | 12 OFDM symbols | 12 OFDM symbols | 12 OFDM symbols |
| # of PDSCH RB | 48 | 24 | 12 or 11 (same as UE Bandwidth) |
| MCS | 0, table 1 (Table 5.1.3.1-1, TS 38.214) | 3, table 1 (Table 5.1.3.1-1, TS 38.214) | 6, table 1 (Table 5.1.3.1-1, TS 38.214) |
| TBS (bits) | 1256 | 1256 | 1256 |
| TB scaling factor | 1 | 1 | 1 |
| VRB-to-PRB mapping | Interleaved | Interleaved | Interleaved |
| Precoding granularity | 2 RB bundle | 2 RB bundle | 2 RB bundle |

Table 12 and Table 13 show the assumptions for Msg2 channel. The settings of (MCS, # of RB) = (0, 3), (0, 6) and (0, 12) for TB scaling of 1, 0.5 and 0.25 respectively.

Table 12: Channel-specific assumptions for Msg2 (Reference UE)

| Parameter | Rel-15 NR UE | Rel-17 RedCap UE |
| --- | --- | --- |
| Target BLER | 10% | 10% |
| Waveform | CP-OFDM | CP-OFDM |
| HARQ configuration | No retransmission | No retransmission |
| DMRS configuration | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol |
| PDSCH duration | 12 OFDM symbols | 12 OFDM symbols |
| # of PDSCH RB | 3 | 3 |
| MCS | 0, table 1 (Table 5.1.3.1-1, TS 38.214) | 0, table 1 (Table 5.1.3.1-1, TS 38.214) |
| TBS (bits) | 72 | 72 |
| TB scaling factor | 1 | 1 |
| VRB-to-PRB mapping | Interleaved | Interleaved |
| Precoding granularity | 2 RB bundle | 2 RB bundle |

Table 13 Channel-specific assumptions for Msg2 (5MHz RedCap UE)

| Parameter | Config 1 | Config 2 | Config 3 (Urban only) |
| --- | --- | --- | --- |
| Target BLER | 10% | 10% | 10% |
| Waveform | CP-OFDM | CP-OFDM | CP-OFDM |
| HARQ configuration | No retransmission | No retransmission | No retransmission |
| DMRS configuration | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol |
| PDSCH duration | 12 OFDM symbols | 12 OFDM symbols | 12 OFDM symbols |
| # of PDSCH RB | 3 | 6 | 12 |
| MCS | 0, table 1 (Table 5.1.3.1-1, TS 38.214) | 3, table 1 (Table 5.1.3.1-1, TS 38.214) | 6, table 1 (Table 5.1.3.1-1, TS 38.214) |
| TBS (bits) | 72 | 72 | 72 |
| TB scaling factor | 1 | 0.5 | 0.25 |
| VRB-to-PRB mapping | Interleaved | Interleaved | Interleaved |
| Precoding granularity | 2 RB bundle | 2 RB bundle | 2 RB bundle |

Table 14 and Table 15 describe the assumptions for SIB4, taking into account

- RB size can be set to 48 and 24, following CORESET#0 span

- Configurations of 11 and 12 RB are additionally included for Urban (30kHz SCS)

- All MCS values are adjusted to accommodate TBS of 1040 bits

Table 14: Channel-specific assumptions for Msg4 (Reference UE)

| Parameter | Rel-15 NR UE | Rel-17 RedCap UE |
| --- | --- | --- |
| Target BLER | 10% | 10% |
| Waveform | CP-OFDM | CP-OFDM |
| HARQ configuration | No retransmission | No retransmission |
| DMRS configuration | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol |
| PDSCH duration | 12 OFDM symbols | 12 OFDM symbols |
| # of PDSCH RB | 48 | 48 |
| MCS | 0, table 1 (Table 5.1.3.1-1, TS 38.214) | 0, table 1 (Table 5.1.3.1-1, TS 38.214) |
| TBS (bits) | 1040 | 1040 |
| TB scaling factor | 1 | 1 |
| VRB-to-PRB mapping | Interleaved | Interleaved |
| Precoding granularity | 2 RB bundle | 2 RB bundle |

Table 15: Channel-specific assumptions for Msg4 (5MHz RedCap UE)

| Parameter | Config 1 | Config 2 | Config 3 (Urban only) |
| --- | --- | --- | --- |
| Target BLER | 10% | 10% | 10% |
| Waveform | CP-OFDM | CP-OFDM | CP-OFDM |
| HARQ configuration | No retransmission | No retransmission | No retransmission |
| DMRS configuration | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol |
| PDSCH duration | 12 OFDM symbols | 12 OFDM symbols | 12 OFDM symbols |
| # of PDSCH RB | 48 | 24 | 12 or 11 (based on UE bandwidth) |
| MCS | 0, table 1 (Table 5.1.3.1-1, TS 38.214) | 2, table 1 (Table 5.1.3.1-1, TS 38.214) | 5, table 1 (Table 5.1.3.1-1, TS 38.214) |
| TBS (bits) | 1040 | 1040 | 1040 |
| TB scaling factor | 1 | 1 | 1 |
| VRB-to-PRB mapping | Interleaved | Interleaved | Interleaved |
| Precoding granularity | 2 RB bundle | 2 RB bundle | 2 RB bundle |

The target data rates for R18 RedCap UEs are:

- FR1 Rural: 250 kbps on DL and 25 kbps in UL

- FR1 Urban: 500 kbps on DL and 250 kbps in UL

- Note: The target data rates are the scaled value in the Rel-17 RedCap SI by a factor of 0.25

Specifically, the following MCS and RB settings are assumed:

- Rural (15kHz SCS): (MCS, #RB) = (0, 32) for both R15 (1Mbps) and R17 RedCap (1Mbps), and (0, 8) for 5M RedCap (0.25 Mbps)

- Urban (30kHz SCS): (MCS, #RB) = (0, 160) for R15 (10Mbps), (0, 32) and R17 RedCap (2Mbps), and (0, 8) for 5M RedCap (0.5 Mbps)

Table 16: Channel-specific assumptions for PDSCH

| Parameter | Rel-15 NR UE | Rel-17 RedCap UE | 5MHz RedCap UE |
| --- | --- | --- | --- |
| Target BLER | 10% | 10% | 10% |
| Waveform | CP-OFDM | CP-OFDM | CP-OFDM |
| HARQ configuration | No retransmission | No retransmission | No retransmission |
| DMRS configuration | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol | Type I, 2 DMRS symbol |
| PDSCH duration | 12 OFDM symbols | 12 OFDM symbols | 12 OFDM symbols |
| Data rate | 10 Mbps | Rural: 1 Mbps Urban: 2 Mbps | Rural: 0.25 Mbps Urban: 0.5 Mbps |
| # of PDSCH RB | Rural: 32 Urban: 160 | 32 | 8 |
| MCS | 0, table 1 (Table 5.1.3.1-1, TS 38.214) | 0, table 1 (Table 5.1.3.1-1, TS 38.214) | 0, table 1 (Table 5.1.3.1-1, TS 38.214) |
| TBS (bits) | Rural: 1000 bits Urban: 5000 bits | 1000 bits | 250 bits |
| TB scaling factor | 1 | 1 | 1 |
| VRB-to-PRB mapping | Interleaved | Interleaved | Interleaved |
| Precoding granularity | 2 RB bundle | 2 RB bundle | 2 RB bundle |

# Evaluation of Coverage Impact

## Introduction to coverage analysis

In the following subsections, the coverage analysis results are based on the link-level simulation results in Appendix A evaluated for bandwidth reduction option BW1 (RF+BB: 5MHz). Detailed link budget analysis results are presented in Appendix B for reference NR UE, reference R17 RedCap UE, and the 5MHz RedCap UE with option BW1.

Regarding coverage evaluation, we follow a similar methodology Rel-17 Coverage Enhancement SI [1] and Rel-17 RedCap SI [3]. A bottleneck channel is first identified among evaluated channels in the reference UE. Then the link budgets of channels evaluated for the UE of interest are compared to the link budget of the identified bottleneck channel of the reference UE to determine whether coverage recovery is required for any channel evaluated for the UE of interest.

In both studies, the identified bottleneck channel in the reference NR UE was PUSCH. After the study, a Rel-17 WI was later carried out to focus on uplink coverage enhancement to compensate the coverage difference between uplink and downlink channels. Both PUSCH including Msg3 and PUCCH were identified as the objectives for study in Rel-17 coverage enhancement WI. Several schemes have been developed for Rel-17 UEs including RedCap UEs to enhance uplink coverage for PUSCH including Msg3 and PUCCH.

Observation 1: Rel-17 Coverage Enhancement WI have provided multiple schemes to enhance uplink coverage for PUCSH including Msg3 and for PUCCH for Rel-17 UEs.

Therefore, in our view, the coverage evaluation in this study should not assume that PUSCH is still the bottleneck channel of the reference NR UE. Instead, PRACH can be considered based on the evaluation results in [1] when we assume both PUCSH (including Msg3) and PUCCH have been enhanced.

Observation 2: When we assume uplink coverage enhancements schemes from Rel-17 WI are applied to enhance PUSCH including Msg3 and PUCCH coverage, the bottleneck channel of the reference NR UE becomes PRACH based on the evaluation results in TR38.830.

Proposal 1: Define PRACH as the bottleneck channel for the reference NR UE with the assumption that uplink coverage of PUCSH including Msg3 and PUCCH have been enhanced since Rel-17.

For each deployment scenario, we first analyze the link budget results of broadcast channels including PBCH, PDCCH CSS, and SIB1. These three were agreed as to be prioritized for coverage evaluation because they are all broadcast channels. Any configuration change to them will impact on legacy UEs that have been incumbent in the cell. We compare the coverage loss of each channel due the further reduction of UE’s bandwidth.

Proposal 2: Coexistence with legacy UEs should be carefully taken int consideration when evaluating coverage impact of PBCH, PDCCH CSS, and SIB1.

Following coverage analysis of broadcast channels, we apply the coverage evaluation methodology described in the above to all downlink channels that we have evaluated and identify whether coverage recovery is required for any channel in the normative work phase.

## Rural scenario at 0.7 GHz

### Broadcast channel coverage impact

In the rural scenario at 0.7 GHz with 15kHz SCS, the 5MHz RedCap UE can receive a full PBCH without any RE truncation. Therefore, no coverage loss is observed for PBCH for the 5MHz RedCap UE in the rural scenario at 0.7GHz with 15kHz.  On the other hand, a 3-4 dB performance loss is observed in both PDCCH CSS and SIB1 when they are truncated from 48RBs to 25RBs at the receiver.

Observation 3: Due to RE truncation, a 3-to-4 dB loss is observed for PDCCH CSS and SIB1 in the 5MHz RedCap UE compared to the reference R17 RedCap UE in the rural scenario at 0.7GHz with 15kHz.

Table 17: MIL and coverage loss of broadcast channels at Rural 0.7GHz

|  | PBCH | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 48RBs/AL16 | 24RBs/AL8 | 48RBs/MCS0 | 24RBs/MCS3 |
| MIL of R17 UE with 20MHz and 51RBs (dB) | 158.8 | 158.5 | 155.6 | 155.6 | 151.7 |
| MIL of 5MHz RedCap with 25RBs (dB) | 158.8 | 155.5 | 155.6 | 151.8 | 151.7 |
| Coverage loss (dB) | 0 | 3 | 0 | 3.8 | 0 |

### Cell coverage impact

In this section, we compare the MIL link budgets of all evaluated downlink channels to two bottleneck channels in the reference NR UE. The first bottleneck channel is PUSCH which has the smallest MIL value among all channels/messages in Table 5.1.1.6-2 of TR38.830 [3]. The assumption behind how the first bottleneck channel is determined is that no coverage enhancement has been applied. In contrast, the second bottleneck channel is determined base on the assumption that at least coverage enhancement schemes from Rel-17 Coverage Enhancements have been applied to enhance uplink coverage for PUSCH including Msg3 and PUCCH. Therefore, without taking PUSCH/PUCCH/Msg3 into consideration, the second bottleneck channel is identified as PRACH from Table 5.1.1.6-2 of TR38.830 [3]. Since PRACH Format 0 is the baseline for Rural scenario, we use it as the second bottleneck channel.

Table 18: Bottleneck channels and MIL values for reference NR UE in Urban 2.6GHz. The bottleneck channel is determined with the assumption that Rel-17 uplink coverage enhancement schemes have been applied to PUSCH including Msg3 and PUCCH.

|  | Bottleneck channel for Reference NR UE | MIL value (dB) |
| --- | --- | --- |
| Without applying uplink coverage enhancement schemes from Rel-17 WI | PUSCH (for eMBB) | 144.76 (Representative value in Table 5.1.1.6-2 of TR38.830) |
| Applying uplink coverage enhancement schemes from Rel-17 WI to enhance PUSCH including Msg3 and PUCCH | PRACH Format 0 | 150.90 (Representative value in Table 5.1.1.6-2 of TR38.830) |

When compared to PUSCH, all evaluated downlink channels have sufficient margins even when a 3dB antenna efficiency loss is considered. On the other hand, the margin of SIB1 is barely positive as shown in Table 19. When a 3dB antenna efficiency loss is assumed, SIB1 and Msg2 have a worse coverage than the bottleneck channel PRACH in reference NR UE. For Msg2 with MCS0 and TBS of 72bits, since the number of RBs is 3, TB scaling can be applied to improve its coverage.

Table 19: Coverage margin (dB) for 5MHz RedCap UE with 25RBs in Rural scenario at 0.7 GHz without antenna efficiency loss compared to bottleneck channels, PUSCH and PRACH, in reference NR UE

| 5MHz ReCap w/ 25RBs | PBCH | PDCCH CSS (AL16, 48RB, 2sym) | PDCCH CSS (AL8, 24RB, 3sym) | SIB1 (MCS0, 48RB) | SIB1 (MCS3, 24RBs) | Msg2 (MCS0, 3RBs) |
| --- | --- | --- | --- | --- | --- | --- |
| MIL (dB) | 158.83 | 155.53 | 155.63 | 151.83 | 151.73 | 152.53 |
| Margin to PUSCH (dB) | 14.07 | 10.77 | 10.87 | 7.07 | 6.97 | 7.77 |
| Margin to PRACH (dB) | 7.93 | 4.63 | 4.73 | 0.93 | 0.83 | 1.63 |
| 5MHz ReCap w/ 25RBs | Msg2 (TB scale 0.5, 6RBs) | Msg2 (TB scale=0.25, 12RBs) | Msg4 (MCS0, 48RBs) | Msg4 (MCS2, 24RBs) | PDCCH USS (AL8, 24RBs, 3sym) | PDSCH (0.25Mbps) |
| MIL (dB) | 155.93 | 159.53 | 154.23 | 153.23 | 154.53 | 154.23 |
| Margin w.r.t PUSCH (dB) | 11.17 | 14.77 | 9.47 | 8.47 | 9.77 | 9.47 |
| Margin w.r.t PRACH (dB) | 5.03 | 8.63 | 3.33 | 2.33 | 3.63 | 3.33 |

Observation 4: When compared with the first bottleneck channel, PUCSH, all the evaluated downlink channels have positive margins for coverage even when a 3dB antenna efficiency is assumed.

Observation 5: When compared with the second bottleneck channel, PRACH, the margin of SIB1 is less than 1dB when antenna efficiency loss is not assumed.

Observation 6: When compared with the second bottleneck channel, PRACH and a 3dB antenna efficiency loss is assumed, SIB1 for the 5MHz RedCap has worse coverage than PRACH for the reference NR UE by about 2dB in the Rural scenario at 0.7GHz with 15kHz.

Observation 7: TB scaling can be applied to improve the coverage performance of Msg2.

## Urban scenario at 2.6 GHz

### Broadcast channel coverage impact

In this section, we compare the link budgets of broadcast channels, i.e. PBCH, PDCCH in common search space, and SIB1, between 5MHz RedCap UE and R17 RedCap UE. Taking coexistence with incumbent UEs in the cell, we assume network transmits a broadcast channel with the same configuration for all UEs in the cell.

Table 20: Link budget MIL and coverage loss of broadcast channels and signals at Urban 2.6GHz compared to R17 RedCap UE

|  | PBCH |  | PDCCH CSS |  | SIB1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | w/o soft combining | w/ Soft combining | 48RBs/AL16 | 24RBs/AL8 | 48RBs/MCS0 | 24RBs/MCS3 |
| MIL of R17 RedCap with 20MHz 51RBs (dB) | 151.9 | 160.6 | 160.1 | 156.9 | 156.8 | 152.8 |
| MIL of 5MHz RedCap with 11RBs (dB) | 143.8 | 156.0 | 150.9 | 150.3 | 145.3 | 144.5 |
| Coverage loss (dB) | 8.1 | 4.6 | 9.2 | 6.6 | 11.5 | 8.3 |

Due to RE truncation at the receiver of a 5MHz RedCap UE, PBCH suffers a link performance loss of 8.1 dB when one-shot decoding without soft combining is performed. Even with soft combining of up to four receptions, a link performance of 4.6 dB is still observed.

For PDCCH in common search space, for example Type0-PDCCH for scheduling SIB1, the performance degradation is 9.2dB if network transmits over 48RBs (i.e. about 20MHz) to ensure coverage for incumbent UEs.

Similarly, for SIB1, 5MHz RedCap with 11 RBs suffers a link performance loss of 11.5dB compared to a reference R17 RedCap UE when NW transmits SIB1 in a configuration that does not impact on incumbent legacy UEs in the cell.

Observation 8: Due to RE truncation, a link performance loss of 4.6dB and 9.1dB is observed in PBCH with and without soft combining, respectively.

Observation 9: When taking coexistence into account and without impacting on legacy UEs’ coverage, the 5MHz RedCap UE suffers a link performance loss of 9.2dB and 11.5dB for PDCCH CSS and SIB1, respectively.

Proposal 3: SIB1 coverage recovery should be studied for both urban and rural scenarios.

### Cell coverage impact

In this section, we compare the MIL link budgets of all evaluated downlink channels to two bottleneck channels in the reference NR UE. The first bottleneck channel is PUSCH which has the smallest MIL value among all channels/messages in Table 5.1.1.2-2 of TR38.830 [3]. The assumption behind how the first bottleneck channel is determined is that no coverage enhancement has been applied. In contrast, the second bottleneck channel is determined based on the assumption that at least coverage enhancement schemes from Rel-17 Coverage Enhancements have been applied to enhance uplink coverage for PUSCH including Msg3 and PUCCH. Therefore, without taking PUSCH/PUCCH/Msg3 into consideration, the second bottleneck channel is identified as PRACH from Table 5.1.1.2-2 of TR38.830 [3]. Since PRACH Format B4 is the baseline for Urban scenario, we use it as the second bottleneck channel.

Table 21: Bottleneck channels and MIL values for reference NR UE in Urban 2.6GHz. The MIL values of the bottleneck channels are obtained from the representative values in Table 5.1.1.2-2 of TR38.830 [3].

|  | Bottleneck channel for Reference NR UE | MIL value (dB) |
| --- | --- | --- |
| Without applying uplink coverage enhancement schemes from Rel-17 WI | PUSCH (for eMBB) | 139.66 (The representative value in Table 5.1.1.2-2 of TR38.830) |
| Applying uplink coverage enhancement schemes from Rel-17 WI to enhance PUSCH including Msg3 and PUCCH | PRACH Format B4 | 152.56 (The representative value in Table 5.1.1.2-2 of TR38.830) |

Table 22: Coverage analysis for 5MHz RedCap with 11 RBs compared to bottleneck channels in reference NR UE without antenna efficiency loss at Urban 2.6GHz

| 5MHz ReCap w/ 11RBs | PBCH | PDCCH CSS (AL16) | PDCCH CSS (AL8) | PDCCH CSS (AL2) | SIB1 (MCS0, 48RBs) | SIB1 (MCS6, 11RBs) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MIL (dB) | 155.97 | 150.87 | 150.27 | 146.17 | 145.27 | 148.07 |  |
| Margin w.r.t PUSCH (dB) | 16.38 | 11.28 | 10.68 | 6.58 | 5.68 | 8.48 |  |
| Margin w.r.t PRACH (dB) | 3.41 | -1.69 | -2.29 | -6.39 | -7.29 | -4.49 |  |
| 5MHz ReCap w/ 11RBs | Msg2 (MCS0, 3RBs) | Msg2 (TB scale=0.5, 6RBs) | Msg4 (MCS0, 48RBs) | Msg4 (MCS5, 11RBs) | PDCCH USS (AL2, 6RBs, 2sym) | PDCCH USS (AL4,12RBs, 3sym) | PDSCH (R17:2Mbps; R18:0.5Mbps) |
| MIL (dB) | 152.67 | 156.27 | 145.17 | 148.87 | 148.57 | 154.77 | 158.47 |
| Margin w.r.t PUSCH (dB) | 13.08 | 16.68 | 5.58 | 9.28 | 8.98 | 15.18 | 18.88 |
| Margin w.r.t PRACH (dB) | 0.11 | 3.71 | -7.39 | -3.69 | -3.99 | 2.21 | 5.91 |

Observation 10: For 5MHz RedCap UE, SIB1 and Msg4 become the bottleneck channels among all the downlink channels evaluated due to increased effective code rates.

Observation 11: A 3dB gain is observed when SIB1/Msg4 is transmitted within 5MHz (over 11 RBs) and is fully received without RE truncation.

Proposal 4: If UE identity is learned during RACH, NW can allocate a Msg4 within the maximum bandwidth of a RedCap UE including a 5MHz RedCap UE to improve Msg4 coverage.

Proposal 5: Unless a dedicated SIB1 is transmitted for 5MHz RedCap UE, NW should not reduce the number of RBs allocated for SIB1 (e.g. from 48RBs to 11RBs) and impact legacy UEs’ coverage.

Observation 12: When compared to the first bottleneck channel PUSCH in the reference NR UE, all the downlink channels evaluated for the 5MHz RedCap UE have a positive coverage margin even after a 3dB antenna efficiency loss is considered.

Observation 13: When compared to the second bottleneck channel PRACH Format B4 in the reference NR UE, SIB1, PDCCH CSS and Msg4 have a negative coverage margin and become the bottleneck channels for the 5MHz RedCap UE.

Table 23: Coverage analysis for 5MHz RedCap with 12 RBs compared to bottleneck channels in reference NR UE without antenna efficiency loss at Urban 2.6GHz

| 5MHz RedCap with 12 RBs | PBCH | PDCCH CSS (AL16, 48RBs, 2sym) | PDCCH CSS (AL8, 24RBs, 2sym) | PDCCH CSS (AL2, 6RBs, 3sym) | PDCCH CSS (AL4, 12RBs, 3sym) | SIB1 (MCS0, 48RBs) | SIB1 (MCS6, 12RBs) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MIL (dB) | 155.97 | 151.57 | 150.97 | 146.17 | 152.47 | 146.37 | 148.07 |  |
| Margin w.r.t PUSCH (dB) | 16.31 | 11.91 | 11.31 | 6.51 | 12.81 | 6.71 | 8.41 |  |
| Margin w.r.t PRACH (dB) | 3.41 | -0.99 | -1.59 | -6.39 | -0.09 | -6.19 | -4.49 |  |
| 5MHz RedCap with 12 RBs | Msg2 (MCS0, 3RBs) | Msg2 (TB scale=0.5, 6RBs) | Msg2 (TB scale=0.25, 12RBs) | Msg4 (MCS0, 48RBs) | Msg4 (MCS5,12RBs) | PDCCH USS (AL2, 6RBs, 2sym) | PDCCH USS (AL4,12RBs, 3sym) | PDSCH (0.5Mbps) |
| MIL (dB) | 152.67 | 156.27 | 159.87 | 146.37 | 149.17 | 148.57 | 155.47 | 158.47 |
| Margin w.r.t PUSCH (dB) | 13.01 | 16.61 | 20.21 | 6.71 | 9.51 | 8.91 | 15.81 | 18.81 |
| Margin w.r.t PRACH (dB) | 0.11 | 3.71 | 7.31 | -6.19 | -3.39 | -3.99 | 2.91 | 5.91 |

Observation 14: When compared to PRACH in the reference NR UE, Msg2 with 72bits and MCS0 transmitted over 3RBs have almost zero coverage margin.

Observation 15: With TB scaling of 0.25, Msg2 coverage is improved by 7.2dB.

Proposal 6: TB scaling can be considered for Msg2 coverage improvement.

Observation 16: For an evaluated downlink channel with same configuration, performance gain of 5MHz RedCap UE with 12RBs is at most around 1dB than 5MHz RedCap UE with 11RBs.

## Summary of coverage evaluation

In this contribution, we present coverage evaluation results of downlink channels for a 5MHz RedCap UE by bandwidth reduction option BW1 with both RF and BB reduced to 5MHz.

For broadcast channels of high priority including PBCH, SIB, and PDCCH CSS, we compare the coverage loss for the 5MHz RedCap UE with respective to the reference R17 RedCap UE.

For PBCH with soft combining, a 4.6dB performance loss due to RE truncation is observed for 5MHz RedCap in Urban scenario. If soft combining is not performed for PBCH, a 9.1dB loss is observed due to RE truncation which implies additional latency in initial access for the 5MHz RedCap UE.

For PDCCH CSS, due to RE truncation, a 3dB and 9.2dB performance loss is observed in the Rural and Urban scenario, respectively, for the 5MHz RedCap UE compared with the reference Rel-17 RedCap UE when the 5MHz RedCap UE receives only 5MHz out of a 20MHz-PDCCH CSS for decoding.

For SIB1, due to RE truncation, a 3.8dB and 11.5dB performance loss is observed in the Rural and Urban scenario, respectively, for the 5MHz RedCap UE compared with the reference Rel-17 RedCap UE when the 5MHz RedCap UE receives only 5MHz out of a 20MHz-SIB1 for decoding.

For the above broadcast channels, considering the coexistence with legacy UEs, network should carefully not to impact the coverage of legacy UEs. Therefore, confining transmission of these broadcast channels to 5MHz should not be considered at all unless separate broadcast channels are to be transmitted dedicatedly for the 5MHz RedCap UE.

Because the effective code rate is increased due to RE truncation or a reduced number of allocated RBs, SIB1/Msg4 become the bottleneck channels among evaluated downlink channels. In the Urban scenario with 30kHz, when SIB1/Msg4 is transmitted within the 5MHz RedCap’s bandwidth (11RBs or 12RBs) so that no RE truncation is performed, around 2-4dB gain is observed than they are transmitted over 48RBs. However, such UE-dependent scheduling flexibility can be only applied to Msg4 once NW learns the UE’s identity by Msg1 or Msg3 during RACH.

When the link budgets of evaluated channels are compared with a bottleneck channel and identify whether any channel requires coverage recovery, we choose both PUSCH and PRACH as our bottleneck channels for the reference NR UE. When we are comparing a UE without applying any coverage enhancement schemes, it is fine to use PUSCH as the bottleneck channel which is very common in coverage studies. But we think the bottleneck should be determined based on the assumption that uplink coverage enhancement schemes developed in the Rel-17 Coverage Enhancement WI have been applied to Rel-17 UEs including RedCap UEs. Hence PUSCH including Msg3 and PUCCH are excluded when we determine the second bottleneck channel for coverage evaluation. In this way, the evaluation is done based on a reference UE with more evolved technology.

In the Rural scenario with 15kHz SCS, all downlink channels have larger link budgets than both bottleneck channels when antenna efficiency loss is not assumed. However, when a 3dB antenna efficiency loss is assumed, SIB1 has a worse coverage by 2dB than the second bottleneck channel, PRACH, of the reference NR UE.

In the Rural scenario with 30kHz SCS, when compared to the first bottleneck channel PUSCH in the reference NR UE, all the downlink channels evaluated for the 5MHz RedCap UE have a positive coverage margin even after a 3dB antenna efficiency loss is considered. However, when compared to the second bottleneck channel PRACH in the reference NR UE, SIB1, PDCCH CSS and Msg4 have a negative coverage margin and become the bottleneck channels for the 5MHz RedCap UE.

# Conclusions

In this contribution, we have made the following observations and proposals.

Observation 1: Rel-17 Coverage Enhancement WI have provided multiple schemes to enhance uplink coverage for PUCSH including Msg3 and for PUCCH for Rel-17 UEs.

Observation 2: When we assume uplink coverage enhancements schemes from Rel-17 WI are applied to enhance PUSCH including Msg3 and PUCCH coverage, the bottleneck channel of the reference NR UE becomes PRACH based on the evaluation results in TR38.830.

Observation 3: Due to RE truncation, a 3-to-4 dB loss is observed for PDCCH CSS and SIB1 in the 5MHz RedCap UE compared to the reference R17 RedCap UE in the rural scenario at 0.7GHz with 15kHz.

Observation 4: When compared with the first bottleneck channel, PUCSH, all the evaluated downlink channels have positive margins for coverage even when a 3dB antenna efficiency is assumed.

Observation 5: When compared with the second bottleneck channel, PRACH, the margin of SIB1 is less than 1dB when antenna efficiency loss is not assumed.

Observation 6: When compared with the second bottleneck channel, PRACH and a 3dB antenna efficiency loss is assumed, SIB1 for the 5MHz RedCap has worse coverage than PRACH for the reference NR UE by about 2dB in the Rural scenario at 0.7GHz with 15kHz.

Observation 7: TB scaling can be applied to improve the coverage performance of Msg2.

Observation 8: Due to RE truncation, a link performance loss of 4.6dB and 9.1dB is observed in PBCH with and without soft combining, respectively.

Observation 9: When taking coexistence into account and without impacting on legacy UEs’ coverage, the 5MHz RedCap UE suffers a link performance loss of 9.2dB and 11.5dB for PDCCH CSS and SIB1, respectively.

Observation 10: For 5MHz RedCap UE, SIB1 and Msg4 become the bottleneck channels among all the downlink channels evaluated due to increased effective code rates.

Observation 11: A 3dB gain is observed when SIB1/Msg4 is transmitted within 5MHz (over 11 RBs) and is fully received without RE truncation.

Observation 12: When compared to the first bottleneck channel PUSCH in the reference NR UE, all the downlink channels evaluated for the 5MHz RedCap UE have a positive coverage margin even after a 3dB antenna efficiency loss is considered.

Observation 13: When compared to the second bottleneck channel PRACH Format B4 in the reference NR UE, SIB1, PDCCH CSS and Msg4 have a negative coverage margin and become the bottleneck channels for the 5MHz RedCap UE.

Observation 14: When compared to PRACH in the reference NR UE, Msg2 with 72bits and MCS0 transmitted over 3RBs have almost zero coverage margin.

Observation 15: With TB scaling of 0.25, Msg2 coverage is improved by 7.2dB.

Observation 16: For an evaluated downlink channel with same configuration, performance gain of 5MHz RedCap UE with 12RBs is at most around 1dB than 5MHz RedCap UE with 11RBs.

Proposal 1: Define PRACH as the bottleneck channel for the reference NR UE with the assumption that uplink coverage of PUCSH including Msg3 and PUCCH have been enhanced since Rel-17.

Proposal 2: Coexistence with legacy UEs should be carefully taken int consideration when evaluating coverage impact of PBCH, PDCCH CSS, and SIB1.

Proposal 3: SIB1 coverage recovery should be studied for both urban and rural scenarios.

Proposal 4: If UE identity is learned during RACH, NW can allocate a Msg4 within the maximum bandwidth of a RedCap UE including a 5MHz RedCap UE to improve Msg4 coverage.

Proposal 5: Unless a dedicated SIB1 is transmitted for 5MHz RedCap UE, NW should not reduce the number of RBs allocated for SIB1 (e.g. from 48RBs to 11RBs) and impact legacy UEs’ coverage.

Proposal 6: TB scaling can be considered for Msg2 coverage improvement.

# References

- RP-213661, “New SID on Study on further NR RedCap UE complexity reduction,” Ericsson, 3GPP TSG RAN meeting #94e, December 2021

- R1-2205696, “FL summary for evaluation templates for Rel-18 RedCap SI”, May 2022

- 3GPP TR 38.830: "Study on NR coverage enhancements".

- 3GPP TR 3.875: “Study on support of reduced capability NR devices” V17.0.0, 2021-03

# Appendix A: Link-level simulation results

## A.1: PBCH

Figure 1

Figure 2

Figure 3:

Figure 4:

## A.2: PDCCH CSS

Figure 5:

Figure 6

## A.3: SIB1

Figure 7

Figure 8

## A.4: Msg2

Figure 9

Figure 10

## A.5: Msg4

Figure 11:

Figure 12

## A.7: PDCCH USS

Figure 13:

Figure 14:

## A.6: PDSCH

Figure 15:

Figure 16:

# Appendix B: Link budget analysis results

## B.1: Rural scenario at 0.7GHz

Table 25: Link budget (MIL, dB) for reference NR UE in Rural scenario at 0.7GHz

| PBCH | PDCCH CSS (AL16) | SIB1 (MCS0, 48RBs) | Msg2 (MCS0, 3RBs) | Msg4 (MCS0, 48RBs) | PDCCH USS (AL16, 48RBs, 2sym) | PDSCH (10Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 161.23 | 162.43 | 159.13 | 157.03 | 159.23 | 161.73 | 158.83 |

Table 26: Link budget (MIL, dB) for reference R17 RedCap UE in Rural scenario at 0.7GHz

| PBCH | PDCCH CSS (AL16) | SIB1 (MCS0, 48RBs) | Msg2 (MCS0, 3RBs) | Msg2 (TB scale 0.5, 6RBs) | Msg2 (Tbscale=0.25, 12RBs) | Msg4 (MCS0, 48RBs) | PDCCH USS (AL16, 48RBs, 2sym) | PDSCH (10Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 158.83 | 158.53 | 155.63 | 152.53 | 155.93 | 159.53 | 155.73 | 157.63 | 155.33 |

Table 27: Link budget in MIL (dB) for 5MHz RedCap UE with 25RBs in Rural scenario at 0.7 GHz without antenna efficiency loss

| PBCH | PDCCH CSS (AL16, 48RB, 2sym) | PDCCH CSS (AL8, 24RB, 3sym) | SIB1 (MCS0, 48RBs) | SIB1 (MCS3, 24RBs) | Msg2 (MCS0, 3RBs) | Msg2 (TB scale 0.5, 6RBs) | Msg2 (Tbscale=0.25, 12RBs) | Msg4 (MCS0, 48RBs) | Msg4 (MCS2, 24RBs) | PDCCH USS (AL8, 24RBs, 3sym) | PDSCH (0.25Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 158.83 | 155.53 | 155.63 | 151.83 | 151.73 | 152.53 | 155.93 | 159.53 | 154.23 | 153.23 | 154.53 | 154.23 |

## B.2: Urban scenario at 2.6GHz

Table 28: Link budget in MIL (dB) for reference NR UE in Urban scenario at 2.6 GHz

| PBCH | PDCCH CSS (AL16) | SIB1 (MCS0, 48RBs) | Msg2 (MCS0, 3RBs) | Msg4 (MCS0, 48RBs) | PDCCH USS (AL16, 48RBs, 2sym) | PDSCH (10Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 166.4 | 165.67 | 162.77 | 159.07 | 162.37 | 169.17 | 166.57 |

Table 29: Link budget in MIL (dB) for reference R17 RedCap UE in Urban scenario at 2.6 GHz

| PBCH | PDCCH CSS (AL16) | SIB1 (MCS0, 48RBs) | Msg2 (MCS0, 3RBs) | Msg2 (TB scale=0.25, 12RBs) | Msg4 (MCS0, 48RBs) | PDCCH USS (AL16,48RBs, 2sym) | PDSCH (2Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 160.57 | 160.07 | 156.77 | 152.67 | 159.87 | 156.57 | 163.37 | 160.17 |

Table 30: Link budget in MIL (dB) for 5MHz RedCap UE with 11RBs in Urban scenario at 2.6 GHz

| PBCH | PDCCH CSS (AL16) | PDCCH CSS (AL8) | PDCCH CSS (AL2) | SIB1 (MCS0, 48RBs) | SIB1 (MCS6, 11RBs) | Msg2 (MCS0, 3RBs) | Msg2 (TBscale=0.5, 6RBs) | Msg4 (MCS0, 48RBs) | Msg4 (MCS5, 11RBs) | PDCCH USS (AL2, 6RBs, 2sym) | PDCCH USS (AL4,12RBs, 3sym) | PDSCH (0.5Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 155.97 | 150.87 | 150.27 | 146.17 | 145.27 | 148.07 | 152.67 | 156.27 | 145.17 | 148.87 | 148.57 | 154.77 | 158.47 |

Table 31: Link budget in MIL (dB) for 5MHz RedCap UE with 12RBs in Urban scenario at 2.6 GHz without antenna efficiency loss

| PBCH | PDCCH CSS (AL16, 48RBs, 2sym) | PDCCH CSS (AL8, 24RBs, 2sym) | PDCCH CSS (AL2, 6RBs, 3sym) | PDCCH CSS (AL4, 12RBs, 3sym) | SIB1 (MCS0, 48RBs) | SIB1 (MCS6, 12RBs) | Msg2 (MCS0, 3RBs) | Msg2 (TB scale=0.5, 6RBs) | Msg2 (TB scale=0.25, 12RBs) | Msg4 (MCS0, 48RBs) | Msg4 (MCS5,12RBs) | PDCCH USS (AL2, 6RBs, 2sym) | PDCCH USS (AL4,12RBs, 3sym) | PDSCH (0.5Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 155.97 | 151.57 | 150.97 | 146.17 | 152.47 | 146.37 | 148.07 | 152.67 | 156.27 | 159.87 | 146.37 | 149.17 | 148.57 | 155.47 | 158.47 |