3GPP TSG RAN WG1 #110		                                    R1-2207131

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.9.2

Source:	InterDigital Inc.

Title:	Supporting two overlapping CRS rate matching patterns

Document for:	Discussion and Decision

# Introduction

A WI on enhancement of NR Dynamic spectrum sharing was approved [1]. The WID includes the following objective:

- Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

The justification for this enhancement is that on top of the LTE cell co-localized with NR cell, a neighboring LTE cell may create significant CRS interference particularly when the UE is at the border between two LTE cells.

# Discussion

In RAN1#109-e, RAN1 made working assumptions to introduce two new RRC parameters (lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18) to support the R18 functionality for rate matching regardless of the configuration of multi-TRP. The functionality supported by the new parameters is the same as for the legacy parameters and both sets cannot be configured at the same time. Introducing new parameters avoids modifications of parameter description in 38.331 and may avoid ambiguity if a UE operates in legacy network. The functionality to select rate matching pattern based on coresetPoolIndex associated to PDSCH when crs-RateMatch-PerCoresetPoolIndex is configured is also supported with the new parameters.

Since the working assumptions seem to properly address the objective of the WID, it is proposed to confirm them.

Proposal 1: Confirm the following working assumptions to introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18:

Working Assumption Introduce two new RRC parameters (i.e., list3 and list4). gNB cannot configure legacy list1/2 and new list3/4 simultaneously. If new list3/4 are configured, UE applies list3/4 regardless of whether CORESETPoolIndex= 1 is configured for any of DL BWPs of the serving cell. Working Assumption - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS. - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig. - For case when UE is not configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied - For case when UE is configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’ - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied. - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e., - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList31, and so on.”

# Conclusion.

This contribution proposed the following:

Proposal 1: Confirm the following working assumptions to introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18:

Working Assumption Introduce two new RRC parameters (i.e., list3 and list4). gNB cannot configure legacy list1/2 and new list3/4 simultaneously. If new list3/4 are configured, UE applies list3/4 regardless of whether CORESETPoolIndex= 1 is configured for any of DL BWPs of the serving cell. Working Assumption - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS. - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig. - For case when UE is not configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied - For case when UE is configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’ - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied. - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e., - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList31, and so on.”

# References

RP-213575, “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”, Ericsson.

R1-2205500, “Feature lead summary #4 on UE support for two overlapping CRS rate matching patterns”, Moderator (ZTE Corporation).