3GPP TSG RAN WG1 #110	R1-2207136

e-Meeting, August 22nd – 26th, 2022

Source:	CableLabs

Title:	On Evaluation Methodology for SL-U

Agenda Item:	9.4.1.1

Document for: 	Discussion and Decision

# Introduction

RANP #95, approved an updated Sidelink Evolution WID [1]. As part of this WID, the Objectives section #2 specifies

<<-ommited->>

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

<<-ommitted->>

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

<<-ommitted->>

We observe:

    - NR-U Evaluation Methodology was developed in Rel 15-16 [2], based on previous LAA-LTE Evaluation Methodology.

    - The NR-U Evaluation Methodology differs from NR Evaluation Methodology, given the coexistence with the incumbent technology.

    - It should be noted that by the time [2] was released, there was no incumbent technology in the 6GHz spectrum. However, lately one chipset vendor announced [3] 1B Wi-Fi6 chipset already shipped. More specifically, SL-U operation intended for 6GHz has to co-exist with the incumbent technology (802.11ax)

    - Under these circumstances, any change to NR-U specifications may cause significant challenges to existent Wi-Fi 6 eco-system.

This paper discusses SL-U Evaluation Methodology, in the light of the above.

# Evaluation Assumptions.

The SL-U SID should make recommendations based on a commonly agreed set of simulations assumptions.

To properly estimate the SL-U coexistence performance in a share unlicensed spectrum environment, all simulations should use the same set of baseline assumptions. Using different sets of assumptions shall not be allowed since the results can’t be assessed.

It is understood that companies could also present other evaluation results based on some optional parameters.

We consider the proposed FCC, ISED (Figure 1) and EU 6 GHz (Figure 2) spectrum allocations, as shown in the Appendix as the spectral base for the propagation assumptions.

We propose the following Evaluation Assumptions based on [2], modified per actual FCC (USA), ISED (Canada) 6 GHz regulatory allowances.

We split the evaluation assumptions into 3 categories: indoor office propagation, outdoor scenarios propagation and SL-U/Wi-Fi coexistence.

Table 1. Propagation Evaluation Parameters, sub 7GHz indoor

| Parameter | Value | Comments |
| --- | --- | --- |
| Carrier Frequency | 6 GHz (mandatory), 6405-6425 MHz, 5 GHz is optional | Channel 6405-6425MHz is the upper EU 6GHz channel and bordering US/ISED U-NII 6 |
| Carrier Channel Bandwidth | 20MHz baseline, 80MHz optional | 6405-6425MHz |
| Number of carriers | 1 |  |
| Number of users per operator | 5 UEs associated with each gNB per 20MHz |  |
| SCS | 30kHz |  |
| Channel Model | NR InH Mixed Office model |  |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |  |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |  |
| BS/AP Antenna gain | 0dBi |  |
| UE/STA Antenna gain | 0dBi |  |
| BS/AP Noise Figure | 5dB |  |
| UE/STA Receiver Noise Figure | 9dB |  |
| Minimum received power from serving cell for UE dropping | -82dBm |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ | TR38.901, section #7.3 |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ | TR38.901, section #7.3 |
| Traffic model | Table A.1.1, TR36.889 | Specific SL-U traffic needs could be discussed. |
| UE/STA to UE/STA link pathloss model | - Indoor Office (Table 7.2-2), - InH-Office propagation model | TR38.901 |
| gNB to gNB link pathloss model | - Indoor Office (Table 7.2-2), - InH-Office propagation model | TR38.901 |

Proposal 1: The sub 7GHz indoor propagation assumptions should be based on Table 1.

Table 2. Propagation Evaluation Parameters, sub 7GHz outdoor

| Parameter | Value | Comments |
| --- | --- | --- |
| Carrier Frequency | 6 GHz (mandatory), 6405-6425 MHz, 5 GHz is optional | Channel 6405-6425MHz is the upper EU 6GHz channel and bordering US/ISED U-NII 6 |
| Carrier Channel Bandwidth | 20MHz baseline , 80MHz optional | 6405-6425MHz |
| Number of carriers | 1 |  |
| Number of users per operator | 5 UEs associated with each gNB per 20MHz |  |
| SCS | 30kHz |  |
| Channel Model | NR UMi street canyon | TR38.901 |
| BS/AP Tx Power | 36dBm per beam | 6dB beam separation considered |
| UE/STA Tx Power | 23dBm (total across all TX antennas) |  |
| BS/AP Antenna gain | 0 dBi |  |
| UE/STA Antenna gain | 0 dBi |  |
| BS/AP Noise Figure | 5dB |  |
| UE/STA Receiver Noise Figure | 9dB |  |
| Minimum received power from serving cell for UE dropping | -82dBm |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ | TR38.901 |
| UE/STA antenna Array configuration | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ | TR38.901 |
| Traffic model | Table A.1.1, TR36.889 | Specific SL-U traffic needs could be discussed. |
| UE/STA to UE/STA link pathloss model | UMi street canyon pathloss model | TR38.901 |
| gNB to gNB link pathloss model | UMi street canyon pathloss model | TR38.901 |

Proposal 2: The sub 7GHz outdoor propagation assumptions should be based on Table 2.

Table 3. SL-U/Wi-Fi Coexistence Parameters (sub 7GHz)

| Parameter | Value | Comments |
| --- | --- | --- |
| Common Assumptions |  |  |
| TDD DL/UL ratio | 50/50 |  |
| Primary LBT | Cat-4 LBT (exponential back-off) |  |
| MCOT duration | 6 ms |  |
| Max Modulation | QAM256 |  |
| CCA ED threshold | -72dBm | Channel BW=20MHz |
| CW{min, max} | DL{15,63} UL{15,1023} |  |
| Max number of air layers | 2 | MIMO rank |
| Slots/Subframe | 2 |  |
| NR-U/SL-U assumptions |  |  |
| DMRS | 1 symbol | Overhead |
| DRS | Enabled, 1ms |  |
| HARQ ACQ/CQI Feedback | 1 symbol per feedback |  |
| PDCCh | 1 symbol/slot |  |
| PDSCh Mapping | Type B |  |
| PUSCh Mapping | Type A |  |
| Scheduling | Proportional fair |  |
| gNB to UE COT sharing | Enabled |  |
| UE to UE COT sharing | N/A |  |
| Wi-Fi |  |  |
| RTS/CTS | Disabled |  |
| MPDU size | 1500 bytes |  |
| Wi-Fi guard interval | Short |  |
| Frame Aggregation | A-MPDU |  |

Proposal 3: The sub 7GHz outdoor propagation assumptions should be based on Table 3.

# Conclusions

This paper proposes 3 detailed sets of assumptions (Indoor/Outdoor Propagation and SL-U/Wi-Fi Coexistence) to support the evaluation of SL-U performance in a shared spectrum environment in the presence of an incumbent access technology (802.11ax).

The use of proprietary sets of assumptions should be allowed as optional simulations assumptions. We made the following proposals:

Proposal 1: The sub 7GHz indoor propagation assumptions should be based on Table 1.

Proposal 2: The sub 7GHz outdoor propagation assumptions should be based on Table 2.

Proposal 3: The sub 7GHz outdoor propagation assumptions should be based on Table 3.

# References

[1] RP-220300, “NR SideLink Evolution”, Ran-P #95e, March 2022

[2] 3GPP TS38.889 (2018-12-19). “Study on NR-based access to unlicensed spectrum” (Release 16).

[3] https://www.broadcom.com/blog/broadcom-ships-one-billion-wi-fi-6-and-6e-chips

[4] Aruba blogs. “The 6 GHz Band: Say Goodbye to the Stone Age of Wi-Fi”,

[5] FCC, “Part 15 Subpart E U-NII 6 GHZ. General Guidance Bands 5, 6, 7, 8”, May 2022

# Appendix

Figure 1. Approved FCC 6GHz spectrum [4]. Canada's ISED follows the same spectrum assignation.

Table 4. Overview of U-NII Rules [5]

Figure 2. EU 6GHz spectrum allocation.