3GPP TSG-RAN WG1 #110	R1-2207152

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.2.2.1

Source:	InterDigital, Inc.

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

During RAN#94e, a study item (SI) on AI/ML for NR Air Interface was approved, with the revised study item description in [1].  The study item identifies use cases to focus on as follows.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

RAN1 #109-e further selected CSI compression as one representative sub-use case [2].

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case. - Note: Study of other sub use cases is not precluded. - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case. Conclusion - Further discuss temporal-spatial-frequency domain CSI compression using two-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion. - Further discuss improving the CSI accuracy based on traditional codebook design using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion. - Further discuss CSI prediction using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion - Further discuss CSI-RS configuration and overhead reduction as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion - Further discuss resource allocation and scheduling as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion - Further discuss joint CSI prediction and compression as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

For the evaluation methodology of the AI/ML based CSI feedback enhancement, RAN1 #109-e adopted system level simulation as baseline [3]. Moreover, it was agreed that “a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.”

Therefore, this contribution discusses evaluation aspects for CSI feedback enhancement and provides system-level simulation results for auto-encoder based CSI compression.

# Autoencoder (AE) based CSI compression for NR

## Dataset generation

A dataset comprised of 84k channel samples was generated using the system level testbench and UMa channel model; 80% of the dataset was used for model training, while 20% was used for validation.

The other parameters for dataset generation are as agreed in [4] (see also the table in Section 3.1 below).

## Model assumption and training

Similar to our previous contribution [5], a CSI-Net [6] based autoencoder model has been utilized where both the encoder and decoder consist of convolutional, fully connected and batch normalization layers.

- Input CSI Type: raw channel matrix estimated by UE

- Output CSI type: compressed channel matrix

- Pre-processing

To effectively compare the performance of the existing CSI feedback and precoding methods with the deep learning-based methods, we utilize the following pre-processing strategy:

  - we average the channel matrix across 2 or 4 resource blocks and across 1 time slot. Therefore, if the size of the raw channel is 624x2x16, representing 52 RBs,  = 16 and  = 2, The pre-processed channel will have a dimension of 26x2x16 or 13x2x16, depending on the averaging across 2 RBs or 4RBs, respectively.

  - Additionally, we normalize the channels to zero mean and unit variance.

- Quantization

The output of the encoder is quantized using a uniform quantizer. The encoder output is passed through a tanh layer to restrict the range of the encoder output for uniform quantization. The quantization operation is included during the training so that the encoder and decoder can learn appropriate weights while taking into account the quantization impact.

- Model Information

  - Base Model: For our evaluation we use the CSI-Net [6] autoencoder model. The model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture: RN->BN->FC

    - Quantizer: Linear quantization

    - Decoder architecture: FC -> RN->RN->RN->CNN, where:

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - BN: Batch Normalization

      - RN: CNN->BN->CNN->BN->CNN->BN with a skip connection from the input to the RN block

  - Loss function

    - We utilize the mean squared error loss function for training. The mean squared error is calculated between the input to the encoder and the output of the decoder.

  - Additional information:

    - Optimizer: ADAM with adaptive learning rate starting from a rate of = 0.0001 and scale it down by a factor of 0.9 every 5 epochs.

# Evaluation results of AI/ML based CSI compression

## Simulation Configuration and Assumptions

We used the simulation configuration agreed in [4], shown below for convenience.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Urban Macro. |
| Frequency Range |  | FR1 only, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO with rank adaptation |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Traffic model |  | Full buffer |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | 4ms delay, ideal |
| Channel estimation |  | ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. |
| Baseline for performance evaluation |  | SVD, Type 2 CSI |

## Simulation Results

We evaluated the performance of several different autoencoders, separately trained, with feedback sizes 512, 384 and 256 bits. The simulation results for the considered autoencoder models, and the comparison to the baseline are shown in Figure 1, for UMa channel models.

The legend in Figure 1 indicates:

- AE: autoencoder, average channel matrix over sub-band/PRG of 2 RBs

  - AE (nnn bits) – refers to the compressed CSI size for the autoencoder feedback

- Type II (754 bits): Rel-15 Type II codebook is used for these simulations, with average feedback size of approximately 754 bits (including CQI and RI)

Figure 1 Autoencoder performance compared to baseline

Figure 1 shows that the AE performance for the different feedback sizes analyzed are very close (where the AE 512 bits just slightly outperforms AE 256 bits). Therefore, increasing the AE feedback size above 256 bits is not justified as it does not provide a clear performance benefit.

Observation 1:  The AE performance for the different feedback sizes (512, 384 and 256 bits) are very close. Increasing the AE feedback size above 256 bits does not provide performance benefits.

Moreover, Figure 1 shows that the performance of AE based CSI compression is close to the baseline Rel-15 Type II, at a significant reduction of the feedback overhead. For example, the AE 256 model provides approximately 66% feedback overhead reduction compared to Rel-15 Type II. While it is expected that Rel-16 Type II codebook has smaller overhead compared to Rel-15 Type II, it also comes at the cost of some loss in performance. Future refining and optimization of the AE model may further improve the AE performance compared to the baseline.

Observation 2: For the scenario analyzed, the AE based CSI compression achieves about 66% feedback overhead reduction compared to the baseline Rel-15 Type II, while providing similar performance.

It should be noted that the inference was performed using different realizations of the same channel model used for the training and validation dataset (UMa). Further model optimization and evaluation over multiple scenarios and configuration is needed for a comprehensive evaluation of model generalization.

Proposal 1: Study the generalization performance of the ML models used for CSI compression.

# Conclusion

In this contribution we discussed a ML model for CSI compression, evaluated the ML model and compared its performance to Type 2 CSI codebook.  We provide the following observations and proposals:

Observation 1:  The AE performance for the different feedback sizes (512, 384 and 256 bits) are very close. Increasing the AE feedback size above 256 bits does not provide performance benefits.

Observation 2: For the scenario analyzed, the AE based CSI compression achieves about 66% feedback overhead reduction compared to the baseline Rel-15 Type II, while providing similar performance.

Proposal 1: Study the generalization performance of the ML models used for CSI compression.

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

R1-2205556, “Summary #2 on other aspects of AI/ML for CSI enhancement”, Apple

R1-2205223, “Summary#2 of [109-e-R18-AI/ML-03]”, Moderator (Huawei)

R1-2205492, “Summary#5 of [109-e-R18-AI/ML-03]”, Moderator (Huawei)

R1-2204050, “Evaluation on AI/ML for CSI feedback enhancement”, InterDigital

C. Wen, W. Shih and S. Jin, "Deep Learning for Massive MIMO CSI Feedback," in IEEE Wireless Communications Letters, vol. 7, no. 5, pp. 748-751, Oct. 2018, doi: 10.1109/LWC.2018.2818160.