3GPP TSG RAN WG1 Meeting #110    	            R1-2207250

Toulouse, France, August 22nd – 26th, 2022

Source:	Qualcomm Incorporated

Title:	UE support for two overlapping CRS rate matching patterns

Agenda Item:	9.9.2

Document for: 	Discussion and Decision

# Introduction

The 2nd objective of Rel-18 WI enhancement of NR DSS is to support two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP.

- Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

At the RAN1#109-e meeting, following agreements/working assumptions have been made [1]:

Working Assumption Introduce two new RRC parameters (i.e., list3 and list4). gNB cannot configure legacy list1/2 and new list3/4 simultaneously. If new list3/4 are configured, UE applies list3/4 regardless of whether CORESETPoolIndex= 1 is configured for any of DL BWPs of the serving cell. Working Assumption - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS. - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig. - For case when UE is not configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied - For case when UE is configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’ - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied. - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e., - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList31, and so on.” Agreement - Introduce new UE capability(ies) for support of two overlapping LTE CRS patterns in Rel-18 DSS - NW can configure two LTE-CRS overlapping rate matching patterns without coresetPoolIndex only if the UE indicates support of the new capability(ies). - Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16. - Maximum number of LTE-CRS rate matching patterns for PDSCH supported by a UE (i.e., maxNumberPatterns-r16 and maxNumberNon-OverlapPatterns-r16) is kept unchanged.

In this contribution, we provide our views on this.

# Discussion

At the RAN1#109-e meeting, two approaches have been discussed: (1) re-use RRC parameters, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, and (2) introduce new RRC parameters, lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig. The consensus was the latter approach.

One uncertainty is whether the RRC parameter “crs-RateMatch-PerCoresetPoolIndex” in the working assumption is “the existing crs-RateMatch-PerCoresetPoolIndex-r16” or “a new crs-RateMatch-PerCoresetPoolIndex-r18”. If the parameter is the existing one, field description of “crs-RateMatch-PerCoresetPoolIndex” in TS38.331 has to be updated to cover the case when lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are configured. If this is a new Rel-18 parameter, the new parameter has to be added in Rel-18.

Although either works, it would be consistent to take the same approach as for lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, i.e., introduce the Rel-18 version of the parameter.

Proposal:

- Clarify that “crs-RateMatch-PerCoresetPoolIndex” in the working assumption is a new Rel-18 parameter indicating how the UE performs rate-matching when lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are configured.

# Conclusion

In this contribution, we propose following for UE support for two overlapping CRS rate matching patterns:

Proposal:

- Clarify that “crs-RateMatch-PerCoresetPoolIndex” in the working assumption is a new Rel-18 parameter indicating how the UE performs rate-matching when lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are configured.

# Reference

- 3GPP RAN1#109-e, Chairman’s note