3GPP TSG RAN WG1 #110                                   R1-2207280

Toulouse, France, August 22nd – 26th, 2022

Source:	Sharp

Title:	Discussion on physical channel design framework for NR sidelink evolution on unlicensed spectrum

Agenda Item:	9.4.1.2

Document for:	Discussion and Decision

Introduction

In RAN#95e meeting, Rel-18 work item on support of [1] has the following objectives to study and specify support of sidelink on unlicensed spectrum.

2. Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we share some views on modifications of physical channel design for adapting sidelink communication on unlicensed spectrum.

Discussions

Physical layer structure

Necessity of “sub-channel” concept

The following was agreed in RAN1#109-e meeting:

Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

Regarding whether sub-channel is “supported” in SL-U, it should be noted that “sub-channel” has been one of the fundamental concepts in NR SL since Rel-16, and it has been the basis of a few other physical layer concepts and UE procedures including e.g.

- Definition and configuration of a resource pool.

- Definition of sidelink measurements (SL RSSI, SL CR, SL CBR) and corresponding UE procedures, e.g. congestion control.

- Mapping between a PSSCH transmission and corresponding PSFCH resource(s).

- Resource allocation/reservation for PSSCH transmissions with either dynamic grant or configured grant.

- UE procedure for determining a subset of resources for resource selection for PSSCH transmissions.

Considering the potential impact of removing “sub-channel” in SL-U to existing specifications, we think the “sub-channel” concept should really be kept in SL-U. Regarding the concern expressed in RAN1#109-e meeting on the potential difference in resource allocation/reservation granularity between SL-U and legacy SL design, we think it is just a matter of mapping one sub-channel to e.g. a number of PRBs, and even if the mapping is a little more complex than the existing one (i.e. one sub-channel mapped to a constant number of contiguous RBs), the overall design is much better than e.g. having to define completely new resource allocation/reservation indications in SCI.

Proposal 1: “Sub-channel” is defined for SL-U.

- One RB Set contains an integer number of sub-channels.

- FFS how a sub-channel is mapped to RB(s)/interlace(s), including whether there is one or multiple sub-channel sizes in a resource pool.

The following was agreed in RAN1#109-e meeting,

Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission (if supported), at least the following candidates can be discussed: - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - FFS: Other resource allocation granularity, e.g., RB-level - 1 sub-channel equals K interlaces if sub-channel is supported - FFS details - Other candidates are not precluded - FFS: mapping of PSCCH to frequency resources - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc.

Regarding frequency domain resource allocation, the existing frequency domain resource assignment indication in SCI should be kept. To that end, sub-channels should be indexed within a resource pool for interlace-based resource pool configuration, as well as for contiguous-RB-based resource pool configuration. The indexing should be done by first increasing the interlace number within an RB Set, and then increasing the RB Set number. Following a similar principle in NR-U, if two consecutive sub-channels respectively in two RB Sets are allocated, the RBs in an intra-cell guard band between the two RB sets should also be allocated.

Proposal 2: Sub-channels in a resource pool are indexed by first increasing the interlace number in an RB set, and then increasing the RB Set number.

Proposal 3: If two consecutive sub-channels respectively in two RB Sets are allocated to a PSSCH, the RBs in an intra-cell guard band, if any, between the two RB Sets, are also allocated to the PSSCH.

Time/frequency allocation for S-SSB

The following was agreed in RAN1#109-e meeting:

Agreement For S-SSB and synchronization in SL-U: - FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc. - Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission - Option 1: Using interlaced RB transmission - Option 2: S-SSB multiplexing with other SL transmissions in the same slot - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth - FFS: whether to support 4 symbols S-SSB - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above - FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission - FFS whether any changes to R16/R17 NR SL synchronization procedure

In our view, the physical-layer structure of S-SSB as specified in Rel-16 should be reused for SL-U as much as possible, e.g. including the sequences of S-PSS and S-SSS, the physical-layer sidelink synchronization identities, the resource mapping within an S-SSB for S-PSS, S-SSS, PSBCH, and DM-RS, and the time-domain locations of S-SSB.

Regarding solutions to meet OCB and PSD requirements, we think the following design principles should be followed:

- Principle 1: An S-SSB transmission should be completely accommodated by one RB Set. Otherwise transmission of S-SSBs would only be possible in wideband operation, and the probability of dropping an S-SSB transmission due to LBT failure would be significantly increased.

- Principle 2: The time/frequency resources for S-SSBs should be configured within a SL BWP rather than within a resource pool. Otherwise UE behaviours for transmission/reception of S-SSBs would be quite uncertain, e.g. it may depend on which resource pool is selected for data transmission.

- Principle 3: Detection of S-SSBs should not be more complex than that in Rel-16. For example, the time/frequency locations of all candidate S-SSBs should be fully determined prior to detection of S-SSBs.

- Principle 4: A unified solution is used for all supported SCSs and all supported carrier bandwidths, regardless of the number of RB Sets configured in a SL BWP.

With Option 1, the 1-slot/11-RB structure of S-SSB since Rel-16 can only be directly reused for some of the channel bandwidth and SCS combinations (e.g. 40 MHz with 15 kHz SCS where some interlaces have 11 RBs in one RB Set), and not for others (e.g. 40 MHz with 30 kHz SCS where all interlaces have 10 RBs in one RB Set); and another problem of Option 1 is that the definition of interlace has to be extended to cover 60 kHz SCS, increasing the specification workload. Option 2 is applicable only when a UE has scheduled other SL transmissions in a same slot, which is not always possible. And Option 4 requires re-design of the S-SSB structure including S-PSS/S-SSS and possibly also PSBCH, which is in our eyes highly undesirable. Option 3 seems the best trade-off between technical feasibility and specification workload, but more discussion is necessary on how repetition is performed in the frequency domain.

Proposal 4: Regarding solutions to meet OCB and PSD requirement for S-SSB transmissions, the following option is supported:

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain.

In case multiple RB Sets are configured in an SL BWP, in order to reduce the probability of dropping an S-SSB transmission due to LBT failure, we propose to support configuring multiple frequency locations for S-SSB transmissions, e.g. each in one RB Set in the SL BWP, excluding RBs within an intra-cell guard band. In a time location configured for S-SSB transmission, a UE can pick an RB Set with LBT success for S-SSB transmission. And in order not to complicate the S-SSB configurations in RRC, the multiple frequency locations can correspond to a single PRB offset with respect to the starting PRB of each RB Set.

Proposal 5: Multiple frequency locations can be configured for S-SSB transmissions in a SL BWP, each in one RB Set.

Proposal 6: RBs of an S-SSB should not overlap with RBs of an intra-cell guard band (if any).

PSFCH transmission for SL-U

The following related to PSFCH and SL-HARQ was agreed in RAN1#109-e meeting:

Agreement For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs

PSFCH format

According to the above agreement, support of R16 NR SL PSFCH format 0 has been agreed for PSFCH and SL-HARQ in SL-U. The R16 NR SL PSFCH format 0 only carries one HARQ-ACK information bit. However, LBT failure would occur on the unlicensed spectrum and then a UE cannot transmit the HARQ-ACK information in a given PSFCH resource due to the LBT failure. It would be beneficial if a UE can transmit multiple HARQ-ACK information bits in one PSFCH transmission opportunity. Given R16 NR SL PSFCH format 0 is supported for SL-U, enhancement to the PSFCH format 0 can be considered so that PSFCH format 0 can carry up to two HARQ-ACK information bits in a PSFCH resources. For example, the sequence cyclic shift {0, 3, 6, 9} as specified in NR Rel-15/16 can be reused for the PSFCH format 0 with 2 HARQ-ACK information bits. On the other hand, a new PSFCH format capable of multiplexing more than 2 bits can be also studied. Interlaced PUCCH format 2 as specified in NR-U can be considered as a starting point to support a PSFCH resource with more than 2 HARQ-ACK bits.

Proposal 7: Enhancement to PSFCH format 0 for multiplexing up to 2 bits is supported for SL-U.

Proposal 8: Introduction of new PSFCH format is supported by taking interlaced PUCCH format 2 in NR-U as a starting point.

PSFCH resource

For Rel-16/17 SL, for a resource pool configured with PSFCH transmission, a set of PRBs is indicated by a bitmap to be used as PSFCH resources. Each PSFCH resource in Rel-16/17 SL occupies one PRB in the frequency domain. However, for the SL-U, considering the necessity of complying with the OCB and PSD requirements, the PSFCH transmission occupying only one PRB cannot directly apply to the SL-U. In the last RAN1 109-e meeting, supporting R16 NR-U interlace RB-based transmission has been agreed as starting point for PSSCH and PSCCH. Likewise, the R16 NR-U interlace RB-based transmissions can be introduced for PSFCH transmission as starting point if OCB and PSD requirements need to be complied with in the resource pool. If the interlaced transmission is supported for PSFCH in the resource pool, each PSFCH should be confined in an interlace in an RB set to reduce the probability of LBT failure.

Proposal 9: Interlaced transmission is supported for PSFCH in SL-U.

- Frequency resource of a PSFCH is mapped to an interlace in an RB set.

PSFCH capacity

One concern accompanying support of the interlace transmission for PSFCH is that PSFCH capacity would be reduced given that the whole PRBs of an interlace needs to be used as one PSFCH resource. According to the Rel-16/17 PSFCH design principle, the required quantity of the PSFCH resources in a resource pool depends on the number of sub-channels in the resource pool, the configured period, the number of cyclic shift pair indexes. As in the WID of Rel-18 SL, the primary motivation of supporting sidelink over unlicensed spectrum is to increase sidelink data rate. Therefore, it is foreseeable that a sub-channel would occupy several interlaces or a PSSCH would occupy several sub-channels. In a sense, the PSFCH capacity issue can be alleviated.

Observation 1: Supporting increased SL data rate over unlicensed spectrum may, in a sense, alleviate the PSFCH capacity issue from interlaced PSFCH transmission.

PSFCH configuration

Implicit mechanism for PSFCH resource determination specified in the Rel-16 SL is used to determine, from (pre-)configured PSFCH resources, a frequency resource and code resource of PSFCH corresponding to a PSSCH slot and a sub-channel. The implicit mechanism therefore leads to a specific one-to-one mapping and the risk of PSFCH resource collision from UEs receiving different PSSCHs would not occur. Regarding the dynamically indication of PSFCH resources, there is a risk of dynamically indicated PSFCH resource collision if the TX UE is not aware of whether the PSFCH resource would be possibly used by another UE. Without coordination among UEs, dynamically indicated PSFCH resource may then collide with other transmission from another UE. Therefore, dynamically indicated PSFCH resource cannot be applied unless the TX UE can guarantee the PSFCH resource would not be occupied by other UEs.

Proposal 10: Implicit mechanism for PSFCH resource determination specified in the Rel-16 SL is supported as a baseline.

- Further study the dynamically indicated PSFCH resource in some specific cases.

Conclusion

In this contribution, we have discussed our views on physical channel design for adapting sidelink communication on unlicensed spectrum and have the following proposals and observation.

Proposal 1: “Sub-channel” is defined for SL-U.

- One RB Set contains an integer number of sub-channels.

- FFS how a sub-channel is mapped to RB(s)/interlace(s), including whether there is one or multiple sub-channel sizes in a resource pool.

Proposal 2: Sub-channels in a resource pool are indexed by first increasing the interlace number in an RB set, and then increasing the RB Set number.

Proposal 3: If two consecutive sub-channels respectively in two RB Sets are allocated to a PSSCH, the RBs in an intra-cell guard band, if any, between the two RB Sets, are also allocated to the PSSCH.

Proposal 4: Regarding solutions to meet OCB and PSD requirement for S-SSB transmissions, the following option is supported:

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain.

Proposal 5: Multiple frequency locations can be configured for S-SSB transmissions in a SL BWP, each in one RB Set.

Proposal 6: RBs of an S-SSB should not overlap with RBs of an intra-cell guard band (if any).

Proposal 7: Enhancement to PSFCH format 0 for multiplexing up to 2 bits is supported for SL-U.

Proposal 8: Introduction of new PSFCH format is supported by taking interlaced PUCCH format 2 in NR-U as a starting point.

Proposal 9: Interlaced transmission is supported for PSFCH in SL-U.

- Frequency resource of a PSFCH is mapped to an interlace in an RB set.

Observation 1: Supporting increased SL data rate over unlicensed spectrum may, in a sense, alleviate the PSFCH capacity issue from interlaced PSFCH transmission.

Proposal 10: Implicit mechanism for PSFCH resource determination specified in the Rel-16 SL is supported as a baseline.

- Further study the dynamically indicated PSFCH resource in some specific cases.

References

RP-220300 “WID revision: NR sidelink evolution”, RAN#95e, OPPO.